Riverbank sound system

TinyControlBoard

Technical software Manual

Anthony Roy [Date]

# TinyControlBoard Firmware Technical Design

This document was generated on 2026-06-12 through automated analysis of the TinyControlBoard PlatformIO firmware project. The analysis was performed directly against the source code, build configuration, and test suites contained within the repository.

To ensure the document reflects the current implementation rather than previously written documentation, existing Markdown files under docs/ were intentionally excluded from the source material. All architecture descriptions, behaviour summaries, protocol details, and implementation notes are derived from the code found in src/, lib/, host_tests/, and test/.

The goal of this document is to provide a code-accurate engineering reference for the firmware as it exists today. Where applicable, the document also highlights design patterns, subsystem interactions, and operational behaviour inferred from the implementation itself rather than external commentary or historical documentation.

## 1. Executive Summary

TinyControlBoard is an ESP32-S3-based front-panel controller that manages local power rails, user inputs, LED indicators, and a UART command link to a Raspberry Pi. Its primary role is to convert physical button and rotary interactions into either local control actions, such as relay switching and brightness adjustments. It manages PowerState transistions and communication with the RPI4.

The firmware is architected around a small top-level orchestrator, controlSystem::ControlBoard, which composes four main concerns:

- hardware bring-up for relays, LEDs, UART, and the MCP I2C input expander,

- an event-driven input pipeline that converts button and rotary activity into IAction objects,

- a policy layer that routes those actions either to power sequencing, local hardware control, or UART transmission,

- a heartbeat-based power-state coordinator that treats Raspberry Pi liveness as part of the system power model.

The intended use case is a custom audio or media front panel where the ESP32 controls panel LEDs, display brightness, and multiple power rails while delegating higher-level media behavior to a Raspberry Pi. The principal design goals visible in code are:

- isolate interrupt-level input capture from application policy,

- make button-to-command mapping data-driven,

- centralize power sequencing and heartbeat waits,

- keep the main task simple and move work into FreeRTOS worker tasks,

- normalize toggle-style UI commands into compact UART messages.

The hardware target is Espressif ESP32-S3-DevKitC-1-N16 (16 MB flash, no PSRAM) configured via PlatformIO with the ESP-IDF framework.

Contents

TinyControlBoard Firmware Technical Design	1

1. Executive Summary	2

2. System Overview	6

2.1 Architectural Summary	6

2.2 Data Flow	6

2.3 External Interfaces	7

2.4 Architecture Diagram	8

3. Hardware Architecture	9

3.1 Hardware Platform	9

3.2 Attached Devices and Functions	9

3.3 GPIO Allocation Table	10

3.4 I2C Devices	11

3.5 SPI Devices	11

3.6 UART Interfaces	11

3.7 PWM Outputs	11

3.8 Interrupt Sources	12

4. Software Architecture	13

4.1 Major Components	14

4.2 Component Responsibilities	14

4.3 Architectural Style	16

4.4 Why It Was Designed This Way	16

4.5 Incomplete Implementations	16

4.6 Dead Code, Compatibility Code, and Technical Debt	16

5. Application Startup Sequence	18

5.1 Step-by-Step Startup	18

5.2 Startup Sequence Diagram	19

5.3 Network Initialization	20

6. Task Architecture	21

6.1 Project-Created Tasks	21

6.2 Task Interaction Diagram	22

6.3 Design Implications	23

7. Interrupt Architecture	23

7.1 Interrupt Inventory	23

7.2 Synchronization Primitives	24

7.3 Interrupt-Related Risks	24

8. State Machines	25

8.1 Power State Machine	25

8.2 Input Action-State Machines	26

8.3 SPI Boot Indicator State Machine	28

8.4 Brightness Control State Logic	28

9. Communication Interfaces	29

9.1 UART	29

9.2 I2C	30

9.3 SPI	30

9.4 Network	31

10. Data Structures	32

10.1 Key Runtime Types	32

10.2  Type Relationships	33

10.3 Buffers and Queues	34

11. User Interface	35

11.1 Inputs	35

11.2 LED Semantics	36

11.3 User Interaction Flow	37

11.4 Sleep-Mode Input Policy	38

12. Error Handling	39

12.1 Error Condition Table	39

12.2 Recovery Strategy Summary	40

12.3 Potential Bugs and Behavioral Risks	40

13. Logging and Diagnostics	41

14. Memory Usage	42

14.1 Allocation Model	42

14.2 Risks	42

14.3 Optimization Opportunities	42

15. Performance Analysis	43

15.1 Timing-Critical Paths	43

15.2 CPU and Latency Concerns	43

15.3 Bottlenecks	43

15.4 Recommendations	43

16. Security Considerations	44

17. Build System	45

17.1 PlatformIO Configuration	45

17.2 CMake Structure	45

17.3 Build Behavior	45

18. Dependency Analysis	46

19. Maintenance Guide	47

19.1 Adding a New Front-Panel Feature	47

19.2 Adding a New Peripheral	47

19.3 Modifying the UART Protocol	47

19.4 Areas Requiring Caution	47

20. Appendix	48

20.1 File Structure	48

20.2 Configuration Parameters	54

20.3 Important Constants and Definitions	55

20.4 Summary of Highest-Value Follow-Up Work	55

## 2. System Overview

### 2.1 Architectural Summary

#### System Diagram

#### Software component descriptions

| Component | Purpose | Inputs | Outputs | Dependencies |
| --- | --- | --- | --- | --- |
| ControlBoard | Top-level runtime composition | app_main(), UART RX callbacks, MCP callbacks | System initialization, action execution | bootstrap helpers, UART transport, relays, input dispatcher, action processor |
| ControlBoardBootstrap | Boot-time wiring helpers | board config constants | Relay setup, serial config, MCP config, indicator startup | relays, McpInputHandler, UartTransport, indicators |
| ButtonEventQueue | Queue and dispatch raw input events | button/rotary callbacks | Calls into ControlBoardInputDispatcher | FreeRTOS queue/task |
| ControlBoardInputDispatcher | UI policy and event-to-action conversion | ButtonEvent objects | IAction objects plus LED changes | action map, indicator sink |
| ControlBoardActionRegistry | Populates the action map | static registration table | ActionMap entries | Dynamic*Action classes |
| ActionProcessor | Executes actions and enforces power gating | IAction objects, inbound UART messages | power transitions, relays, UART sends, brightness changes | transport, relay controller, power handler, indicators |
| ActionUartDispatcher | Converts actions into wire-format UART messages | executed actions | UartMessage sends | command catalog, UART sink |
| UartTransport | UART framing, handshake, RX task, heartbeat monitor | bytes from UART, GPIO42 IRQ | parsed UartMessage objects and heartbeat timeout callback | UART driver, GPIO ISR, UartReceiver |
| McpInputHandler | I2C expander access and rotary decode | GPIO18 IRQ and I2C reads | button and rotary callbacks | I2C driver, GPIO ISR |
| PowerStateTransitionHandler | Boot, sleep, and deep-sleep sequencing | power actions and current state | relay operations, LED state changes, waits | relay controller, boot manager, serial |
| RpiBootManager | Synchronizes heartbeat-based waits | heartbeat received/timeout events | boot/shutdown wait results | FreeRTOS event group |
| RelayController | Encapsulates relay operations | action/power requests | GPIO changes | StandardRelay |
| MonitorBrightnessController | Brightness persistence and display-off mode | brightness commands, power state changes | PWM duty changes, NVS writes | LEDC, NVS |
| PowerLed | User-visible power-state indicator | power state changes | active/standby LED patterns | LEDC, esp_timer |
| StatusLed | User-visible activity and button-status indication | status messages | LEDC duty and blink patterns | FreeRTOS queue, timer, tasks |
| SpiBootIndicator | Boot-wait/failure indicator using 16 button LEDs | boot outcome | SPI LED blink patterns | SPI LED driver |

### ControlBoard  action Areas

We will now describe each of the macro functions for this board and the process by which they are enabled

##### Controlboard power applied

If the current state is OFF, SLEEP, or DEEPSLEEP, a power-button release enters the power-on path.

- A ControlBoard instance is constructed.

- ControlBoard::init() initializes NVS.

- Startup indicators are prepared by setting the power LED state to TURNING_ON.

- ControlBoard::initTransport() acquires the singleton UART transport and relay interface and configures serial RX and heartbeat callbacks.

- ControlBoard::initComponents() allocates the action processor and input dispatcher.

- Relay GPIOs are initialized and forced off.

- The MCP input expander is initialized over I2C, its interrupt pin is configured, and mcp_int_task is created.

- ButtonEventQueue creates its queue and action_task.

The late-heartbeat promotion path is now partly isolated behind DelayedBootRecoveryState, which gives the host-side CMake test suite direct coverage over the one-shot degraded-boot recovery decision without needing the ESP-IDF runtime.

- MCP callbacks are installed so button/rotary changes feed the queue.

- UART is initialized, the Pi data-ready input ISR is attached, and uart_rx_task plus heartbeat_task are started.

- The action map is populated.

- A final fixed delay of 5000 ms is applied by finalizeStartupIndicators().

- ActionProcessor::triggerInitialPowerOn() synthesizes a power command.

- Power-on sequencing enables screen, DAC, output stage, and Pi relays in order.

- SpiBootIndicator flashes all button LEDs while the firmware waits for heartbeat.

- If heartbeat arrives, the system enters ON; otherwise it falls back to SLEEP and initialization is reported as failed.

- If initialization fails, app_main() deinitializes portions of the board, waits 1 second, and retries indefinitely.

If initialization succeeds, app_main() remains alive and all runtime work is performed by background tasks.

##### Shutdown

###### Sleep Sequence

If the current state is ON and the power button hold time is less than 3000 ms, the firmware enters the sleep path.

Current sequence:

- power LED state becomes GOING_TO_SLEEP,

- CMD_SYS_RPI_SHUTDOWN is sent over UART,

- firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout,

- Raspberry Pi power relay is turned off,

- firmware delays 500 ms,

- screen power relay is turned off,

- firmware delays 5000 ms,

- power LED state becomes SLEEP,

- activity status becomes sleeping.

Practical result:

- the Raspberry Pi and screen are shut down,

- DAC and output stage power remain on,

- the board stays in a lower-power standby style state rather than a full deep power-down.

###### DeepSleep

If the current state is ON and the power button hold time is 3000 ms or more, the firmware enters the deep-sleep path.

Current sequence:

- power LED state becomes GOING_TO_SLEEP,

- CMD_SYS_RPI_SHUTDOWN is sent over UART,

- firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout,

- Raspberry Pi power relay is turned off,

- firmware delays 500 ms,

- screen power relay is turned off,

- DAC relay is turned off,

- output stage relay is turned off,

- power LED state becomes DEEPSLEEP,

- activity status becomes sleeping.

Practical result:

- Raspberry Pi power is removed,

- screen power is removed,

- DAC power is removed,

- output stage power is removed,

- board logic still remains present enough to respond later, so this is a project-specific deep sleep state, not necessarily ESP-IDF chip deep sleep.

##### Buttons

###### Press

###### Release

##### Rotary control

###### Turn Left

###### Turn Right

### 2.2 Data Flow

#### Button and rotary data flow:

- MCP interrupt asserts on GPIO18.

- McpInputHandler::gpioIsr() wakes mcp_int_task.

- McpInputHandler::handleInterrupt() reads GPIO state over I2C and emits button press, release, or rotary callbacks.

- ControlBoard callbacks enqueue ButtonEvent objects into ButtonEventQueue.

- action_task dequeues the event and calls ControlBoardInputDispatcher.

- The dispatcher updates UI status, optionally toggles button LEDs, and invokes the mapped IActionSource.

- The produced IAction is passed to ActionProcessor.

- ActionProcessor executes the action locally or routes it to UART.

#### Inbound serial data flow:

- Raspberry Pi asserts the data-ready input on GPIO42.

- UartTransport::gpioIsrHandler() wakes uart_rx_task.

- UartTransport::handleUartRx() reads bytes from UART2 and feeds UartReceiver.

- UartReceiver scans the byte stream for valid 18-byte protocol frames.

- Parsed UartMessage objects are forwarded to ControlBoard::handleSerialRxMessage().

- Heartbeat commands update boot/shutdown synchronization; all other inbound commands currently log and fall through because inbound protocol handling is still scaffolded.

### 2.3 External Interfaces

- UART2 to Raspberry Pi at 115200 baud.

- GPIO handshake lines for UART data-ready signaling.

- I2C master to the MCP input expander at address 0x20.

- SPI output to a 16-bit LED shift-register or similar latch-based LED driver.

- GPIO outputs for power relays.

- LEDC PWM outputs for power-state LEDs, button status LED, and monitor brightness control.

- NVS for persistent brightness level storage.

### 2.4 Architecture Diagram

Figure 1

graph TD
    Buttons[Buttons and Rotary] --> MCP[MCP Input Expander]
    MCP -->|I2C + IRQ| ESP32[ESP32-S3 Firmware]
    ESP32 -->|SPI| ButtonLEDs[Button LED Shift Register]
    ESP32 -->|PWM| PowerLEDs[Power and Status LEDs]
    ESP32 -->|PWM| Brightness[Monitor Brightness Output]
    ESP32 -->|GPIO| Relays[Power Relays]
    ESP32 -->|UART2 + Handshake| RPi[Raspberry Pi]
    RPi -->|Heartbeat + Commands| ESP32

## 3. Hardware Architecture

### 3.1 Hardware Platform

- MCU: ESP32-S3.

- Board definition: esp32-s3-devkitc-1-16mb.

- Flash: 16 MB.

- Framework: ESP-IDF under PlatformIO.

- CPU clock configured to 240 MHz.

- Board definition advertises Wi-Fi and Bluetooth capability, but the firmware does not use either stack.

### 3.2 Attached Devices and Functions

- I2C device: one MCP23018-style 16-bit GPIO expander at address 0x20.

- SPI device: one 16-bit LED driver or shift-register chain with a manual latch line.

- UART device: Raspberry Pi control link.

- Relays: seven GPIO-controlled outputs.

- LEDs:

  - active power LED,

  - standby power LED,

  - working status LED,

  - button status LED PWM output,

  - 16 SPI-driven button LEDs.

- Rotary encoder: represented in firmware as two MCP input bits and decoded in software.

- Display: no display protocol driver exists in firmware; the firmware only controls monitor brightness and a display-off brightness mode.

- Sensors: none present in code.

- Network peripherals: none initialized in firmware.

### 3.3 GPIO Software Allocation Table

| GPIO | Function | Direction | Notes |
| --- | --- | --- | --- |
| 1 | UART2 RX | Input | Raspberry Pi to ESP32 serial RX |
| 2 | UART2 TX | Output | ESP32 to Raspberry Pi serial TX |
| 3 | Active power LED PWM | Output | PowerLed active channel |
| 4 | Standby power LED PWM | Output | PowerLed standby channel |
| 5 | SPI LED latch | Output | Manual latch pulse after 16-bit transfer |
| 6 | SPI LED clock | Output | SPI2 SCLK |
| 7 | SPI LED data | Output | SPI2 MOSI |
| 9 | Output-stage power relay | Output | Power sequencing |
| 10 | Proto DAC enable relay | Output | Exposed but not used in runtime policy |
| 11 | Raspberry Pi power relay | Output | Power sequencing |
| 12 | DAC power relay | Output | Power sequencing and toggle DAC action |
| 13 | Screen power relay | Output | Power sequencing |
| 15 | I2C SCL | Output/Open-drain | MCP bus clock |
| 16 | I2C SDA | Bidirectional/Open-drain | MCP bus data |
| 17 | I2C enable | Output | Gates external I2C hardware enable |
| 18 | MCP interrupt | Input | Negative-edge ISR source |
| 21 | Button status LED PWM | Output | StatusLed with default SolidIdle |
| 39 | General relay 2 | Output | Initialized but not used by action policy |
| 41 | ESP32 data-ready to Pi | Output | Asserted after UART transmit |
| 42 | Pi data-ready to ESP32 | Input | Rising-edge ISR source for UART RX |
| 43 | Monitor brightness PWM | Output | Backlight or brightness control |
| 47 | General relay 1 | Output | Initialized but not used by action policy |
| 48 | Activity status LED PWM | Output | StatusLed for work/idle/sleep patterns |

### 3.4 I2C Devices

| Device | Address | Role | Access Pattern |
| --- | --- | --- | --- |
| MCP23018-style input expander | 0x20 | Reads 16 button lines and rotary bits | Interrupt-driven, read-on-change |

The code uses the MCP register model directly, enables pull-ups on both banks, and enables interrupt-on-change for all 16 inputs.

### 3.5 SPI Devices

| Device | Role | Data Width | Notes |
| --- | --- | --- | --- |
| SPI LED driver / shift register | Drives button LEDs | 16 bits | Low byte sent first, then high byte |

### 3.6 UART Interfaces

| Interface | Port | Baud | Purpose |
| --- | --- | --- | --- |
| Main control link | UART_NUM_2 | 115200 | ESP32 <-> Raspberry Pi command protocol |

### 3.7 PWM Outputs

| Output | Implementation | Purpose |
| --- | --- | --- |
| Active power LED | LEDC | ON state indication |
| Standby power LED | LEDC | OFF, sleep, and transition indication |
| Activity status LED | LEDC + timer/task | Work/active/sleep blips |
| Button status LED | LEDC + timer/task | Idle brightness and sleep/active blips |
| Monitor brightness | LEDC | Display backlight or monitor dimming |

### 3.8 Interrupt Sources

| Source | GPIO | Trigger | Consumer |
| --- | --- | --- | --- |
| MCP interrupt | 18 | Negative edge | McpInputHandler::gpioIsr() |
| Pi data-ready | 42 | Positive edge | UartTransport::gpioIsrHandler() |

## 4. Software Architecture

### 4.1 Major Components

graph LR
    Main[app_main] --> Board[ControlBoard]
    Board --> Bootstrap[ControlBoardBootstrap]
    Board --> InputQ[ButtonEventQueue]
    Board --> Dispatcher[ControlBoardInputDispatcher]
    Board --> Processor[ActionProcessor]
    Board --> UART[UartTransport]
    Board --> MCP[McpInputHandler]
    Board --> Indicators[ledManager singletons]

    Dispatcher --> Sources[IActionSource registry]
    Sources --> Actions[IAction objects]
    Actions --> Processor

    Processor --> Power[PowerStateTransitionHandler]
    Processor --> Relay[RelayController]
    Processor --> UartDispatch[ActionUartDispatcher]
    Processor --> BootMgr[RpiBootManager]
    UartDispatch --> Sink[SerialUartCommandSink]
    Sink --> UART

### 4.2 Component Responsibilities

### 4.3 Architectural Style

Three design choices dominate the codebase:

- Event capture is separated from policy. GPIO ISRs only wake tasks. The tasks then read hardware state and enqueue higher-level events.

- Button behavior is data-driven. ControlBoardActionRegistry uses a registration table so mapping changes mostly affect one file.

- Side effects are command-oriented. Buttons do not directly manipulate relays or UART; they create IAction objects that are executed later with an ActionContext.

### 4.4 Why It Was Designed This Way

The design reduces the amount of logic in interrupt context and avoids coupling user input semantics to transport or relay code. The Raspberry Pi is treated as an external application processor, so the ESP32 owns front-panel determinism, local safety, and power rails while delegating media behavior and UI context to the Pi.

The power model also explains several implementation choices:

- heartbeat is used as a boot-complete and shutdown-complete signal,

- toggle commands are normalized on the wire to avoid duplicating Raspberry Pi handlers,

- the monitor brightness controller persists state locally because brightness must survive Pi resets.

### 4.5 Incomplete Implementations

- ActionProcessor::handleInboundUartMessage() is explicitly a scaffold and currently only logs inbound non-heartbeat messages.

- No network stack, web API, Wi-Fi client, Ethernet, or MQTT code is present.

- SystemAction only performs meaningful work for CMD_SYS_RPI_SHUTDOWN; CMD_EXIT_ITEM is a log-only no-op.

- The board definition supports Wi-Fi and Bluetooth, but the firmware does not initialize them.

### 4.6 Dead Code, Compatibility Code, and Technical Debt

Observed dormant or compatibility-oriented paths:

- actions::Action in actionsResponse.hpp is a no-op test helper and is not produced by the live firmware pipeline.

- SimpleCommandAction.hpp is a backward-compatibility include shim; the implementation lives in actionTemplates.hpp.

- Template action sources ToggleAction<>, TimedAction<>, and RotaryAction<> remain defined, but the live registry uses DynamicToggleAction, DynamicTimedAction, and DynamicRotaryAction.

- StatusLed::startBreatheEffect() and StatusLed::getBlinkInterval() are defined but not referenced by the runtime path.

- UartTransport::sendData(const char *p_message) is declared in the header but has no implementation.

- CMD_ROTARY_LEFT and CMD_ROTARY_RIGHT remain in the protocol catalog, but the active runtime emits CMD_ROTARY_ACTION with parameter 0 or 1.

- ControlBoardPowerState::SHUTTING_DOWN and GOING_INTO_DEEP_SLEEP are handled in indicators but are not emitted by the current transition policy.

Technical debt worth calling out:

- Power-management configuration in app_main() is annotated by the source as ineffective.

- A fixed 5-second startup delay exists solely to wait for power to stabilize.

- Hardware pin aliases are split between board::... namespaces and legacy PIN_* macros.

- Command routing rules are distributed across multiple places: the registration table, ActionCommandRoutingPolicy, and ActionUartDispatcher toggle normalization.

## 5. Application Startup Sequence

### 5.1 Step-by-Step Startup

- Power is applied to the ESP32-S3 board.

- ESP-IDF initializes the runtime and enters app_main().

- app_main() configures esp_pm_config_t with max 240 MHz, min 40 MHz, and light sleep enabled.

- app_main() waits 5000 ms before any board work begins.

- A ControlBoard instance is constructed.

- ControlBoard::init() initializes NVS.

- Startup indicators are prepared by setting the power LED state to TURNING_ON.

- ControlBoard::initTransport() acquires the singleton UART transport and relay interface and configures serial RX and heartbeat callbacks.

- ControlBoard::initComponents() allocates the action processor and input dispatcher.

- Relay GPIOs are initialized and forced off.

- The MCP input expander is initialized over I2C, its interrupt pin is configured, and mcp_int_task is created.

- ButtonEventQueue creates its queue and action_task.

The late-heartbeat promotion path is now partly isolated behind DelayedBootRecoveryState, which gives the host-side CMake test suite direct coverage over the one-shot degraded-boot recovery decision without needing the ESP-IDF runtime.

- MCP callbacks are installed so button/rotary changes feed the queue.

- UART is initialized, the Pi data-ready input ISR is attached, and uart_rx_task plus heartbeat_task are started.

- The action map is populated.

- A final fixed delay of 5000 ms is applied by finalizeStartupIndicators().

- ActionProcessor::triggerInitialPowerOn() synthesizes a power command.

- Power-on sequencing enables screen, DAC, output stage, and Pi relays in order.

- SpiBootIndicator flashes all button LEDs while the firmware waits for heartbeat.

- If heartbeat arrives, the system enters ON; otherwise it falls back to SLEEP and initialization is reported as failed.

- If initialization fails, app_main() deinitializes portions of the board, waits 1 second, and retries indefinitely.

- If initialization succeeds, app_main() remains alive and all runtime work is performed by background tasks.

### 5.2 Startup Sequence Diagram

Figure 2

sequenceDiagram
    participant ROM as ESP-IDF Runtime
    participant Main as app_main
    participant Board as ControlBoard
    participant MCP as McpInputHandler
    participant UART as UartTransport
    participant Power as PowerStateTransitionHandler
    participant RPi as Raspberry Pi

    ROM->>Main: enter app_main()
    Main->>Main: configure esp_pm
    Main->>Main: delay 5000 ms
    Main->>Board: init()
    Board->>Board: initNvs()
    Board->>Board: prepareStartupIndicators()
    Board->>UART: configure callbacks
    Board->>Board: allocate ActionProcessor and Dispatcher
    Board->>Board: setupRelays()
    Board->>MCP: begin(I2C, IRQ)
    Board->>Board: start ButtonEventQueue
    Board->>UART: initUart() + startHeartbeatMonitor()
    Board->>Board: populate action map
    Board->>Board: finalizeStartupIndicators() delay 5000 ms
    Board->>Power: triggerInitialPowerOn()
    Power->>Power: enable relays in sequence
    Power->>RPi: wait for heartbeat
    alt Heartbeat received
        RPi-->>UART: heartbeat frame
        UART-->>Board: RX callback
        Board-->>Power: boot complete via RpiBootManager
    else Timeout
        Power-->>Board: boot failed
    end
    Board-->>Main: init result
    Main->>Main: retry or idle loop

### 5.3 Network Initialization

There is no network initialization path. No Wi-Fi, Ethernet, TCP/IP, HTTP, or MQTT code is present in the firmware.

## 6. Task Architecture

### 6.1 Project-Created Tasks

| Task | Priority | Stack | Purpose | Execution Model | Dependencies |
| --- | --- | --- | --- | --- | --- |
| action_task | 5 | 4096 | Dequeue ButtonEvent objects and call the dispatcher | Blocks on queue receive | ButtonEventQueue, dispatcher |
| uart_rx_task | 10 | 4096 | Handle UART receive after GPIO42 data-ready interrupt | Woken by task notification | GPIO42 ISR, UART driver |
| heartbeat_task | 5 | 4096 | Poll last-RX timestamp and detect heartbeat timeout | 100 ms periodic loop | esp_timer_get_time(), callback |
| mcp_int_task | 10 | 4096 | Read MCP state and decode button/rotary changes | Woken by task notification | GPIO18 ISR, I2C driver |
| LED_Task (activity LED) | 5 | 4096 | Drive the activity-status LED state machine | Queue-driven | FreeRTOS queue and timer |
| LED_Task (button status LED) | 5 | 4096 | Drive the button-status LED state machine | Queue-driven | FreeRTOS queue and timer |
| spi_boot_ind | tskIDLE_PRIORITY + 1 | 4096 | Flash all SPI LEDs during boot wait or failure | Delay loop until stop flag | SPI LED driver |
| BreatheTask | 5 | 2048 | Unused runtime breathe effect for StatusLed | Delay loop | Not referenced by live runtime |

Additional execution contexts used by the firmware:

- app_main() task from ESP-IDF.

- esp_timer callback context for PowerLed::handleTimer().

- FreeRTOS software timer service task for StatusLed blink timers.

### 6.2 Task Interaction Diagram

Figure 3

graph TD
    MCPIRQ[GPIO18 ISR] --> MCPTask[mcp_int_task]
    MCPTask --> BtnQueue[action_task queue]
    BtnQueue --> ActionTask[action_task]
    ActionTask --> Dispatcher[Input Dispatcher]
    Dispatcher --> Processor[ActionProcessor]

    UartIRQ[GPIO42 ISR] --> UartTask[uart_rx_task]
    UartTask --> Processor
    HeartbeatTask[heartbeat_task] --> Processor

    Processor --> PowerTask[Power sequencing in caller context]
    Processor --> UartTx[UART TX path]
    Processor --> RelayOps[Relay operations]

    ActivityLEDQ[activity LED queue] --> ActivityLEDTask[LED_Task]
    ButtonLEDQ[button LED queue] --> ButtonLEDTask[LED_Task]
    BootIndicator[spi_boot_ind] --> SPILEDs[SPI LEDs]

### 6.3 Design Implications

- Input processing is serialized through action_task, which simplifies button logic and avoids race-heavy action execution.

- Both MCP and UART receive work run at priority 10, above the action and status tasks.

- Heartbeat monitoring is coarse-grained at 100 ms, which is acceptable for 30-second liveness windows.

- Several blocking delays occur in the caller context of power transitions and UART TX signaling; those are acceptable for the current architecture because those paths are not ISR-bound, but they do reduce responsiveness during long sequencing steps.

## 7. Interrupt Architecture

### 7.1 Interrupt Inventory

| Source | Trigger | ISR | Actions Performed | Synchronization |
| --- | --- | --- | --- | --- |
| MCP interrupt on GPIO18 | Negative edge | McpInputHandler::gpioIsr() | Notify mcp_int_task only | Task notification |
| Pi data-ready on GPIO42 | Positive edge | UartTransport::gpioIsrHandler() | Notify uart_rx_task only | Task notification |

The project follows a sound ISR policy: the ISR body only performs vTaskNotifyGiveFromISR() and optional portYIELD_FROM_ISR().

### 7.2 Synchronization Primitives

Queues:

- ButtonEventQueue uses a depth-16 FreeRTOS queue to move input events from callback context to action_task.

- Each StatusLed instance owns a depth-1 queue and uses xQueueOverwrite() so the latest status always wins.

Task notifications:

- mcp_int_task waits on a notification from GPIO18.

- uart_rx_task waits on a notification from GPIO42.

- StatusLed::sendStatus() also uses task notification to wake the LED task quickly after queue overwrite.

Event groups:

- RpiBootManager owns one event group.

- One bit represents heartbeat received.

- One bit represents heartbeat timeout, which the code interprets as shutdown confirmation.

Semaphores:

- No explicit FreeRTOS semaphores are used by the application code.

Atomics:

- SystemState::powerState is atomic.

- UartTransport uses atomics for task-stop flags, initialization state, and last-RX timestamp.

- StatusLed and SpiBootIndicator use atomics for state and stop flags.

### 7.3 Interrupt-Related Risks

- The UART RX wakeup relies on a separate data-ready GPIO rather than the UART driver’s own event queue, so any mismatch between GPIO handshake timing and UART framing can starve or burst the RX task.

- GPIO ISR service installation is attempted in both UART and MCP layers; the code tolerates ESP_ERR_INVALID_STATE, which is practical but indicates shared-global initialization concerns.

## 8. State Machines

### 8.1 Power State Machine

The primary state machine is ControlBoardPowerState plus evaluatePowerTransition().

Actual decision states in the current runtime:

- OFF

- TURNING_ON

- ON

- GOING_TO_SLEEP

- SLEEP

- DEEPSLEEP

Declared but not actively emitted by the transition policy:

- SHUTTING_DOWN

- GOING_INTO_DEEP_SLEEP

Figure 4

stateDiagram-v2
    [*] --> OFF
    OFF --> TURNING_ON: CMD_SYS_POWER
    SLEEP --> TURNING_ON: CMD_SYS_POWER
    DEEPSLEEP --> TURNING_ON: CMD_SYS_POWER
    TURNING_ON --> ON: heartbeat received
    TURNING_ON --> SLEEP: boot timeout
    ON --> GOING_TO_SLEEP: short press
    GOING_TO_SLEEP --> SLEEP: shutdown path complete
    ON --> GOING_TO_SLEEP: long press
    GOING_TO_SLEEP --> DEEPSLEEP: deep sleep path complete

Entry and exit behavior:

- Enter TURNING_ON: set LED indicators, enable relays in order, start boot-wait SPI blink.

- Enter ON: stop boot indicator, restore brightness, set activity to active.

- Enter SLEEP: screen and Pi off, LEDs cleared, activity becomes sleeping.

- Enter DEEPSLEEP: screen, Pi, DAC, and output-stage rails off, activity becomes sleeping.

### 8.2 Input Action-State Machines

Button behavior is implemented by IActionSource variants rather than a single explicit enum.

Timed action state machine, used by the power button:

stateDiagram-v2
    [*] --> Idle
    Idle --> Pressed: button press
    Pressed --> Idle: button release / emit CMD_SYS_POWER with releaseTimeMillis

Figure 5

Toggle action state machine, used by cover, repeat, random, DAC, and meter toggles:

stateDiagram-v2
    [*] --> OffSemantic
    OffSemantic --> OnSemantic: press / emit ON command
    OnSemantic --> OffSemantic: press / emit OFF command

Figure 6

Rotary State Machine:

Figure 7

stateDiagram-v2
    [*] --> Stable
    Stable --> Stable: quadrature transition / emit CMD_ROTARY_ACTION param 0 or 1

### 8.3 SPI Boot Indicator State Machine

stateDiagram-v2
    [*] --> Idle
    Idle --> Booting: startWaiting()
    Booting --> Idle: notifySuccess()
    Booting --> Failed: notifyFailure()
    Failed --> Idle: notifySuccess() or task stop#

Figure 8

Behavior:

- Booting: flash all button LEDs at 500 ms half-period.

- Failed: flash all button LEDs at 150 ms half-period.

### 8.4 Brightness Control State Logic

The monitor brightness controller keeps an independent mode flag, m_displayOffActive, which temporarily forces brightness to zero and ignores further brightness changes until toggled back.

## 9. Communication Interfaces

### 9.1 UART

Configuration:

- port: UART_NUM_2

- baud rate: 115200

- frame format: 8 data bits, no parity, 1 stop bit

- flow control: disabled

- receive buffer configured in firmware: 256 bytes requested from board::serial::k_bufferSize, doubled to 512 bytes when installing the driver

#### Packet structure:

| Byte Offset | Field | Size (bytes) |
| --- | --- | --- |
| 0 | start byte | 1 |
| 1 | protocol version | 1 |
| 2 | source app | 1 |
| 3 | message type | 1 |
| 4 | sequence | 1 |
| 5–6 | command ID | 2 |
| 7–16 | five 16-bit parameters | 10 |
| 17 | checksum | 1 |

Message Types

| Name | Value |
| --- | --- |
| MSG_COMMAND | 0x01 |
| MSG_STATUS | 0x02 |
| MSG_ACK | 0x03 |
| MSG_NACK | 0x04 |

Application ID’s

| Name | Value |
| --- | --- |
| APP_ESP32 | 0x01 |
| APP_PI | 0x02 |

#### Command Catalog

| Command | Value |
| --- | --- |
| CMD_NO_ACTION | 0x0000 |
| CMD_SYS_POWER | 0x0001 |
| CMD_SYS_RPI_SHUTDOWN | 0x0002 |
| CMD_SYS_HEARTBEAT | 0x0003 |
| CMD_SYS_NOHEARTBEAT | 0x0004 |
| CMD_NEXT_TRACK | 0x0100 |
| CMD_PREVIOUS_TRACK | 0x0101 |
| CMD_PLAY_PAUSE | 0x0102 |
| CMD_STOP_TRACK | 0x0103 |
| CMD_SKIP_FORWARD | 0x0104 |
| CMD_SKIP_BACK | 0x0105 |
| CMD_PREV_MENU_ITEM | 0x0106 |
| CMD_NEXT_MENU_ITEM | 0x0107 |
| CMD_ITEM_SELECT | 0x0108 |
| CMD_EXIT_ITEM | 0x0109 |
| CMD_TOGGLE_DAC_ON | 0x010A |
| CMD_DISPLAY_OFF | 0x010B |
| CMD_TOGGLE_METER_ON | 0x010C |
| CMD_TOGGLE_METER_OFF | 0x010D |
| CMD_DISPLAY_ON | 0x010E |
| CMD_TOGGLE_DAC_OFF | 0x010F |
| CMD_ROTARY_LEFT | 0x0110 |
| CMD_ROTARY_RIGHT | 0x0111 |
| CMD_ROTARY_ACTION | 0x0112 |
| CMD_TOGGLE_DAC | 0x0113 |
| CMD_TOGGLE_DISPLAY | 0x0114 |
| CMD_TOGGLE_METER | 0x0115 |
| CMD_CYCLE_BRIGHTNESS | 0x0116 |
| CMD_COVER_VIEW_ON | 0x0117 |
| CMD_COVER_VIEW_OFF | 0x0118 |
| CMD_TOGGLE_COVER_VIEW | 0x0119 |
| CMD_REPEAT_ON | 0x011A |
| CMD_REPEAT_OFF | 0x011B |
| CMD_TOGGLE_REPEAT | 0x011C |
| CMD_RANDOM_ON | 0x011D |
| CMD_RANDOM_OFF | 0x011E |
| CMD_TOGGLE_RANDOM | 0x011F |

#### Checksum

The checksum is computed as the sum of bytes 1 through 16, masked to 8 bits. Pi-side Python expression: sum(packet_bytes[1:17]) & 0xFF

#### 4.6 Heartbeat

Heartbeat command ID: CMD_SYS_HEARTBEAT = 0x0003

Current firmware compatibility note: the firmware also still accepts the legacy heartbeat command 0x9999.

Heartbeat behavior:

the ESP32 starts a heartbeat timeout monitor during initialization,

when a heartbeat packet is received, the action processor is notified,

if the timeout expires (30 seconds), the action processor is notified that the Pi is offline,

during power-on, absence of heartbeat for 60 seconds causes boot timeout.

#### Supported outbound command patterns:

- simple one-shot commands such as track controls,

- normalized toggle commands such as cover, repeat, meter, and random,

- CMD_ROTARY_ACTION with parameter 0 for left and 1 for right,

- CMD_SYS_RPI_SHUTDOWN for power sequencing.

Responses and inbound commands:

- heartbeat is fully handled,

- legacy heartbeat 0x9999 is still accepted,

- all other inbound messages are currently only logged.

UART protocol diagram:

sequenceDiagram
    participant Btn as Button Event
    participant Disp as Input Dispatcher
    participant Proc as ActionProcessor
    participant Uart as ActionUartDispatcher
    participant Tx as UartTransport
    participant Pi as Raspberry Pi

    Btn->>Disp: press or release
    Disp->>Proc: IAction
    Proc->>Uart: execute UART action
    alt toggle command
        Uart->>Tx: send normalized wire command + param
    else simple command
        Uart->>Tx: send raw command ID
    else rotary
        Uart->>Tx: send CMD_ROTARY_ACTION + direction
    end
    Tx->>Pi: UART frame
    Pi-->>Tx: heartbeat frame

Figure 9

### 9.2 I2C

- one bus on GPIO15/16,

- one target at address 0x20,

- 50 kHz clock rate,

- used only for MCP register access,

- interrupt-driven read-on-change, not periodic polling.

### 9.3 SPI

- SPI2 host,

- 1 MHz clock,

- MOSI-only data path,

- no chip-select line; latch is controlled manually on GPIO5,

- used only for button LED state output.

Data flow:

- ControlBoardInputDispatcher decides LED policy.

- ControlBoard::setButtonLed() calls SpiLedDriver::setLed().

- SpiLedDriver updates a 16-bit software bitmask.

- The driver transmits 16 bits over SPI and pulses the latch GPIO.

### 9.4 Network

No network communication interface exists in the current firmware. There is no Wi-Fi station setup, no AP mode, no sockets, no HTTP endpoints, and no MQTT client.

## 10. Data Structures

### 10.1 Key Runtime Types

| Type | Purpose | Key Members |
| --- | --- | --- |
| UartMessage | Wire-format message DTO | header fields, commandId, params[5], checksum |
| ButtonEvent | Queue payload from input callbacks | type, buttonId, rotaryDelta |
| SystemState | Minimal cross-component runtime state | atomic powerState |
| ButtonConfig | Maps a button slot to behavior | action, ledPolicy |
| ActionContext | Dependency bundle for IAction::execute() | UART dispatcher, power handler, relays, serial, state, LEDs, brightness |
| IAction | Executable command object | command, parameters, releaseTimeMillis, virtual execute() |
| IActionSource | Event-to-action factory | virtual produce(bool) |

### 10.2  Type Relationships

Figure 10

classDiagram
    class IActionSource {
        <<interface>>
        +produce(isPressed) unique_ptr~IAction~
    }

    class IAction {
        <<interface>>
        +CommandId command
        +uint16_t parameters[5]
        +uint16_t releaseTimeMillis
        +bool requiresPowerOn()
        +execute(ActionContext)
    }

    class ControlBoardInputDispatcher {
        +handleButtonPressed(id)
        +handleButtonReleased(id)
        +handleRotaryMovement(dir)
    }

    class ButtonEventQueue {
        +start(dispatcher)
        +enqueuePress(pin)
        +enqueueRelease(pin)
        +enqueueRotary(delta)
    }

    class ActionProcessor {
        +process(action)
        +handleInboundUartMessage(msg)
    }

    class ActionContext {
        +uartDispatcher
        +powerHandler
        +relayController
        +serial
        +systemState
        +powerLed
        +brightnessController
    }

    ButtonEventQueue --> ControlBoardInputDispatcher
    ControlBoardInputDispatcher --> IActionSource
    IActionSource --> IAction
    ActionProcessor --> ActionContext
    IAction --> ActionContext

### 10.3 Buffers and Queues

| Item | Size | Purpose |
| --- | --- | --- |
| ButtonEventQueue queue | 16 events | Absorb bursty button and rotary activity |
| StatusLed queue | 1 status value | Keep only latest UI status |
| UART temporary buffer | 64 bytes | Pull chunks from driver |
| UartReceiver buffer | 256 bytes | Assemble frames and resynchronize on stream errors |
| ESP-IDF UART RX ring | 512 bytes | Installed as bufferSize * 2 |

## 11. User Interface

### 11.1 Inputs

User-facing controls exposed by code:

| Button ID | Function | Behavior Type | Effect |
| --- | --- | --- | --- |
| 0 | Power | timed | power on, sleep, or deep sleep depending on hold duration and current state |
| 1 | Previous track | simple | UART command |
| 2 | Next track | simple | UART command |
| 3 | Skip forward | simple | UART command |
| 4 | Skip back | simple | UART command |
| 5 | Play/pause | simple | UART command |
| 6 | Toggle display | simple local command | toggles monitor brightness blanking mode |
| 7 | Cover | toggle | normalized UART toggle command |
| 8 | Repeat | toggle | normalized UART toggle command |
| 9 | Random | toggle | normalized UART toggle command |
| 10 | Toggle DAC | toggle | local relay control |
| 11 | Next panel | simple | UART command |
| 12 | Toggle meter | toggle | normalized UART toggle command |
| 13 | Rotary left event | rotary | UART rotary command param 0 |
| 14 | Rotary right event | rotary | UART rotary command param 1 |
| 15 | Cycle brightness | simple local command | cycles persisted brightness level |

### Full Button Mapping Table

For toggle-backed buttons, the action can emit one of two raw command IDs depending on current state, and the action processor may then normalize that into a different UART command.

| Index | Button Name | Action Type | SPI Bit Pattern | SPI TX (low, high) | Raw Command ID(s) | Final Routed Command | Final Effect |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Power | TimedAction | n/a | n/a | 0x0001 CMD_SYS_POWER | CMD_SYS_POWER | Enters ON, SLEEP, or DEEPSLEEP based on state + hold time; no SPI LED |
| 1 | Previous Track | Simple | 0000 0000 0000 0010 | 0x02 0x00 | 0x0101 CMD_PREVIOUS_TRACK | CMD_PREVIOUS_TRACK | UART command to Pi |
| 2 | Next Track | Simple | 0000 0000 0000 0100 | 0x04 0x00 | 0x0100 CMD_NEXT_TRACK | CMD_NEXT_TRACK | UART command to Pi |
| 3 | Skip Forward | Simple | 0000 0000 0000 1000 | 0x08 0x00 | 0x0104 CMD_SKIP_FORWARD | CMD_SKIP_FORWARD | UART command to Pi |
| 4 | Skip Back | Simple | 0000 0000 0001 0000 | 0x10 0x00 | 0x0105 CMD_SKIP_BACK | CMD_SKIP_BACK | UART command to Pi |
| 5 | Play/Pause | Simple | 0000 0000 0010 0000 | 0x20 0x00 | 0x0102 CMD_PLAY_PAUSE | CMD_PLAY_PAUSE | UART command to Pi |
| 6 | Stop | Simple | 0000 0000 0100 0000 | 0x40 0x00 | 0x0103 CMD_STOP_TRACK | CMD_STOP_TRACK | UART command to Pi |
| 7 | Cover | Toggle | 0000 0000 1000 0000 | 0x80 0x00 | 0x0117/0x0118 | 0x0119 CMD_TOGGLE_COVER_VIEW param 1/0 | Pi toggles cover view |
| 8 | Repeat | Toggle | 0000 0001 0000 0000 | 0x00 0x01 | 0x011A/0x011B | 0x011C CMD_TOGGLE_REPEAT param 1/0 | Pi toggles repeat mode |
| 9 | Toggle Random | Toggle | 0000 0010 0000 0000 | 0x00 0x02 | 0x011D/0x011E | 0x011F CMD_TOGGLE_RANDOM param 1/0 | Pi toggles random mode |
| 10 | Toggle DAC | Toggle | 0000 0100 0000 0000 | 0x00 0x04 | 0x010A/0x010F | local relay toggle only | Toggles DAC power relay |
| 11 | Next Panel | Simple | 0000 1000 0000 0000 | 0x00 0x08 | 0x0107 CMD_NEXT_MENU_ITEM | CMD_NEXT_MENU_ITEM | Pi cycles to next moOde Display panel |
| 12 | Toggle Meter | Toggle | 0001 0000 0000 0000 | 0x00 0x10 | 0x010C/0x010D | 0x0115 CMD_TOGGLE_METER param 1/0 | Display meter ON/OFF |
| 13 | Rotary Left | Rotary | n/a | n/a | 0x0112 CMD_ROTARY_ACTION | CMD_ROTARY_ACTION param 0 | Pi interprets as previous/left |
| 14 | Rotary Right | Rotary | n/a | n/a | 0x0112 CMD_ROTARY_ACTION | CMD_ROTARY_ACTION param 1 | Pi interprets as next/right |
| 15 | Cycle Brightness | Simple | 1000 0000 0000 0000 | 0x00 0x80 | 0x0116 CMD_CYCLE_BRIGHTNESS | local brightness cycle | Cycles monitor and button LED brightness |

### Per Button Notes

Power button (index 0):
- Press starts a timer; release emits CMD_SYS_POWER with releaseTimeMillis.
- Hold < 3000 ms → sleep path; hold ≥ 3000 ms → deep-sleep path.
- No corresponding SPI LED.

Toggle buttons (Cover, Repeat, Random, DAC, Meter):
- Keep internal software state in the action object.
- First press uses the action's default state; subsequent presses alternate ON/OFF.
- LED retention (keepLedActive) keeps the LED latched when the toggle state is ON.

Rotary (indices 13/14):
- Both directions use the same command ID; direction is carried in parameter 0.
- Rotary movement does not currently update a button LED.

SPI LED notes:
- Button 0 (power) is skipped — setLed() is not called for it; SPI LED bit 0 is unused in the normal button path.
- For all other buttons, ledIndex = buttonId.
- The SPI driver transmits the 16-bit register low byte first: byte 0 = bits 0–7, byte 1 = bits 8–15.
- If a toggle-backed LED is latched on, the transmitted SPI value will be the OR-combination of all active bits.

### 5.5 Local vs UART Actions

Actions that send UART commands to the Pi:
previous track, next track, skip forward, skip back, play/pause, stop, next menu, menu select, panel cycling, cover view toggle, meter toggle, rotary action, Raspberry Pi shutdown during power-down paths.

Actions that stay local on the ESP32:
power sequencing, DAC relay toggle, brightness cycling, relay shutdown sequencing, heartbeat wait and timeout handling.

### 11.2 LED Semantics

Power LED meanings:

- OFF: standby LED medium brightness.

- TURNING_ON: standby LED flashes.

- ON: active LED steady at configured duty.

- GOING_TO_SLEEP: standby LED flashes.

- SLEEP: standby LED breathes.

- DEEPSLEEP: standby LED blips periodically.

Activity LED meanings:

- doingWork: full brightness steady on.

- Active: short periodic blip.

- Idle: off.

- SolidIdle: steady idle duty.

- sleeping: brief blip every 10 seconds.

Button LED meanings:

- momentary buttons light while pressed,

- toggle buttons flip retained LED state on each press,

- power button and rotary do not use button LED feedback,

- boot wait and boot failure temporarily override all button LEDs through SpiBootIndicator.

### 11.3 User Interaction Flow

flowchart TD
    Start[User presses button or turns rotary] --> MCPIRQ[MCP interrupt]
    MCPIRQ --> Decode[Decode changed bits and rotary state]
    Decode --> Queue[Enqueue ButtonEvent]
    Queue --> Dispatch[ControlBoardInputDispatcher]
    Dispatch --> LED[Apply LED policy]
    Dispatch --> Action[Produce IAction]
    Action --> Exec[ActionProcessor executes]
    Exec --> Local[Relays or brightness]
    Exec --> Remote[UART command to Raspberry Pi]

Figure 11

### 11.4 Sleep-Mode Input Policy

When the background status is sleeping, all inputs except the power button are suppressed. This prevents accidental wake-path side effects from transport or panel buttons while the system is in sleep or deep sleep.

## 12. Error Handling

### 12.1 Error Condition Table

| Error | Cause | Detection | Recovery |
| --- | --- | --- | --- |
| NVS init failure | partition issues or version mismatch | nvs_flash_init() result | erase-and-retry for known page/version errors; otherwise log warning and continue without persistence |
| Relay setup or UART init failure | GPIO or driver errors | return value from bootstrap helpers | fail ControlBoard::init() and retry whole board init from app_main() |
| MCP init failure | I2C or GPIO interrupt setup failure | begin() returns error | fail init, flash failure indicator, retry from app_main() |
| Button queue full | bursty input or blocked consumer | xQueueSend() failure | drop event and log every 16th drop |
| UART checksum or framing error | corrupted or partial byte stream | deserializeMessage() false | byte-by-byte resynchronization in UartReceiver |
| No Pi heartbeat during boot | Pi absent or link down | waitForRpiToBoot() timeout | set boot-failure blink pattern, move to SLEEP, report init failure |
| No Pi heartbeat while ON | Pi crash, cable issue, or heartbeat loss | heartbeat_task timeout callback | set event bit and force SystemState to SLEEP unless debug simulate mode is enabled |
| SPI LED driver not initialized | premature caller use | m_started guard | log warning and ignore request |
| Status LED task creation failure | heap or RTOS failure | xTaskCreate() return | log error; corresponding indicator may not function |

### 12.2 Recovery Strategy Summary

- Initialization errors are mostly handled by failing ControlBoard::init() and letting app_main() retry forever.

- Runtime UART corruption is handled by frame resynchronization rather than transport reset.

- Heartbeat loss is treated as a semantic system-state event, not just a communication error.

- Several low-level errors only log and continue, especially LED and NVS failures.

### 12.3 Potential Bugs and Behavioral Risks

- UartTransport::sendData() checks PIN_ESP32_DATA_READY before transmit even though the log says it is testing whether the Raspberry Pi is ready. That condition appears to inspect the ESP32 output line rather than the Pi input-ready line, so the readiness guard is likely wrong.

- ActionProcessor::triggerInitialPowerOn() returns failure when boot heartbeat times out, and app_main() responds by retrying initialization indefinitely. On a bench without a Pi, this can create repeated power-cycle behavior rather than a stable degraded mode.

- StatusLed objects are global statics and call init() from their constructors. That means FreeRTOS objects and LEDC configuration are created during static initialization, which is an unusual lifetime model and increases startup-order risk.

- test/test_simple_command_action/test_main.cpp still targets an old ActionResponse/execute() API, which suggests the embedded test suite contains drifted or stale tests.

- PowerStateTransitionHandler::runRpiShutdownSequence() ignores the boolean result of waitForRpiShutdown(). The shutdown path proceeds even if no heartbeat-timeout confirmation ever arrives.

## 13. Logging and Diagnostics

The firmware uses ESP_LOGI, ESP_LOGW, and ESP_LOGE extensively. Logging is not centralized; each subsystem owns its own log tag.

Diagnostic capabilities visible in code:

- startup logs for each major initialization phase,

- relay and UART operation logs,

- heartbeat received and timeout logs,

- MCP interrupt and rotary decode logs,

- NVS restore/save logs for brightness,

- compatibility warning when legacy heartbeat 0x9999 is received.

Available test assets:

- host-side CMake test target for protocol, routing, and dispatcher logic,

- Unity-based embedded tests for UART protocol, SPI LED driver, SPI boot indicator, and power LED.

Diagnostic limitations:

- there is no structured fault manager,

- there are no persistent error counters,

- inbound UART ACK/NACK handling is not implemented,

- no watchdog-specific recovery behavior is present in project code,

- stale tests reduce confidence that all declared tests still represent the current runtime model.

Troubleshooting guidance for maintainers:

- verify GPIO18 and GPIO42 interrupts first when input or serial paths appear dead,

- inspect heartbeat timing before assuming a power-state bug,

- confirm board::debug::k_simulateRpiBoot build behavior before bench testing without a Pi,

- treat boot-failure SPI flashing and power LED state separately because they come from different subsystems.

## 14. Memory Usage

### 14.1 Allocation Model

Static allocation:

- global singleton-like indicator objects in ledManager.cpp,

- fixed-size buffers in UartReceiver and UartTransport,

- compile-time action registration table.

Dynamic allocation:

- std::make_unique for ActionProcessor, ControlBoardInputDispatcher, SerialUartCommandSink, ActionUartDispatcher, RpiBootManager, RelayController, and PowerStateTransitionHandler,

- std::make_unique for each registered IActionSource,

- one heap allocation per produced IAction via createAction(),

- FreeRTOS queues, tasks, software timers, and event groups,

- UART driver allocation inside ESP-IDF.

### 14.2 Risks

- Frequent short-lived action-object allocation can fragment heap over long runtimes if button usage is heavy.

- Multiple worker tasks with 4096-byte stacks consume nontrivial RAM on a device without PSRAM.

- The project does not expose any stack high-water-mark or heap telemetry.

- Static-initialization creation of StatusLed RTOS objects makes memory lifetime harder to reason about.

- Brightness persistence depends on NVS being available; if NVS is degraded the system silently falls back to nonpersistent behavior.

### 14.3 Optimization Opportunities

- Replace heap-allocated IAction objects with a fixed-capacity object pool or value-type command struct.

- Collapse the two StatusLed task implementations into a shared worker or more timer-driven design.

- Consider explicit stack watermark logging for the MCP, UART RX, and action tasks.

## 15. Performance Analysis

### 15.1 Timing-Critical Paths

- MCP interrupt handling, because it must preserve rapid button and rotary transitions.

- UART RX wake and frame parsing, because the data-ready handshake depends on timely consumption.

- Power sequencing, because it includes long waits and determines user-perceived boot behavior.

### 15.2 CPU and Latency Concerns

- The UART heartbeat task polls every 100 ms, which is cheap.

- Power transitions block in the caller context with vTaskDelay() calls ranging from hundreds of milliseconds to tens of seconds.

- SpiBootIndicator continuously toggles all LEDs with its own worker task, which is simple but not the cheapest way to generate patterns.

- PowerLed uses a 50 ms periodic esp_timer callback to maintain flash, breathe, and blip behavior.

### 15.3 Bottlenecks

- action_task is a single consumer for all input events.

- I2C reads are performed during every MCP interrupt handling pass and include multiple register reads.

- UartReceiver uses repeated memmove() while resynchronizing corrupted streams, which is acceptable at current protocol volume but not ideal for higher throughput.

### 15.4 Recommendations

- Fix the UART handshake readiness check before attempting higher-rate command traffic.

- Replace the hard-coded startup delays with explicit power-good or rail-ready signals if hardware supports them.

- Consider a nonblocking or state-machine-based power sequencer if future work adds more concurrent runtime behavior.

- If button traffic increases, move from heap-allocated actions to a fixed message structure.

## 16. Security Considerations

The firmware does not implement authentication, encryption, or secure session management. Security concerns are therefore limited to local robustness and input validation.

Observed protections:

- UART frame checksum validation,

- start-byte framing and resynchronization,

- bounds checks on LED indices and button IDs,

- fixed-size queues and buffers,

- command gating so most non-power actions are ignored when the system is not ON.

Observed gaps:

- any device on the UART link that can emit valid frames can control the Raspberry Pi-facing command path,

- no authorization layer exists for system or relay-affecting commands,

- no replay protection or sequence validation is enforced,

- no tamper detection exists on I2C or GPIO inputs.

## 17. Build System

### 17.1 PlatformIO Configuration

- Platform: espressif32 @ ~6.5.0

- Framework: espidf

- Default environment: esp32-s3-devkitc-1-16mb

- Release environment: esp32-s3-devkitc-1-16mb-release

- Monitor speed: 115200

- CPU frequency: 240000000L

- Partition table: partitions.csv

- Library dependency finder mode: deep+

- Shared build flag: -Ilib

- Release-only extra flag: -DNDEBUG

### 17.2 CMake Structure

- Top-level CMakeLists.txt registers an ESP-IDF project.

- src/CMakeLists.txt glob-registers all .c and .cpp files in src/ and major lib/ subtrees.

- host_tests/CMakeLists.txt builds a separate host executable for logic tests on a desktop compiler.

### 17.3 Build Behavior

- Debug builds enable board::debug::k_simulateRpiBoot unless overridden, which changes boot behavior materially.

- Release builds define NDEBUG, which disables simulated-RPi boot by default.

## 18. Dependency Analysis

graph TD
    Firmware[TinyControlBoard firmware]
    Firmware --> App[app layer]
    Firmware --> HAL[hal layer]
    Firmware --> Indicators[indicator layer]
    Firmware --> Power[power layer]
    Firmware --> Protocol[protocol layer]
    Firmware --> Board[board config layer]

    App --> Protocol
    App --> Power
    App --> Indicators
    App --> HAL
    HAL --> Board
    Indicators --> HAL
    Indicators --> Power
    Power --> HAL
    Power --> Indicators

    HAL --> ESPIDF[ESP-IDF drivers and FreeRTOS]
    Indicators --> ESPIDF
    Power --> ESPIDF
    Protocol --> StandardCPP[C++ standard library]
    App --> StandardCPP

    HAL --> Hardware[Relays, MCP expander, UART, SPI LEDs]
    Protocol --> RPi[Raspberry Pi protocol peer]

External dependencies actually used by code:

- ESP-IDF FreeRTOS primitives,

- ESP-IDF UART, GPIO, I2C, SPI, LEDC, NVS, and esp_timer APIs,

- C++ standard library features such as std::unique_ptr, std::function, std::array, and std::atomic.

## 19. Maintenance Guide

### 19.1 Adding a New Front-Panel Feature

For a new button-driven command:

- Add the new CommandId in uartProtocol.hpp if it needs a protocol ID.

- If it is a simple remote command, add one row to ControlBoardActionRegistry.cpp.

- If it is a local-only action, also update ActionCommandRoutingPolicy.hpp and potentially ActionFactory.cpp if a new action class is needed.

- If it is a toggle that should normalize to one wire command plus a boolean parameter, update ActionUartDispatcher.cpp.

- Update tests for dispatcher, protocol, and any new runtime policy.

### 19.2 Adding a New Peripheral

- Add pins and constants to boardConfig.hpp.

- Encapsulate the peripheral in lib/hal/ or lib/indicators/ depending on whether it is generic hardware or UI-specific.

- Expose it through ControlBoard only if it participates in application behavior.

- Avoid direct ISR-to-policy coupling; follow the existing callback plus queue pattern.

### 19.3 Modifying the UART Protocol

- Update CommandId or frame semantics in uartProtocol.hpp and uartProtocol.cpp.

- Update command naming in commandCatalog.cpp.

- Update ActionUartDispatcher.cpp if routing or normalization changes.

- Update both host tests and Unity protocol tests.

- Keep Raspberry Pi-side handlers synchronized, especially for normalized toggle commands and heartbeat IDs.

### 19.4 Areas Requiring Caution

- Do not change power-state timing casually; it affects relay sequencing, user-visible LEDs, and Raspberry Pi boot assumptions.

- Be careful with static object construction in ledManager.cpp; indicator lifetime is unusual and tightly coupled to runtime startup.

- Avoid broadening ISR responsibilities. The current design depends on tasks doing the real work.

- Treat debug-mode simulated Pi boot as a behavioral fork, not just a logging option.

- Verify embedded tests before trusting them; at least one test target is stale relative to the runtime action API.

## 20. Appendix

### 20.1 File Structure

#### src

| File | Purpose |
| --- | --- |
| src/main.cpp | Firmware entry point, PM configuration, startup delay, init retry loop. |
| src/main.h | Minimal include wrapper that pulls ControlBoard into main.cpp. |

#### lib/app

| File | Purpose |
| --- | --- |
| lib/app/ActionCommandRoute.hpp | Enum describing intrinsic command routing categories. |
| lib/app/ActionCommandRoutingPolicy.hpp | Compile-time policy that maps command IDs to routing categories. |
| lib/app/ActionContext.hpp | Dependency bundle passed into IAction::execute(). |
| lib/app/ActionFactory.cpp | Creates concrete IAction implementations from command IDs. |
| lib/app/ActionFactory.hpp | Factory declaration for command-to-action creation. |
| lib/app/actionProcessor.cpp | Executes actions, enforces power gating, handles heartbeat, scaffolds inbound UART handling. |
| lib/app/actionProcessor.hpp | ActionProcessor public interface. |
| lib/app/ActionUartDispatcher.cpp | Normalizes and sends UART-facing actions. |
| lib/app/ActionUartDispatcher.hpp | ActionUartDispatcher and UART sink interface declarations. |
| lib/app/ButtonEvent.hpp | Queue payload type for press, release, and rotary events. |
| lib/app/ButtonEventQueue.cpp | FreeRTOS queue/task wrapper for serialized input dispatch. |
| lib/app/ButtonEventQueue.hpp | ButtonEventQueue declaration and queue/task constants. |
| lib/app/ControlBoard.cpp | Top-level orchestrator implementation and UART callback owner. |
| lib/app/ControlBoard.hpp | ControlBoard declaration and owned subsystem references. |
| lib/app/ControlBoardActionRegistry.cpp | Populates the button-to-action map from a registration table. |
| lib/app/ControlBoardActionRegistry.hpp | Registry declaration. |
| lib/app/ControlBoardBootstrap.cpp | Boot helper functions for relays, serial, MCP, and indicators. |
| lib/app/ControlBoardBootstrap.hpp | Bootstrap helper declarations and callback typedefs. |
| lib/app/ControlBoardButtonIds.hpp | Namespace alias from board button IDs into app code. |
| lib/app/ControlBoardInputDispatcher.cpp | Applies sleep suppression, LED policy, and action production for inputs. |
| lib/app/ControlBoardInputDispatcher.hpp | Dispatcher interfaces, LED policy enum, and action-map types. |
| lib/app/SerialHeartbeatRouter.hpp | Heartbeat compatibility constants and helper predicate. |
| lib/app/SerialUartCommandSink.cpp | Thin adapter from command sink interface to UartTransport. |
| lib/app/SerialUartCommandSink.hpp | SerialUartCommandSink declaration. |
| lib/app/SystemState.hpp | Atomic runtime power-state container. |

#### lib/app/commands

| File | Purpose |
| --- | --- |
| lib/app/commands/BrightnessAction.cpp | Executes local display-off and brightness-cycle commands. |
| lib/app/commands/BrightnessAction.hpp | Brightness action declaration. |
| lib/app/commands/PowerTransitionAction.cpp | Executes power-state transitions through the power handler. |
| lib/app/commands/PowerTransitionAction.hpp | Power transition action declaration. |
| lib/app/commands/RelayAction.cpp | Executes local DAC relay toggles. |
| lib/app/commands/RelayAction.hpp | Relay action declaration. |
| lib/app/commands/SystemAction.cpp | Executes local system commands such as Pi shutdown. |
| lib/app/commands/SystemAction.hpp | System action declaration. |
| lib/app/commands/UartDispatchAction.cpp | Executes remote-command dispatch via ActionUartDispatcher. |
| lib/app/commands/UartDispatchAction.hpp | UART dispatch action declaration. |

#### lib/board

| File | Purpose |
| --- | --- |
| lib/board/boardButtonIds.hpp | Canonical button and rotary ID assignments. |
| lib/board/boardConfig.hpp | Timing, serial, I2C, relay, indicator, and debug configuration constants. |
| lib/board/boardIdentity.hpp | Board identity strings such as model and firmware version. |

#### lib/hal/buttons

| File | Purpose |
| --- | --- |
| lib/hal/buttons/mcpInputHandler.cpp | MCP input expander driver, interrupt-task loop, and rotary decode. |
| lib/hal/buttons/mcpInputHandler.hpp | MCP register constants, callbacks, and class declaration. |
| lib/hal/buttons/project_cfg.hpp | Optional MCP debug-scan compile-time flag holder. |

#### lib/hal/leds

| File | Purpose |
| --- | --- |
| lib/hal/leds/pwmLed.cpp | Small LEDC wrapper implementation. |
| lib/hal/leds/pwmLed.hpp | LedPwm abstraction declaration. |
| lib/hal/leds/spiLedDriver.cpp | SPI-backed 16-bit LED driver implementation. |
| lib/hal/leds/spiLedDriver.hpp | SPI LED driver declaration and LED count constant. |

#### lib/hal/relay

| File | Purpose |
| --- | --- |
| lib/hal/relay/relay.cpp | GPIO-based relay initialization and state setting. |
| lib/hal/relay/relay.hpp | Relay pin aliases and StandardRelay singleton interface. |

#### lib/hal/storage

| File | Purpose |
| --- | --- |
| lib/hal/storage/nvsStorage.cpp | Generic NVS read/write wrapper implementation. |
| lib/hal/storage/nvsStorage.hpp | NVS wrapper declaration for typed key-value access. |

#### lib/hal/uart

| File | Purpose |
| --- | --- |
| lib/hal/uart/serial.cpp | UART transport implementation, GPIO handshake, RX task, and heartbeat monitor. |
| lib/hal/uart/serial.hpp | UART transport declaration, pin aliases, and buffer members. |
| lib/hal/uart/uartReceiver.cpp | Byte-stream framing and resynchronization logic. |
| lib/hal/uart/uartReceiver.hpp | UartReceiver declaration and fixed buffer sizes. |

#### lib/indicators

| File | Purpose |
| --- | --- |
| lib/indicators/activityStatus.hpp | Working-status enum and activity-status sink interface. |
| lib/indicators/ledDefinitions.hpp | LEDC channel and timing macros. |
| lib/indicators/ledManager.cpp | Singleton-style construction and lazy init accessors for indicators. |
| lib/indicators/ledManager.hpp | Indicator accessor declarations. |
| lib/indicators/MonitorBrightnessController.cpp | Brightness persistence, display-off mode, and PWM updates. |
| lib/indicators/monitorBrightnessController.hpp | Brightness controller declaration and state members. |
| lib/indicators/powerLed.cpp | Dual-LED power-state indicator with timer-driven patterns. |
| lib/indicators/powerLed.hpp | Power LED class declaration. |
| lib/indicators/SpiBootIndicator.cpp | Boot/failure flashing task over SPI button LEDs. |
| lib/indicators/SpiBootIndicator.hpp | SPI boot indicator declaration and state enum. |
| lib/indicators/statusLed.cpp | Queue-driven status LED controller with blink and breathe helpers. |
| lib/indicators/statusLed.hpp | Status LED declaration. |

#### lib/input

| File | Purpose |
| --- | --- |
| lib/input/input.hpp | Aggregation header for input-related includes. |

#### lib/input/actions

| File | Purpose |
| --- | --- |
| lib/input/actions/actionsResponse.cpp | Empty implementation unit retained for build compatibility. |
| lib/input/actions/actionsResponse.hpp | Test-helper no-op Action implementation. |
| lib/input/actions/actionTemplates.hpp | Live IActionSource implementations for simple, toggle, timed, and rotary behavior. |
| lib/input/actions/buttonAction.hpp | IActionSource interface declaration. |
| lib/input/actions/IAction.hpp | Core executable action interface and payload fields. |
| lib/input/actions/SimpleCommandAction.hpp | Backward-compatibility include shim for the simple action source. |

#### lib/power

| File | Purpose |
| --- | --- |
| lib/power/powerState.hpp | Power-state enum. |
| lib/power/PowerStateTransitionHandler.cpp | Boot, sleep, and deep-sleep sequencing logic. |
| lib/power/PowerStateTransitionHandler.hpp | Power transition handler declaration. |
| lib/power/PowerStateTransitionPolicy.hpp | Power-button decision policy based on current state and hold time. |
| lib/power/RelayController.cpp | Relay-sequencing helper implementation. |
| lib/power/RelayController.hpp | Relay controller declaration. |
| lib/power/RPIBootManager.cpp | Event-group-based boot and shutdown wait logic. |
| lib/power/RPIBootManager.hpp | Boot manager declaration and event bits. |

#### lib/protocol

| File | Purpose |
| --- | --- |
| lib/protocol/commandCatalog.cpp | Maps command IDs to names and simple UART log tags. |
| lib/protocol/commandCatalog.hpp | Command-catalog lookup declarations. |
| lib/protocol/uartProtocol.cpp | UART serialization, deserialization, and checksum implementation. |
| lib/protocol/uartProtocol.hpp | Protocol constants, enums, and UartMessage definition. |

#### host_tests

| File | Purpose |
| --- | --- |
| host_tests/host_test_action_stubs.cpp | Desktop-link stubs for action execute methods. |
| host_tests/test_logic.cpp | Host-side unit tests for protocol, routing, dispatcher, and heartbeat helpers. |

#### test

| File | Purpose |
| --- | --- |
| test/test_power_led/test_main.cpp | Unity tests for power LED constructor defaults and brightness clamping. |
| test/test_simple_command_action/test_main.cpp | Embedded test targeting an older simple-action API; appears stale relative to current code. |
| test/test_spi_boot_indicator/test_main.cpp | Unity tests for SPI boot-indicator state and task behavior. |
| test/test_spi_led_driver/test_main.cpp | Unity tests for SPI LED driver guards and constants. |
| test/test_uart_protocol/test_main.cpp | Unity tests for UART message defaults, serialization, and checksum validation. |

### 20.2 Configuration Parameters

| Parameter | Value | Meaning |
| --- | --- | --- |
| board::timing::k_heartbeatTimeoutMs | 30000 ms | Loss-of-heartbeat threshold |
| board::timing::k_initDelayMs | 5000 ms | Additional startup settle delay in bootstrap |
| board::timing::k_powerSettleDelayMs | 1500 ms | Delay after enabling DAC/output-stage rails |
| board::timing::k_screenOnDelayMs | 1000 ms | Screen and Pi power-on delay |
| board::timing::k_rpiBootTimeoutMs | 60000 ms | Max wait for boot heartbeat |
| board::timing::k_rpiShutdownTimeoutMs | 60000 ms | Max wait for shutdown confirmation |
| board::timing::k_rpiShutdownSettleDelayMs | 500 ms | Delay after Pi relay off |
| board::timing::k_screenPowerOffDelayMs | 5000 ms | Delay before sleep finalization |
| board::serial::k_baudRate | 115200 | UART link speed |
| board::serial::k_bufferSize | 256 bytes | Requested UART buffer size |
| board::i2c::k_clockSpeedHz | 50000 Hz | I2C bus speed |
| board::i2c::k_mcpTimeoutMs | 10 ms | I2C command timeout |
| controlSystem::kLongPressThresholdMs | 3000 ms | Power short-vs-long press threshold |

### 20.3 Important Constants and Definitions

| Constant | Value | Notes |
| --- | --- | --- |
| UART_START_BYTE | 0xAA | Start of frame |
| UART_PROTOCOL_VERSION | 0x01 | Protocol version |
| UART_PACKET_SIZE | 18 | Fixed frame length |
| APP_ESP32 | 0x01 | Source app ID |
| APP_PI | 0x02 | Peer app ID |
| MSG_COMMAND | 0x01 | UART message type |
| MSG_STATUS | 0x02 | UART message type |
| MSG_ACK | 0x03 | UART message type |
| MSG_NACK | 0x04 | UART message type |
| board::debug::k_simulateRpiBoot | build-dependent | true by default in debug, false by default in release |
| kLegacyHeartbeatCommandId | 0x9999 | Backward compatibility heartbeat alias |

### 20.4 Summary of Highest-Value Follow-Up Work

- Fix the UART handshake readiness check so transmit gating uses the correct peer-ready signal.

- Decide whether missing Pi heartbeat at boot should produce a stable degraded mode instead of perpetual init retries.

- Replace or validate stale embedded tests, especially the simple-command action test target.

- Consolidate command-routing rules so adding a new command does not require touching multiple disconnected tables.

- Remove or explicitly classify dormant helper code to reduce ambiguity for future maintainers.