PROJECT DOCUMENTATION PACK

TinyControlBoard Documentation

Consolidated engineering reference and implementation guide

Generated 2026-04-27 19:56

Included source documents

docs/project-guide.md

docs/architecture.md

docs/wiring-reference.md

docs/protocol-reference.md

docs/power-sequencing.md

docs/raspberry-pi-setup.md

scripts/rpi/README.md

docs/button-command-map.md

docs/add-button-how-to.md

docs/project-structure-plan.md

# Table Of Contents

Update fields in Word to populate the table of contents.

Source: docs/project-guide.md

# TinyControlBoard Project Guide

This is the landing page for the TinyControlBoard documentation set.

The goal is to explain the project in plain language first, then point to the source files that own each part of the behavior. It is intended to grow over time as the hardware, firmware, and Raspberry Pi side evolve.

## Documentation Set

Use this page as the overview, then jump into the focused reference docs as needed.

Core documents:

docs/architecture.md

docs/add-button-how-to.md

docs/button-command-map.md

docs/power-sequencing.md

docs/wiring-reference.md

docs/protocol-reference.md

docs/raspberry-pi-setup.md

docs/project-structure-plan.md

## 1. What This Project Does

TinyControlBoard is an ESP32-S3-based control surface that:

reads physical buttons and rotary input,

controls relays and indicator outputs,

talks to a Raspberry Pi over UART,

reacts to Raspberry Pi heartbeat status,

translates user input into commands such as playback control, display control, DAC control, and power-state changes.

In practical terms, the board is the hardware front end and the Raspberry Pi is the system it controls.

## 2. System At A Glance

The main runtime flow is:

The ESP32 boots and waits briefly for power to settle.

app_main() creates a ControlBoard instance.

ControlBoard::init() initializes LEDs, relays, input handling, and serial communication.

Button and rotary events are converted into ActionResponse objects.

ActionProcessor decides what to do with those responses.

Some actions change local hardware state, and others send UART commands to the Raspberry Pi.

The Raspberry Pi sends heartbeat packets back so the ESP32 knows the Pi is still online.

Primary entry point:

src/main.cpp

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

## 3. Core Runtime Pieces

### 3.1 main.cpp

app_main() is intentionally small. It delays briefly, creates the control board object, initializes it, and then keeps the firmware alive.

Reference:

src/main.cpp

### 3.2 ControlBoard

ControlBoard is the top-level orchestrator for the firmware. It wires together the major subsystems and owns the event callbacks for buttons, rotary movement, and received UART messages.

Responsibilities include:

setting initial LED and brightness state,

initializing the serial bus singleton,

starting heartbeat monitoring,

initializing relays,

initializing the MCP input handler,

creating the button-to-action map,

forwarding action results to the action processor.

References:

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

### 3.3 ActionProcessor

ActionProcessor is the decision layer between input events and side effects. It receives ActionResponse objects and performs the next step, such as:

changing relays,

coordinating Raspberry Pi boot and shutdown behavior,

sending UART commands,

reacting to heartbeat received or heartbeat timeout.

References:

lib/app/actionProcessor.hpp

### 3.4 Serial Bus

The serial layer manages UART communication with the Raspberry Pi. It also manages the extra GPIO handshake lines used to signal when data is ready on either side.

Main responsibilities:

configure UART,

configure GPIO interrupt/output handshake pins,

send UART packets,

receive UART packets,

track heartbeat timing,

invoke a callback when a complete message is received.

References:

lib/transport/uart/serial.hpp

lib/transport/uart/serial.cpp

### 3.5 Protocol Layer

The protocol layer defines the wire format used between the ESP32 and the Raspberry Pi.

It owns:

packet size,

start byte,

message layout,

command IDs,

app IDs,

message structure,

serialization and deserialization entry points.

Reference:

lib/protocol/uartProtocol.hpp

## 4. Input, Outputs, And Hardware Roles

### 4.1 Inputs

User input appears to come primarily from an MCP23018-based input expander and rotary input handling.

Relevant code:

lib/input/buttons/mcpInputHandler.hpp

lib/input/buttons/mcpInputHandler.cpp

The button index mapping currently lives in:

lib/board/boardConfig.hpp

### 4.2 Relays And Power Control

The board controls multiple relay outputs for major subsystems, including:

screen power,

DAC power,

Raspberry Pi power,

output stage power,

prototype DAC enable,

general-purpose relay outputs.

The current relay pin assignments live in:

lib/board/boardConfig.hpp

Related code:

lib/relays/relay.hpp

lib/power/RelayController.hpp

lib/power/RPIBootManager.hpp

### 4.3 Indicators

The board also drives visual feedback hardware, including status LEDs, a power LED, button lighting, SPI LED driving, and monitor brightness control.

Relevant code lives mainly under:

lib/indicators

lib/led

## 5. Current Hardware Configuration Reference

The main hardware constants are centralized in:

lib/board/boardConfig.hpp

### 5.1 UART And Handshake Pins On The ESP32-S3

Current firmware pin assignments:

| Function | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| UART port | UART_NUM_2 | Main Pi communication port |
| UART TX | GPIO2 | ESP32 -> Pi UART data |
| UART RX | GPIO1 | Pi -> ESP32 UART data |
| Pi data-ready input | GPIO42 | Pi pulses this to tell ESP32 data is available |
| ESP32 data-ready output | GPIO41 | ESP32 pulses this to tell Pi data is available |

### 5.2 Raspberry Pi Side Wiring Reference

The Raspberry Pi companion scripts currently use:

| Function | Raspberry Pi BCM | Physical Pin | Notes |
| --- | --- | --- | --- |
| ESP32 -> Pi data-ready | 23 | 16 | Pi listener waits for rising edge here |
| Pi -> ESP32 data-ready | 24 | 18 | Pi sender script pulses this line |
| UART5 TX | 12 | 32 | Standard dtoverlay=uart5 mapping |
| UART5 RX | 13 | 33 | Standard dtoverlay=uart5 mapping |

Important wiring summary:

Pi BCM23 / physical pin 16 connects to ESP32 GPIO41.

Pi BCM24 / physical pin 18 connects to ESP32 GPIO42.

Pi UART5 TX connects to ESP32 GPIO1.

Pi UART5 RX connects to ESP32 GPIO2.

Grounds must be common between both boards.

Raspberry Pi companion references:

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

scripts/rpi/README.md

## 6. UART Protocol Summary

The firmware and Raspberry Pi use an 18-byte packet format.

At a high level, a message contains:

a start byte,

protocol version,

source application ID,

message type,

sequence number,

16-bit command ID,

five 16-bit parameters,

checksum.

Important protocol facts:

start byte is 0xAA,

packet size is 18 bytes,

the protocol supports command, status, ACK, and NACK message types,

heartbeat is command ID 0x0003.

Current command IDs are defined in:

lib/protocol/uartProtocol.hpp

Examples include:

system power,

Raspberry Pi shutdown,

heartbeat,

next/previous track,

play/pause,

stop,

skip forward/back,

DAC toggle,

display toggle,

meter toggle,

cover view toggle,

rotary action.

## 7. Heartbeat Behavior

Heartbeat exists so the ESP32 can detect whether the Raspberry Pi is still alive.

Current behavior in the code:

the ESP32 starts a heartbeat timeout monitor during initialization,

when a heartbeat packet is received, the action processor is notified,

if the timeout expires, the action processor is notified that the Pi is offline,

the code currently accepts the current heartbeat command ID and also a legacy heartbeat ID 0x9999.

Relevant references:

lib/app/ControlBoard.cpp

lib/transport/uart/serial.hpp

scripts/rpi/home/antho/heartbeat_sender.py

## 8. Build And Test Reference

### 8.1 Firmware Build

The verified workspace build path is the VS Code task:

PlatformIO Build

Top-level configuration files:

platformio.ini

CMakeLists.txt

### 8.2 Host-Side Tests

There is also a separate host-side test setup under:

host_tests

Current note from repository context:

host tests can be run with CMake/MSVC on this machine,

PlatformIO native is not currently usable here without a GCC-compatible compiler in PATH.

### 8.3 In-Repo Test Areas

Current test folders include:

test/test_power_led

test/test_simple_command_action

test/test_uart_protocol

## 9. Folder Guide

This is a simple description of the current project layout.

| Area | Purpose |
| --- | --- |
| src/ | firmware entry point |
| lib/board/ | board-specific constants and identity |
| lib/app/ | orchestration and top-level runtime composition |
| lib/input/buttons/ | button and input-expander handling |
| lib/input/actions/ | action definitions and action results |
| lib/transport/uart/ | UART transport and handshake handling |
| lib/protocol/ | packet definitions and command IDs |
| lib/indicators/ | LED/status/brightness behavior |
| lib/power/ | power lifecycle and shutdown coordination |
| lib/relays/ | relay abstraction |
| scripts/rpi/ | Raspberry Pi listener, sender, and setup docs |
| docs/ | project documentation |
| host_tests/ | host-based test project |
| test/ | embedded/unit-oriented test directories |

Also relevant:

docs/project-structure-plan.md describes the remaining cleanup after the transport and script migration work.

## 10. Documentation Roadmap

The project now has focused documents for architecture, wiring, protocol, Raspberry Pi setup, power sequencing, and button mapping.

The next useful additions would be:

a relay-by-relay electrical purpose reference,

a troubleshooting matrix for heartbeat and UART failures,

a deployment checklist for production hardware,

photos or diagrams for the physical build.

## 11. Known Gaps And Follow-Up Topics

This section is intentionally here so the document can grow with the project.

Suggested next sections to add later:

Full button-to-command mapping table.

Power-state behavior and startup/shutdown sequencing.

LED behavior reference with screenshots or state tables.

Detailed relay ownership and electrical purpose.

Complete UART command catalog with sender, receiver, and expected effect.

Raspberry Pi service setup summary for production deployment.

Troubleshooting guide for wiring, heartbeat, and UART failures.

## 12. Quick Reference

If you only need the shortest summary:

main.cpp starts ControlBoard.

ControlBoard initializes the hardware and registers callbacks.

button and rotary events become ActionResponse objects.

ActionProcessor converts those responses into hardware actions and UART commands.

Serial moves packets between the ESP32 and Raspberry Pi.

uartProtocol.hpp defines what those packets look like.

Raspberry Pi scripts in scripts/rpi/ implement the current Pi-side listener and heartbeat sender.

Source: docs/architecture.md

# TinyControlBoard Architecture

This document explains how the current firmware is structured and how control flows through the system.

## 1. High-Level View

The firmware is built around one top-level orchestrator: ControlBoard.

At runtime, the system does four main jobs:

initialize the board and connected hardware,

read user input from buttons and rotary input,

convert those inputs into local actions or UART commands,

monitor the Raspberry Pi link and react to heartbeat state.

Core entry points:

src/main.cpp

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

## 2. Startup Flow

Current startup sequence:

app_main() waits 5 seconds for power to settle.

A ControlBoard instance is created.

ControlBoard::init() sets initial LED state and brightness.

The serial singleton and relay singleton are acquired.

The serial receive callback is registered.

Heartbeat monitoring is started.

The action processor is created.

Relays are initialized and set to default off states.

The MCP input handler is initialized.

Button and rotary callbacks are registered.

UART is initialized.

The button-to-action map is created.

After kInitDelayMs, the power LED is moved to SLEEP.

References:

src/main.cpp

lib/app/ControlBoard.cpp

lib/board/boardConfig.hpp

## 3. Main Components

### 3.1 ControlBoard

ControlBoard is the integration layer. It owns callback registration and connects the input, serial, relay, and action-processing subsystems.

Main responsibilities:

initialize relays,

initialize UART and serial callbacks,

initialize the MCP input handler,

route button press, button release, and rotary events,

route incoming UART messages,

maintain the button-action lookup table.

References:

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

### 3.2 ActionProcessor

ActionProcessor takes an ActionResponse and decides what side effect should happen.

That can include:

local relay changes,

local brightness changes,

sending UART commands to the Raspberry Pi,

handling power-state transitions,

waiting for Raspberry Pi heartbeat during boot/shutdown coordination.

Reference:

lib/app/actionProcessor.hpp

lib/app/actionProcessor.cpp

### 3.3 Serial

The serial layer owns the ESP32 side of the Pi link.

It does three distinct jobs:

configure and use UART2,

manage the two extra data-ready handshake GPIOs,

track heartbeat timing and pass completed messages up through a callback.

References:

lib/transport/uart/serial.hpp

lib/transport/uart/serial.cpp

### 3.4 RpiBootManager

RpiBootManager is a synchronization helper built around a FreeRTOS event group.

It does not directly control hardware. Instead, it waits for lifecycle signals:

heartbeat received means the Pi is alive or has finished booting,

heartbeat timeout is treated as shutdown/offline confirmation.

References:

lib/power/RPIBootManager.hpp

lib/power/RPIBootManager.cpp

### 3.5 RelayController

RelayController is a thin helper around the relay abstraction. It currently provides:

relay state changes with optional delay,

DAC relay toggling,

Raspberry Pi relay shutdown,

screen relay shutdown.

References:

lib/power/RelayController.hpp

lib/power/RelayController.cpp

## 4. Input Flow

The current input path is:

MCP input or rotary movement is detected.

ControlBoard callback receives the event.

The button index is used to find a ButtonAction object.

The action returns an ActionResponse.

ActionProcessor::process() applies the result.

Related files:

lib/input/buttons/mcpInputHandler.hpp

lib/input/buttons/mcpInputHandler.cpp

lib/input/actions/buttonActions.hpp

lib/input/actions/buttonActions.cpp

lib/input/actions/actionTemplates.hpp

lib/input/actions/actionsResponse.hpp

## 5. UART Flow

The current ESP32 -> Pi path is:

An action processor branch decides to send a command.

A UartMessage or command ID is passed to Serial.

Serial::sendData() writes bytes on UART.

The ESP32 raises its data-ready pin briefly so the Pi knows to read.

The Pi -> ESP32 path is:

The Raspberry Pi writes a UART packet.

The Pi pulses its data-ready line.

The ESP32 ISR wakes the UART RX task.

Incoming bytes are added to the receiver buffer.

Complete messages are deserialized and passed to the registered callback.

Important current limitation:

ControlBoard::handleSerialRxMessage() actively handles heartbeat messages.

Other received UART messages are logged but are not yet processed into local behavior.

References:

lib/transport/uart/serial.cpp

lib/app/ControlBoard.cpp

lib/protocol/uartProtocol.hpp

## 6. State And Status Model

### 6.1 Power State

The main power LED state model currently includes:

OFF

TURNING_ON

ON

SLEEP

GOING_TO_SLEEP

DEEPSLEEP

Reference:

lib/power/powerState.hpp

### 6.2 Working Status

The activity/status LED model currently includes:

doingWork

Idle

SolidIdle

sleeping

MaintenanceMode

Active

Reference:

lib/indicators/activityStatus.hpp

## 7. Folder Ownership

This is the practical ownership model for the current codebase.

| Area | Current Role |
| --- | --- |
| src/ | firmware entry point |
| lib/board/ | board constants and identity |
| lib/app/ | orchestration and integration layer |
| lib/input/actions/ | reusable button action objects and templates |
| lib/input/buttons/ | input expander and input capture |
| lib/transport/uart/ | UART transport and handshake logic |
| lib/protocol/ | wire format and command IDs |
| lib/indicators/ | LEDs, status behavior, brightness control |
| lib/power/ | power state, relay sequencing, and Pi boot/shutdown coordination |
| lib/relays/ | low-level relay abstraction |
| scripts/rpi/ | Raspberry Pi listener, sender, and setup docs |

## 8. Notable Current Design Characteristics

ControlBoard is the integration hub and currently owns a lot of orchestration.

Input actions are represented as objects, which makes it straightforward to remap buttons without rewriting processor logic.

Toggle actions maintain internal software state, so their first emitted command depends on the starting state in firmware.

Heartbeat is treated as the Raspberry Pi liveness signal for both boot completion and shutdown detection.

Non-heartbeat Pi-originated commands are not yet fully consumed on the ESP32 side.

## 9. Related Docs

docs/project-guide.md

docs/button-command-map.md

docs/power-sequencing.md

docs/wiring-reference.md

docs/protocol-reference.md

docs/raspberry-pi-setup.md

Source: docs/wiring-reference.md

# Wiring Reference

This document collects the current known wiring and pin assignments for the ESP32-S3 firmware side and the Raspberry Pi companion side.

It also mirrors the button-to-command and button-to-SPI-LED mapping so the firmware view can be compared directly against the physical schematic blocks for button inputs and button LED drive.

Current hardware schematic image:

TinyControlBoard schematic

Primary source:

lib/board/boardConfig.hpp

Related Raspberry Pi references:

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

## 1. ESP32-S3 Pin Assignments

### 1.1 UART And Handshake

| Function | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| UART port | UART_NUM_2 | main Raspberry Pi link |
| UART TX | GPIO2 | ESP32 -> Pi data |
| UART RX | GPIO1 | Pi -> ESP32 data |
| Pi data-ready input | GPIO42 | Pi tells ESP32 UART data is ready |
| ESP32 data-ready output | GPIO41 | ESP32 tells Pi UART data is ready |

### 1.2 I2C And Input Expander

| Function | ESP32-S3 Pin |
| --- | --- |
| I2C SCL | GPIO15 |
| I2C SDA | GPIO16 |
| MCP interrupt | GPIO18 |
| MCP enable | GPIO17 |
| MCP address | 0x20 |

### 1.3 Relay Outputs

| Function | ESP32-S3 Pin |
| --- | --- |
| Screen power relay | GPIO13 |
| DAC power relay | GPIO12 |
| Raspberry Pi power relay | GPIO11 |
| Output stage power relay | GPIO9 |
| Prototype DAC enable relay | GPIO10 |
| General relay 1 | GPIO47 |
| General relay 2 | GPIO39 |

### 1.4 Indicator Outputs

| Function | ESP32-S3 Pin |
| --- | --- |
| App active LED | GPIO3 |
| App standby LED | GPIO4 |
| Working status LED | GPIO48 |
| Button LED PWM | GPIO21 |
| Monitor brightness control | GPIO43 |
| SPI LED data | GPIO7 |
| SPI LED clock | GPIO6 |
| SPI LED latch | GPIO5 |

### 1.5 Button Inputs And SPI LED Drive

From the firmware side, the button path is split into two hardware blocks:

button inputs are read through the I2C GPIO expander and reported as button indices 0..15,

button LEDs are driven by a 16-bit SPI-backed shift-register state where ControlBoard now passes buttonId directly as ledIndex for non-power buttons.

This is the same logical mapping documented in docs/button-command-map.md, placed here so it can be read alongside the physical circuit diagram.

| Button Index | Button Name | Input Source | SPI LED Register Bit | SPI Register Value From Idle | SPI TX Bytes | Command ID | Command Name | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Power | MCP input index 0 | none | n/a | n/a | 0x0001 | CMD_SYS_POWER | timed action, command emitted on release, no SPI LED |
| 1 | Previous Track | MCP input index 1 | bit 1 | 0x0002 | 0x02 0x00 | 0x0101 | CMD_PREVIOUS_TRACK | UART command |
| 2 | Next Track | MCP input index 2 | bit 2 | 0x0004 | 0x04 0x00 | 0x0100 | CMD_NEXT_TRACK | UART command |
| 3 | Skip Forward | MCP input index 3 | bit 3 | 0x0008 | 0x08 0x00 | 0x0104 | CMD_SKIP_FORWARD | UART command |
| 4 | Skip Back | MCP input index 4 | bit 4 | 0x0010 | 0x10 0x00 | 0x0105 | CMD_SKIP_BACK | UART command |
| 5 | Play/Pause | MCP input index 5 | bit 5 | 0x0020 | 0x20 0x00 | 0x0102 | CMD_PLAY_PAUSE | UART command |
| 6 | Stop | MCP input index 6 | bit 6 | 0x0040 | 0x40 0x00 | 0x0103 | CMD_STOP_TRACK | UART command |
| 7 | Cover | MCP input index 7 | bit 7 | 0x0080 | 0x80 0x00 | 0x0117 or 0x0118 | CMD_COVER_VIEW_ON or CMD_COVER_VIEW_OFF | normalized to CMD_TOGGLE_COVER_VIEW on UART |
| 8 | Next Menu | MCP input index 8 | bit 8 | 0x0100 | 0x00 0x01 | 0x0107 | CMD_NEXT_MENU_ITEM | UART command |
| 9 | Menu Select | MCP input index 9 | bit 9 | 0x0200 | 0x00 0x02 | 0x0108 | CMD_ITEM_SELECT | UART command |
| 10 | Toggle DAC | MCP input index 10 | bit 10 | 0x0400 | 0x00 0x04 | 0x010A or 0x010F | CMD_TOGGLE_DAC_ON or CMD_TOGGLE_DAC_OFF | local relay control |
| 11 | Toggle Display | MCP input index 11 | bit 11 | 0x0800 | 0x00 0x08 | 0x010B or 0x010E | CMD_DISPLAY_OFF or CMD_DISPLAY_ON | normalized to CMD_TOGGLE_DISPLAY on UART |
| 12 | Toggle Meter | MCP input index 12 | bit 12 | 0x1000 | 0x00 0x10 | 0x010C or 0x010D | CMD_TOGGLE_METER_ON or CMD_TOGGLE_METER_OFF | normalized to CMD_TOGGLE_METER on UART |
| 13 | Rotary Left | MCP input index 13 | none in current rotary path | n/a | n/a | 0x0112 | CMD_ROTARY_ACTION | sent with param 0, no SPI LED write |
| 14 | Rotary Right | MCP input index 14 | none in current rotary path | n/a | n/a | 0x0112 | CMD_ROTARY_ACTION | sent with param 1, no SPI LED write |
| 15 | Cycle Brightness | MCP input index 15 | bit 15 | 0x8000 | 0x00 0x80 | 0x0116 | CMD_CYCLE_BRIGHTNESS | local brightness control |

Important hardware interpretation:

the SPI LED driver accepts a zero-based ledIndex (0–15) and has no knowledge of button IDs,

ControlBoard now maps non-power button IDs directly to LED indices: ledIndex = buttonId,

button 0 (power) is skipped and does not drive any SPI LED bit,

SPI LED bit 0 is therefore unused in the normal button path,

the SPI LED driver shifts the 16-bit LED register low byte first, then high byte,

the values above assume an idle LED register before the button is pressed,

if another LED is already latched on, the transmitted SPI value is the OR-combination of active LED bits,

rotary movement currently bypasses setLed() and therefore does not light a corresponding SPI LED bit.

### 1.6 Button Pin To LED Relationship

Looking at the schematic and the firmware together, the relationship is logical rather than a direct hardwired button-to-LED coupling inside the board logic:

each button input arrives on a numbered btn_in_x net through the debouncer and MCP23016/23018 input expander,

the ESP32 converts that numbered input into a firmware button index,

ControlBoard maps the button index directly to an LED index for non-power buttons (ledIndex = buttonId),

button 0 (power) is skipped — it has no corresponding LED,

the SPI shift register then drives the corresponding LED output.

So the normal pattern is:

btn_in_1 drives firmware button index 0 (power), which has no LED,

btn_in_2 drives firmware button index 1, which lights SPI LED bit 1,

btn_in_3 drives firmware button index 2, which lights SPI LED bit 2,

and so on.

The main exception is rotary movement: those inputs are still read as indices 13 and 14, but the current firmware does not call setLed() for rotary events, so no SPI LED bit is written for them.

| Schematic Input Net | Firmware Button Index | Firmware Name | Command Name | LED Relationship |
| --- | --- | --- | --- | --- |
| btn_in_1 | 0 | kPower | CMD_SYS_POWER | no LED (power button skipped) |
| btn_in_2 | 1 | kPrevTrack | CMD_PREVIOUS_TRACK | lights SPI bit 1, register 0x0002, LED output 1 |
| btn_in_3 | 2 | kNextTrack | CMD_NEXT_TRACK | lights SPI bit 2, register 0x0004, LED output 2 |
| btn_in_4 | 3 | kSkipForward | CMD_SKIP_FORWARD | lights SPI bit 3, register 0x0008, LED output 3 |
| btn_in_5 | 4 | kSkipBack | CMD_SKIP_BACK | lights SPI bit 4, register 0x0010, LED output 4 |
| btn_in_6 | 5 | kPlayPause | CMD_PLAY_PAUSE | lights SPI bit 5, register 0x0020, LED output 5 |
| btn_in_7 | 6 | kStop | CMD_STOP_TRACK | lights SPI bit 6, register 0x0040, LED output 6 |
| btn_in_8 | 7 | kCover | CMD_COVER_VIEW_ON or CMD_COVER_VIEW_OFF | lights SPI bit 7, register 0x0080, LED output 7 |
| btn_in_9 | 8 | kNextMenu | CMD_NEXT_MENU_ITEM | lights SPI bit 8, register 0x0100, LED output 8 |
| btn_in_10 | 9 | kMenuSelect | CMD_ITEM_SELECT | lights SPI bit 9, register 0x0200, LED output 9 |
| btn_in_11 | 10 | kToggleDac | CMD_TOGGLE_DAC_ON or CMD_TOGGLE_DAC_OFF | lights SPI bit 10, register 0x0400, LED output 10 |
| btn_in_12 | 11 | kToggleDisplay | CMD_DISPLAY_OFF or CMD_DISPLAY_ON | lights SPI bit 11, register 0x0800, LED output 11 |
| btn_in_13 | 12 | kToggleMeter | CMD_TOGGLE_METER_ON or CMD_TOGGLE_METER_OFF | lights SPI bit 12, register 0x1000, LED output 12 |
| btn_in_14 | 13 | kRotaryEventLeft | CMD_ROTARY_ACTION with param 0 | no LED update in current firmware |
| btn_in_15 | 14 | kRotaryEventRight | CMD_ROTARY_ACTION with param 1 | no LED update in current firmware |
| btn_in_16 | 15 | kCycleBrightness | CMD_CYCLE_BRIGHTNESS | lights SPI bit 15, register 0x8000, LED output 15 |

In short: button 0 (power) has no LED. For all other push buttons, input number N maps to LED bit N. Firmware stores the button as a zero-based index, skips index 0 for power, and now passes the remaining button indices directly to the SPI LED driver. Rotary left and right break that pattern because they are input-only events in the current code path.

### 1.7 40-Pin Front-Panel Molex Connector

The attached 40-pin front-panel connector drawing can be matched to the current firmware naming as follows.

Power and status lines:

| Connector Pin | Diagram Label | Current Firmware Symbol | Notes |
| --- | --- | --- | --- |
| 1 | Power On (LED) | app active / power-state LED path | separate from the 16 SPI button LEDs |
| 2 | Power StandBy (LED) | app standby / power-state LED path | separate from the 16 SPI button LEDs |
| 35 | GND | ground | return |
| 36 | GND | ground | return |
| 37 | GND | ground | return |
| 38 | GND | ground | return |
| 39 | 5V | 5V rail | supply |
| 40 | 5V | 5V rail | supply |

Button and command inputs:

| Connector Pin | Diagram Label | Firmware Button Index | Current Firmware Mapping | Match Status |
| --- | --- | --- | --- | --- |
| 3 | Btn_Power | 0 | kPower -> CMD_SYS_POWER | matches |
| 5 | CMD_Previous_Track | 1 | kPrevTrack -> CMD_PREVIOUS_TRACK | matches |
| 7 | CMD_Next_Track | 2 | kNextTrack -> CMD_NEXT_TRACK | matches |
| 9 | CMD_Skip_Forrard | 3 | kSkipForward -> CMD_SKIP_FORWARD | matches |
| 11 | CMD_Skip_Back | 4 | kSkipBack -> CMD_SKIP_BACK | matches |
| 13 | CMD_Play_Pause | 5 | kPlayPause -> CMD_PLAY_PAUSE | matches |
| 15 | CMD_Stop_Track | 6 | kStop -> CMD_STOP_TRACK | matches |
| 17 | CMD_Prev_Menu_Item | 7 | current firmware uses kCover -> CMD_COVER_VIEW_ON/OFF | mismatch |
| 4 | CMD_Next_Menu_Item | 8 | kNextMenu -> CMD_NEXT_MENU_ITEM | matches |
| 6 | CMD_Item_Select | 9 | kMenuSelect -> CMD_ITEM_SELECT | matches |
| 8 | CMD_Toggle_Dac | 10 | kToggleDac -> CMD_TOGGLE_DAC_ON/OFF | matches |
| 10 | CMD_Display_Off | 11 | kToggleDisplay -> CMD_DISPLAY_OFF/ON | partial match: connector names only one half of the toggle |
| 12 | CMD_Toggle_Meter_On | 12 | kToggleMeter -> CMD_TOGGLE_METER_ON/OFF | partial match: connector names only one half of the toggle |
| 14 | CMD_Rotary_Action | 13 | kRotaryEventLeft -> CMD_ROTARY_ACTION with param 0 | matches as rotary-left |
| 16 | CMD_Rotary_Action | 14 | kRotaryEventRight -> CMD_ROTARY_ACTION with param 1 | matches as rotary-right |
| 18 | CMD_Cycle_Brightness | 15 | kCycleBrightness -> CMD_CYCLE_BRIGHTNESS | matches |

SPI button LED outputs on the same connector:

| Connector Pin | Diagram Label | SPI LED Output | Current Firmware Use |
| --- | --- | --- | --- |
| 19 | LED_0_1 | bit 0 | lights for Previous Track |
| 21 | LED_0_2 | bit 1 | lights for Next Track |
| 23 | LED_0_3 | bit 2 | lights for Skip Forward |
| 25 | LED_0_4 | bit 3 | lights for Skip Back |
| 27 | LED_0_5 | bit 4 | lights for Play/Pause |
| 29 | LED_0_6 | bit 5 | lights for Stop |
| 31 | LED_0_7 | bit 6 | lights for firmware button index 7 |
| 33 | LED_0_8 | bit 7 | lights for Next Menu |
| 20 | LED_0_9 | bit 8 | lights for Menu Select |
| 22 | LED_0_10 | bit 9 | lights for Toggle DAC |
| 24 | LED_0_11 | bit 10 | lights for Toggle Display |
| 26 | LED_0_12 | bit 11 | lights for Toggle Meter |
| 28 | LED_0_13 | bit 12 | present on connector, not driven by current rotary path |
| 30 | LED_0_14 | bit 13 | present on connector, not driven by current rotary path |
| 32 | LED_0_15 | bit 14 | lights for Cycle Brightness |
| 34 | LED_0_16 | bit 15 | present on connector, currently unused by firmware |

Important connector-level mismatch:

the connector drawing labels pin 17 as CMD_Prev_Menu_Item,

current firmware assigns that logical slot to Cover instead,

CMD_PREV_MENU_ITEM exists in the codebase, but createButtonActionMap() does not currently bind it to any front-panel button.

If the connector drawing is the intended hardware truth, button index 7 would need to be reassigned from CoverViewInstance to PreviousMenuInstance in ControlBoard::createButtonActionMap().

## 2. Raspberry Pi Side Wiring

### 2.1 Confirmed Script-Level Pin Usage

| Function | Pi BCM | Pi Physical Pin | Notes |
| --- | --- | --- | --- |
| ESP32 -> Pi data-ready input | 23 | 16 | listener waits on rising edge |
| Pi -> ESP32 data-ready output | 24 | 18 | sender script pulses this line |

### 2.2 UART5 Header Mapping

Using the standard dtoverlay=uart5 mapping on a 40-pin Raspberry Pi header:

| Function | Pi BCM | Pi Physical Pin |
| --- | --- | --- |
| UART5 TX | 12 | 32 |
| UART5 RX | 13 | 33 |

## 3. End-To-End Connection Table

| Function | Raspberry Pi Side | ESP32-S3 Side |
| --- | --- | --- |
| Pi UART5 TX -> ESP32 UART RX | BCM12, pin 32 | GPIO1 |
| Pi UART5 RX <- ESP32 UART TX | BCM13, pin 33 | GPIO2 |
| ESP32 -> Pi data-ready | BCM23, pin 16 | GPIO41 |
| Pi -> ESP32 data-ready | BCM24, pin 18 | GPIO42 |
| Ground | Pi GND | ESP32 GND |

Important rule:

UART TX always connects to the other side's RX.

## 4. Handshake Meaning

The design uses two extra GPIO lines in addition to UART.

### 4.1 ESP32 -> Pi Notification

ESP32 writes a UART packet.

ESP32 raises its data-ready output briefly.

Pi listener wakes on BCM23 and reads from /dev/ttyAMA5.

### 4.2 Pi -> ESP32 Notification

Pi writes a UART packet.

Pi pulses BCM24.

ESP32 sees GPIO42 rising edge and services UART RX.

This means the UART bytes and the ready notification are separate signals.

## 5. Pi Numbering Reminder

The Python scripts use:

GPIO.setmode(GPIO.BCM)

That means values like 23 and 24 are BCM GPIO numbers, not physical header pin numbers.

## 6. Related Docs

docs/button-command-map.md

docs/project-guide.md

docs/protocol-reference.md

docs/raspberry-pi-setup.md

Source: docs/protocol-reference.md

# UART Protocol Reference

This document summarizes the current UART protocol shared between the ESP32 firmware and the Raspberry Pi scripts.

Primary source:

lib/protocol/uartProtocol.hpp

Related implementations:

lib/transport/uart/serial.cpp

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

## 1. Packet Format

The current packet size is 18 bytes.

Layout:

| Byte Offset | Field | Size |
| --- | --- | --- |
| 0 | start byte | 1 |
| 1 | protocol version | 1 |
| 2 | source app | 1 |
| 3 | message type | 1 |
| 4 | sequence | 1 |
| 5-6 | command ID | 2 |
| 7-16 | five 16-bit parameters | 10 |
| 17 | checksum | 1 |

Current constants:

start byte: 0xAA

protocol version: 0x01

packet size: 18

## 2. Message Structure

The in-memory firmware struct is UartMessage.

It contains:

startByte

version

srcApp

msgType

sequence

commandId

params[5]

checksum

Reference:

lib/protocol/uartProtocol.hpp

## 3. Message Types

Current message types are:

| Name | Value |
| --- | --- |
| MSG_COMMAND | 0x01 |
| MSG_STATUS | 0x02 |
| MSG_ACK | 0x03 |
| MSG_NACK | 0x04 |

## 4. Application IDs

Current application IDs are:

| Name | Value |
| --- | --- |
| APP_ESP32 | 0x01 |
| APP_PI | 0x02 |

## 5. Command IDs

Current command catalog from lib/protocol/uartProtocol.hpp:

| Command | Value |
| --- | --- |
| CMD_NO_ACTION | 0x0000 |
| CMD_SYS_POWER | 0x0001 |
| CMD_SYS_RPI_SHUTDOWN | 0x0002 |
| CMD_SYS_HEARTBEAT | 0x0003 |
| CMD_SYS_NOHEARTBEAT | 0x0004 |
| CMD_NEXT_TRACK | 0x0100 |
| CMD_PREVIOUS_TRACK | 0x0101 |
| CMD_PLAY_PAUSE | 0x0102 |
| CMD_STOP_TRACK | 0x0103 |
| CMD_SKIP_FORWARD | 0x0104 |
| CMD_SKIP_BACK | 0x0105 |
| CMD_PREV_MENU_ITEM | 0x0106 |
| CMD_NEXT_MENU_ITEM | 0x0107 |
| CMD_ITEM_SELECT | 0x0108 |
| CMD_EXIT_ITEM | 0x0109 |
| CMD_TOGGLE_DAC_ON | 0x010A |
| CMD_DISPLAY_OFF | 0x010B |
| CMD_TOGGLE_METER_ON | 0x010C |
| CMD_TOGGLE_METER_OFF | 0x010D |
| CMD_DISPLAY_ON | 0x010E |
| CMD_TOGGLE_DAC_OFF | 0x010F |
| CMD_ROTARY_LEFT | 0x0110 |
| CMD_ROTARY_RIGHT | 0x0111 |
| CMD_ROTARY_ACTION | 0x0112 |
| CMD_TOGGLE_DAC | 0x0113 |
| CMD_TOGGLE_DISPLAY | 0x0114 |
| CMD_TOGGLE_METER | 0x0115 |
| CMD_CYCLE_BRIGHTNESS | 0x0116 |
| CMD_COVER_VIEW_ON | 0x0117 |
| CMD_COVER_VIEW_OFF | 0x0118 |
| CMD_TOGGLE_COVER_VIEW | 0x0119 |

## 6. Checksum

The Pi listener script computes checksum as:

sum of bytes 1 through 16, masked to 8 bits.

Current Python expression:

sum(packet_bytes[1:17]) & 0xFF

Reference:

scripts/rpi/home/antho/UAart5Listener.py

## 7. Current Pi Script Expectations

The Pi listener currently expects messages such as:

shutdown,

next/previous track,

play/pause,

stop,

skip forward/back,

rotary action,

toggle meter,

toggle cover view.

The Pi-side script interprets some commands directly and some as command-plus-parameter combinations.

Examples:

CMD_ROTARY_ACTION uses parameter 0 for direction,

CMD_TOGGLE_METER uses parameter 0 for enabled or disabled,

CMD_TOGGLE_COVER_VIEW uses parameter 0 for enabled or disabled,

CMD_TOGGLE_DISPLAY is generated by firmware with parameter 0 or 1.

## 8. Heartbeat

Heartbeat command ID is:

CMD_SYS_HEARTBEAT = 0x0003

Current firmware compatibility note:

the firmware also still accepts legacy heartbeat command 0x9999.

Reference:

lib/app/ControlBoard.cpp

## 9. Current Practical Limitations

The protocol reference is centralized, but some older macro aliases still exist in the header.

The firmware actively reacts to heartbeat messages, but other incoming Pi-originated messages are not yet deeply handled in ControlBoard.

The Pi scripts and firmware should be kept aligned on command IDs and parameter semantics.

## 10. Related Docs

docs/project-guide.md

docs/button-command-map.md

docs/wiring-reference.md

Source: docs/power-sequencing.md

# Power Sequencing

This document explains the current power-state behavior implemented in the firmware.

Source files:

lib/app/actionProcessor.cpp

lib/power/RPIBootManager.cpp

lib/power/RelayController.cpp

lib/power/powerState.hpp

## 1. Current Power States

The current power state enum is:

OFF

SHUTTING_DOWN

ON

TURNING_ON

SLEEP

GOING_TO_SLEEP

DEEPSLEEP

GOING_INTO_DEEP_SLEEP

In practice, the code paths currently used most clearly are:

TURNING_ON

ON

SLEEP

GOING_TO_SLEEP

DEEPSLEEP

## 2. Timing Constants Used Today

Current power-related timing constants in lib/app/actionProcessor.cpp:

| Constant | Value | Meaning |
| --- | --- | --- |
| POWER_SETTLE_DELAY_MS | 1500 | delay used after enabling some relays |
| SCREEN_ON_DELAY_MS | 1000 | delay used during power-on for screen and Pi relay steps |
| LONG_PRESS_THRESHOLD_MS | 3000 | separates sleep from deep sleep |
| RPI_BOOT_TIMEOUT_MS | 60000 | max time to wait for heartbeat after power-on |
| RPI_SHUTDOWN_TIMEOUT_MS | 60000 | max time to wait for heartbeat timeout during shutdown |

## 3. How The Power Button Works

The power button uses a timed action.

Behavior:

button press stores the current time,

button release computes the hold duration,

release emits CMD_SYS_POWER plus releaseTimeMillis,

ActionProcessor chooses the power transition based on current power state and held duration.

Reference:

lib/input/actions/actionTemplates.hpp

## 4. Power-On Sequence

If the current state is OFF, SLEEP, or DEEPSLEEP, a power-button release enters the power-on path.

Current sequence:

power LED state is set to TURNING_ON,

screen relay is enabled with a 1000 ms delay,

DAC relay is enabled with a 1500 ms delay,

output stage relay is enabled with a 1500 ms delay,

Raspberry Pi relay is enabled with a 1000 ms delay,

firmware waits up to 60 seconds for Pi heartbeat,

power LED moves to ON,

activity status becomes Active.

Important note:

the power LED is moved to ON even if the wait for heartbeat times out, because the current code sets ON immediately after waitForRpiToBoot() returns.

## 5. Sleep Sequence

If the current state is ON and the power button hold time is less than 3000 ms, the firmware enters the sleep path.

Current sequence:

power LED state becomes GOING_TO_SLEEP,

CMD_SYS_RPI_SHUTDOWN is sent over UART,

firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout,

Raspberry Pi power relay is turned off,

firmware delays 500 ms,

screen power relay is turned off,

firmware delays 5000 ms,

power LED state becomes SLEEP,

activity status becomes sleeping.

Practical result:

the Raspberry Pi and screen are shut down,

DAC and output stage power remain on,

the board stays in a lower-power standby style state rather than a full deep power-down.

## 6. Deep-Sleep Sequence

If the current state is ON and the power button hold time is 3000 ms or more, the firmware enters the deep-sleep path.

Current sequence:

power LED state becomes GOING_TO_SLEEP,

CMD_SYS_RPI_SHUTDOWN is sent over UART,

firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout,

Raspberry Pi power relay is turned off,

firmware delays 500 ms,

screen power relay is turned off,

DAC relay is turned off,

output stage relay is turned off,

power LED state becomes DEEPSLEEP,

activity status becomes sleeping.

Practical result:

Raspberry Pi power is removed,

screen power is removed,

DAC power is removed,

output stage power is removed,

board logic still remains present enough to respond later, so this is a project-specific deep sleep state, not necessarily ESP-IDF chip deep sleep.

## 7. Heartbeat Role In Power Sequencing

Heartbeat is the synchronization signal between the ESP32 and the Raspberry Pi.

Current behavior:

heartbeat received sets the event-group bit used for boot completion,

heartbeat timeout sets the event-group bit used for shutdown confirmation,

power-on waits for heartbeat reception,

sleep and deep-sleep wait for heartbeat timeout.

This means the system uses absence of heartbeat as a shutdown/offline signal.

## 8. Relay Ownership In Power Paths

Current relay assignments from lib/board/boardConfig.hpp:

| Relay Purpose | ESP32 GPIO |
| --- | --- |
| Screen power | 13 |
| DAC power | 12 |
| Raspberry Pi power | 11 |
| Output stage power | 9 |
| Prototype DAC enable | 10 |
| General 1 | 47 |
| General 2 | 39 |

Current dedicated relay helper methods:

shutdownRpi() turns off the Raspberry Pi relay,

shutdownScreen() turns off the screen relay,

handleToggleDac() toggles the DAC relay.

## 9. Current Behavior Caveats

wait parameters in relay shutdown helpers are currently unused.

The code path sets GOING_TO_SLEEP for both sleep and deep-sleep paths.

The enum includes states like SHUTTING_DOWN and GOING_INTO_DEEP_SLEEP, but the current processor code does not clearly transition through them.

waitForRpiShutdown() is satisfied by heartbeat timeout, which is a practical signal but not a strong explicit shutdown acknowledgment packet.

## 10. Recommended Future Documentation Additions

Useful follow-ups for this file later:

add a timing diagram for ON -> SLEEP and ON -> DEEPSLEEP,

document expected relay electrical loads,

document which subsystems remain powered in each state,

document recovery behavior if heartbeat never appears during boot.

## 11. Related Docs

docs/project-guide.md

docs/architecture.md

docs/wiring-reference.md

Source: docs/raspberry-pi-setup.md

# Raspberry Pi Setup Reference

This document condenses the current Raspberry Pi side setup and companion scripts used by the project.

Primary source:

scripts/rpi/README.md

Companion scripts:

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

## 1. What Runs On The Pi

The Raspberry Pi side currently has two main Python processes:

a UART5 listener that receives commands from the ESP32 and runs local commands,

a heartbeat sender that periodically transmits a heartbeat packet back to the ESP32.

## 2. UART Listener Role

The listener script:

opens /dev/ttyAMA5,

waits on BCM GPIO23 for a rising edge,

reads one 18-byte packet,

verifies the checksum,

dispatches the command to local handlers.

Current handled actions include:

Raspberry Pi shutdown,

playback next/previous,

play/pause,

stop,

seek forward/back,

rotary next/previous,

meter toggle,

cover view toggle.

Reference:

scripts/rpi/home/antho/UAart5Listener.py

## 3. Heartbeat Sender Role

The heartbeat script:

opens /dev/ttyAMA5,

waits until the ESP32 data-ready line is low,

sends a heartbeat packet every 10 seconds,

pulses BCM GPIO24 to notify the ESP32 that data is waiting.

Reference:

scripts/rpi/home/antho/heartbeat_sender.py

## 4. Required Pi Configuration

The Pi setup docs currently require:

dtoverlay=uart5 in /boot/firmware/config.txt,

Python serial support,

pigpiod,

systemd services for listener and heartbeat sender.

The config file included in the repo also shows dtoverlay=uart5.

Reference:

scripts/rpi/boot/firmware/config.txt

## 5. Service Summary

Current intended services:

| Service | Role |
| --- | --- |
| uart_listener.service | receives ESP32 commands |
| heartbeat.service | periodically sends heartbeat to ESP32 |
| pigpiod | GPIO support used by the Pi-side setup |

From the current setup notes, the service start commands are based on:

/usr/bin/python3 /home/antho/uart5_listener.py

/usr/bin/python3 /home/antho/heartbeat_sender.py

## 6. Important Pi File Locations

Current documented paths:

| Purpose | Path |
| --- | --- |
| UART5 overlay config | /boot/firmware/config.txt |
| listener script | /home/antho/uart5_listener.py |
| listener service | /etc/systemd/system/uart_listener.service |
| heartbeat service | /etc/systemd/system/heartbeat.service |
| pigpio daemon | /usr/local/bin/pigpiod |

## 7. Wiring Expectations On The Pi

Current script-level GPIO usage:

| Purpose | Pi BCM | Physical Pin |
| --- | --- | --- |
| listener data-ready input | 23 | 16 |
| heartbeat data-ready output | 24 | 18 |

Typical UART5 mapping with dtoverlay=uart5:

| Purpose | Pi BCM | Physical Pin |
| --- | --- | --- |
| UART5 TX | 12 | 32 |
| UART5 RX | 13 | 33 |

## 8. Operational Summary

The intended behavior is:

ESP32 sends a command to the Pi and pulses its ready line.

Pi listener wakes and executes the requested action.

Pi heartbeat sender continues to send heartbeat packets.

ESP32 uses those packets to decide whether the Pi is online, booted, or shut down.

## 9. Troubleshooting Notes

Current troubleshooting guidance in the repo includes:

verify dtoverlay=uart5,

confirm /dev/ttyAMA5 exists,

make sure no conflicting serial console is enabled,

inspect systemctl status for the services,

watch logs with journalctl -u <service> -f.

## 10. Related Docs

docs/project-guide.md

docs/wiring-reference.md

docs/protocol-reference.md

Source: scripts/rpi/README.md

# UART5 Listener Setup on Moode (Raspberry Pi)

This guide explains how to enable UART5 on a Raspberry Pi running Moode, install required dependencies, build and install pigpio, and configure systemd services to automatically run:

A UART5 listener

A UART heartbeat sender

It also covers auto-login configuration and troubleshooting.

## Repo Layout

This repo keeps Raspberry Pi companion assets under scripts/rpi/.

Important tracked files:

scripts/rpi/home/antho/UAart5Listener.py: Pi-side UART listener

scripts/rpi/home/antho/heartbeat_sender.py: Pi-side heartbeat sender

scripts/rpi/boot/firmware/config.txt: example firmware config with dtoverlay=uart5

scripts/rpi/boot/firmware/config-user.txt: example user overlay additions

scripts/rpi/boot/firmware/cmdline.txt: example boot command line

Deployment note:

These files are source-controlled examples, not their final runtime locations on the Pi.

Copy the scripts and config fragments from this folder into the deployment paths referenced below when setting up a Pi.

Current listener behavior note:

On startup, the UART listener forces the display to the firmware default meter-off state (webui) before processing incoming commands.

## 1. Install Moode Using Raspberry Pi Imager

Use the official Raspberry Pi Imager to flash Moode onto your SD card.

Once Moode is running, update the system:

sudo apt update
sudo apt upgrade -y

## 2. Install Required Packages

sudo apt install -y ddcutil

## 3. Enable UART5

Edit the Raspberry Pi firmware configuration:

sudo nano /boot/firmware/config.txt

Add or confirm the following entry:

dtoverlay=uart5

Reboot to apply:

sudo reboot

## 4. Install Python Serial Support

### 4.1 Preferred Installation (pip)

sudo python3 -m pip install pyserial

Verify installation:

python3 -c "import serial; print(serial.__version__)"

### 4.2 If pip Is Missing

sudo apt install -y python3-pip
sudo python3 -m pip install pyserial

### 4.3 If You Encounter externally-managed-environment

Install the distro-managed version instead:

sudo apt install -y python3-serial

## 5. Install pigpio

### 5.1 Install Build Tools

sudo apt update
sudo apt install -y git make gcc

### 5.2 Download and Build pigpio

cd /tmp
git clone https://github.com/joan2937/pigpio.git
cd pigpio
make
sudo make install

This installs:

/usr/local/bin/pigpiod

Python module pigpio

Command-line tools (pigs, etc.)

### 5.3 Enable and Start pigpiod

sudo systemctl enable pigpiod
sudo systemctl start pigpiod

Verify installation:

which pigpiod

Expected output:

/usr/local/bin/pigpiod

### 5.4 Create pigpiod systemd Service (Manual)

sudo nano /etc/systemd/system/pigpiod.service

[Unit]
Description=Pigpio daemon
After=network.target

[Service]
Type=forking
ExecStart=/usr/local/bin/pigpiod -l
ExecStop=/bin/kill -s TERM $MAINPID
Restart=always

[Install]
WantedBy=multi-user.target

Reload and start:

sudo systemctl daemon-reexec
sudo systemctl daemon-reload
sudo systemctl enable pigpiod
sudo systemctl start pigpiod

Verify it is running (number increments each run):

pigs t

## 6. Create the UART Listener systemd Service

sudo nano /etc/systemd/system/uart_listener.service

[Unit]
Description=UART5 Listener for ESP32
After=network.target

[Service]
Type=simple
ExecStart=/usr/bin/python3 /home/antho/uart5_listener.py
Restart=always
User=antho
Group=antho
WorkingDirectory=/home/antho
StandardOutput=journal
StandardError=journal

[Install]
WantedBy=multi-user.target

## 7. Create the UART Listener Script

nano /home/antho/uart5_listener.py

Paste the contents of your UART listener script.

Set permissions:

chmod +x /home/antho/uart5_listener.py
sudo chown antho:antho /home/antho/uart5_listener.py

## 8. Enable and Start the UART Listener Service

sudo systemctl daemon-reload
sudo systemctl enable uart_listener.service
sudo systemctl start uart_listener.service
sudo systemctl status uart_listener.service

View logs:

journalctl -u uart_listener.service -f

## 9. Create the Heartbeat Sender Service

### 9.1 Create the Script

sudo nano /usr/local/bin/heartbeat_sender.py
sudo chmod +x /usr/local/bin/heartbeat_sender.py

Paste the contents of heartbeat_sender.py and save.

### 9.2 Create the systemd Service

sudo nano /etc/systemd/system/heartbeat.service

[Unit]
Description=UART5 Heartbeat Sender
After=network.target multi-user.target

[Service]
Type=simple
ExecStart=/usr/bin/python3 /home/antho/heartbeat_sender.py
Restart=always
WorkingDirectory=/home/antho

[Install]
WantedBy=multi-user.target

### 9.3 Enable and Start the Service

sudo systemctl daemon-reload
sudo systemctl enable heartbeat.service
sudo systemctl restart heartbeat.service

Check logs:

journalctl -u heartbeat.service -f

## 10. File Locations Summary

| Purpose | File Path | Notes |
| --- | --- | --- |
| UART5 overlay configuration | /boot/firmware/config.txt | Must contain dtoverlay=uart5 |
| UART listener script | /home/antho/uart5_listener.py | Must be executable |
| UART listener service | /etc/systemd/system/uart_listener.service | Auto-start on boot |
| Heartbeat service | /etc/systemd/system/heartbeat.service | Periodic UART TX |
| pigpiod binary | /usr/local/bin/pigpiod | Built from source |
| Logs | journalctl | journalctl -u <service> -f |

## Set auto logging

Create and open a new file

sudo mkdir -p /etc/systemd/system/getty@tty1.service.d
sudo nano /etc/systemd/system/getty@tty1.service.d/autologin.conf

Add the following code

[Service]
ExecStart=
ExecStart=-/sbin/agetty --autologin pi --noclear %I $TERM

save the file this will enable auto login

## Hide the login Text

sudo nano /boot/firmware/cmdline.txt

Add the following to the end of the line:

quiet loglevel=0 vt.global_cursor_default=0

## Add the splash Screen

copy the splashscreen to the RPI

move into the folder where the splash screen is to be stored

make sure that the f copy the file from the location where you original stored the file

```bash

cd /opt
sudo mv /home/antho/filename.png  splash.png
sudo chmod +x splash.png

## add meters

copy you meter configuration into opt/1024x600



## 11. Enable Auto-Login on Local Console

```bash
sudo raspi-config

Navigate to:

System Options → Boot / Auto Login

Choose one:

B2 Console Autologin

B4 Desktop Autologin

Reboot when prompted.

## 12. Troubleshooting

### 12.1 UART5 Not Working

Check overlay:

grep uart5 /boot/firmware/config.txt

Check devices:

ls -l /dev/ttyAMA* /dev/ttyS* 2>/dev/null

Ensure no serial console is enabled:

sudo nano /boot/firmware/cmdline.txt

Remove any console= entry referencing UART.

### 12.2 Service Not Starting

sudo systemctl status uart_listener.service

Reload systemd if needed:

sudo systemctl daemon-reload

### 12.3 Permission Denied Accessing UART

sudo usermod -a -G dialout antho
sudo reboot

### 12.4 Python serial Module Not Found

sudo apt install python3-serial

or

sudo python3 -m pip install pyserial

Source: docs/button-command-map.md

# Button And Command Map

This document maps the current firmware button indices to action objects and the UART, SPI, or local effects they produce.

The source of truth for the current mapping is:

lib/board/boardConfig.hpp

lib/app/ControlBoard.cpp

lib/input/actions/buttonActions.hpp

lib/input/actions/buttonActions.cpp

lib/input/actions/actionTemplates.hpp

lib/app/actionProcessor.cpp

lib/indicators/spiLedDriver.cpp

## 1. How To Read This Map

There are four main action patterns in the current code:

SimpleCommandAction: sends one command when the button is pressed.

ToggleAction: flips internal state on press and emits an ON or OFF command.

TimedAction: starts timing on press and emits one command on release with the held duration.

RotaryAction: emits CMD_ROTARY_ACTION with parameter 0 for left and 1 for right.

Important behavior detail:

toggle actions change state on button press only,

timed actions emit their command on button release,

simple command actions do nothing on release.

SPI LED behavior detail:

the SPI LED driver accepts a zero-based ledIndex (0–15) and knows nothing about button IDs,

the caller (ControlBoard) now passes the button ID directly as the LED index for non-power buttons: ledIndex = buttonId,

button 0 (power) has no corresponding LED and is skipped — setLed() is not called for it,

because button 0 is skipped, SPI LED bit 0 is currently unused in the normal button path,

for all other buttons, the driver sets bit 1 << ledIndex in the 16-bit SPI LED register,

the driver shifts out that full 16-bit register low byte first,

the value shown in the table below is the per-button bit value contributed by that press, not a guarantee that the full transmitted register will contain only that bit,

release normally clears the bit again, but toggle actions can leave it latched when keepLedActive = true,

rotary movement currently does not call setLed() in handleRotaryMovement(), so no SPI LED update is sent for rotary left or rotary right events.

## 2. Button Index Reference

Current button indices from lib/board/boardConfig.hpp:

| Index | Symbolic Name |
| --- | --- |
| 0 | kPower |
| 1 | kPrevTrack |
| 2 | kNextTrack |
| 3 | kSkipForward |
| 4 | kSkipBack |
| 5 | kPlayPause |
| 6 | kStop |
| 7 | kCover |
| 8 | kNextMenu |
| 9 | kMenuSelect |
| 10 | kToggleDac |
| 11 | kToggleDisplay |
| 12 | kToggleMeter |
| 13 | kRotaryEventLeft |
| 14 | kRotaryEventRight |
| 15 | kCycleBrightness |

## 3. Current Mapping Table

The table below separates three things that were previously blended together:

the physical button index,

the SPI LED bit pattern written when that button press lights the corresponding LED,

the command ID and command name produced by the button action.

For toggle-backed buttons, the action can emit one of two raw command IDs depending on current state, and the action processor may then normalize that into a different UART command.

| Index | Button Name | Action Type | SPI Register | SPI Bit Pattern | SPI TX Bytes (low, high) | Raw Command ID(s) | Raw Command Name(s) | Final Routed Command | Final Effect |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Power | TimedAction<CMD_SYS_POWER> | n/a | n/a | n/a | 0x0001 | CMD_SYS_POWER | 0x0001 CMD_SYS_POWER | enters ON, SLEEP, or DEEPSLEEP path depending on current state and hold time; no SPI LED |
| 1 | Previous Track | simple command | 0x0002 | 0000 0000 0000 0010 | 0x02 0x00 | 0x0101 | CMD_PREVIOUS_TRACK | 0x0101 CMD_PREVIOUS_TRACK | UART command to Pi |
| 2 | Next Track | simple command | 0x0004 | 0000 0000 0000 0100 | 0x04 0x00 | 0x0100 | CMD_NEXT_TRACK | 0x0100 CMD_NEXT_TRACK | UART command to Pi |
| 3 | Skip Forward | simple command | 0x0008 | 0000 0000 0000 1000 | 0x08 0x00 | 0x0104 | CMD_SKIP_FORWARD | 0x0104 CMD_SKIP_FORWARD | UART command to Pi |
| 4 | Skip Back | simple command | 0x0010 | 0000 0000 0001 0000 | 0x10 0x00 | 0x0105 | CMD_SKIP_BACK | 0x0105 CMD_SKIP_BACK | UART command to Pi |
| 5 | Play/Pause | simple command | 0x0020 | 0000 0000 0010 0000 | 0x20 0x00 | 0x0102 | CMD_PLAY_PAUSE | 0x0102 CMD_PLAY_PAUSE | UART command to Pi |
| 6 | Stop | simple command | 0x0040 | 0000 0000 0100 0000 | 0x40 0x00 | 0x0103 | CMD_STOP_TRACK | 0x0103 CMD_STOP_TRACK | UART command to Pi |
| 7 | Cover | ToggleAction<CMD_COVER_VIEW_ON, CMD_COVER_VIEW_OFF> | 0x0080 | 0000 0000 1000 0000 | 0x80 0x00 | 0x0117 / 0x0118 | CMD_COVER_VIEW_ON / CMD_COVER_VIEW_OFF | 0x0119 CMD_TOGGLE_COVER_VIEW with param 1 or 0 | Pi toggles cover view |
| 8 | Next Menu | simple command | 0x0100 | 0000 0001 0000 0000 | 0x00 0x01 | 0x0107 | CMD_NEXT_MENU_ITEM | 0x0107 CMD_NEXT_MENU_ITEM | UART command to Pi |
| 9 | Menu Select | simple command | 0x0200 | 0000 0010 0000 0000 | 0x00 0x02 | 0x0108 | CMD_ITEM_SELECT | 0x0108 CMD_ITEM_SELECT | UART command to Pi |
| 10 | Toggle DAC | ToggleAction<CMD_TOGGLE_DAC_ON, CMD_TOGGLE_DAC_OFF> | 0x0400 | 0000 0100 0000 0000 | 0x00 0x04 | 0x010A / 0x010F | CMD_TOGGLE_DAC_ON / CMD_TOGGLE_DAC_OFF | local-only relay toggle, no normalized UART command | toggles DAC power relay |
| 11 | Toggle Display | ToggleAction<CMD_DISPLAY_OFF, CMD_DISPLAY_ON> | 0x0800 | 0000 1000 0000 0000 | 0x00 0x08 | 0x010B / 0x010E | CMD_DISPLAY_OFF / CMD_DISPLAY_ON | local monitor PWM blank/unblank | blanks or restores the display backlight |
| 12 | Toggle Meter | ToggleAction<CMD_TOGGLE_METER_ON, CMD_TOGGLE_METER_OFF> | 0x1000 | 0001 0000 0000 0000 | 0x00 0x10 | 0x010C / 0x010D | CMD_TOGGLE_METER_ON / CMD_TOGGLE_METER_OFF | 0x0115 CMD_TOGGLE_METER with param 1 or 0 | Pi meter display change |
| 13 | Rotary Left | RotaryAction<CMD_ROTARY_ACTION> | n/a | n/a | n/a | 0x0112 | CMD_ROTARY_ACTION | 0x0112 CMD_ROTARY_ACTION with param 0 | Pi interprets as previous/left |
| 14 | Rotary Right | RotaryAction<CMD_ROTARY_ACTION> | n/a | n/a | n/a | 0x0112 | CMD_ROTARY_ACTION | 0x0112 CMD_ROTARY_ACTION with param 1 | Pi interprets as next/right |
| 15 | Cycle Brightness | simple command | 0x8000 | 1000 0000 0000 0000 | 0x00 0x80 | 0x0116 | CMD_CYCLE_BRIGHTNESS | 0x0116 CMD_CYCLE_BRIGHTNESS | local brightness cycle |

## 4. Important Per-Button Notes

### 4.1 Power Button

The power button is special.

press starts a timer,

release sends CMD_SYS_POWER,

releaseTimeMillis decides which power transition runs.

Current threshold:

short press is less than 3000 ms,

long press is 3000 ms or more.

References:

lib/input/actions/actionTemplates.hpp

lib/app/actionProcessor.cpp

### 4.2 Toggle Buttons

Toggle actions keep internal software state in the action object.

That means:

the first press uses the action's default state,

later presses alternate between ON and OFF outputs,

LED retention behavior comes from keepLedActive in the response.

Current toggle-backed buttons:

Cover

Toggle DAC

Toggle Display

Toggle Meter

### 4.3 Display Toggle Specifics

The display toggle is currently defined as:

ON command type: CMD_DISPLAY_OFF

OFF command type: CMD_DISPLAY_ON

Because the toggle state starts as false, the first press emits CMD_DISPLAY_OFF.

That is now handled locally on the ESP32 by setting the monitor PWM duty to 0 for blank and restoring the saved brightness level for unblank.

### 4.4 Rotary Events

Both rotary directions use the same action object class and the same command ID.

Direction is carried in parameter 0:

0 = left

1 = right

Unlike the physical push buttons, rotary movement is handled by handleRotaryMovement() and does not currently light a button LED through the SPI shift register.

### 4.5 SPI Payload Format For Button LEDs

When a physical button press reaches handleButtonPressed(), the firmware uses the button ID directly as the LED index and calls indicators::getSpiLedDriver().setLed(buttonId, true).

Button 0 (power) is skipped — it has no corresponding LED. For all other buttons, the LED index is buttonId, which produces a 16-bit LED register where:

bit 0 is currently unused in the normal button path,

bit 1 = button index 1 (Previous Track),

bit 2 = button index 2 (Next Track),

...

bit 15 = button index 15 (Cycle Brightness).

The SPI driver (SpiLedDriver) accepts a raw ledIndex (0–15) and sets bit 1 << ledIndex. It has no knowledge of button IDs — the mapping is entirely in ControlBoard.

The SPI driver then transmits:

byte 0 = low 8 bits of the register,

byte 1 = high 8 bits of the register.

Examples from an idle LED state:

pressing Previous Track (button 1) sets LED index 1, register value 0x0002, transmitted as 0x02 0x00,

pressing Cover (button 7) sets LED index 7, register value 0x0080, transmitted as 0x80 0x00,

pressing Toggle Display (button 11) sets LED index 11, register value 0x0800, transmitted as 0x00 0x08,

pressing Cycle Brightness (button 15) sets LED index 15, register value 0x8000, transmitted as 0x00 0x80.

If another toggle-backed LED is already latched on, the transmitted SPI value will be the OR-combination of the active bits instead of the single-bit value shown in the table.

## 5. Local Actions Versus UART Actions

### 5.1 Commands That End Up On UART

These actions send a UART command or message to the Raspberry Pi:

previous track

next track

skip forward

skip back

play/pause

stop

next menu

menu select

cover view toggle

meter toggle

rotary action

Raspberry Pi shutdown in power-down paths

### 5.2 Actions That Stay Local On The ESP32

These actions currently stay local:

power sequencing,

DAC relay toggle,

display blanking via monitor PWM,

brightness cycling,

relay shutdown sequencing,

heartbeat wait and timeout handling.

## 6. Currently Unmapped Or Not Exposed Through A Dedicated Button

Several protocol commands exist but are not currently exposed as separate dedicated button entries in createButtonActionMap(), including:

CMD_PREV_MENU_ITEM

CMD_EXIT_ITEM

CMD_SYS_NOHEARTBEAT

This does not mean they are invalid. It only means there is no direct current button assignment for them.

## 7. Related Docs

docs/project-guide.md

docs/architecture.md

docs/power-sequencing.md

docs/protocol-reference.md

Source: docs/add-button-how-to.md

# How To Add A New Button Action

This guide shows the current firmware path for adding another button-driven action.

It covers three common cases:

a button that sends a simple UART command to the Raspberry Pi,

a button that sends a UART message with parameters,

a button that stays local on the ESP32 and does not send UART.

The current canonical ownership is:

button IDs and hardware pin constants: lib/board/boardConfig.hpp

control-board button constants: lib/app/ControlBoardButtonIds.hpp

action object declarations: lib/input/actions/buttonActions.hpp

action object definitions: lib/input/actions/buttonActions.cpp

button-to-action map: lib/app/ControlBoardActionRegistry.cpp

command routing and side effects: lib/app/actionProcessor.cpp

UART translation layer: lib/app/ActionUartDispatcher.cpp

protocol command IDs: lib/protocol/uartProtocol.hpp

Do not add new work to the legacy compatibility wrappers under lib/controlSystem, lib/actions, or lib/buttons unless you are intentionally maintaining backward compatibility. The canonical implementation lives in the files listed above.

## 1. Decide What Kind Of Button You Are Adding

Before editing code, decide which of these paths matches the behavior you want.

### 1.1 Simple UART Command

Use this when one press should produce one command ID and send it to the Raspberry Pi.

Examples already in the codebase:

next track,

play/pause,

next menu,

cycle brightness if it were remote instead of local.

Typical action type:

SimpleCommandAction in lib/input/actions/buttonActions.cpp

or MomentaryAction<CMD> from lib/input/actions/actionTemplates.hpp

### 1.2 Parameterized UART Command

Use this when the button action needs more than a raw command ID, such as a toggle state or a direction value.

Examples already in the codebase:

cover view toggle,

meter toggle,

rotary direction.

Typical action type:

ToggleAction<CMD_ON, CMD_OFF>

RotaryAction<CMD_ROTARY_ACTION>

These actions usually need a translation step in lib/app/ActionUartDispatcher.cpp so the final UART packet can carry parameters.

### 1.3 Local-Only Action

Use this when the button should affect hardware or local firmware state without sending anything to the Raspberry Pi.

Examples already in the codebase:

DAC relay toggle,

display blank/unblank,

brightness cycling,

power state transitions.

These actions still produce an ActionResponse, but the side effect happens in lib/app/actionProcessor.cpp rather than in the UART dispatcher.

## 2. Step-By-Step: Add The Command ID First

If the behavior needs a new command ID, define it first in lib/protocol/uartProtocol.hpp.

Steps:

Pick a command name consistent with the current naming scheme, such as CMD_FOO_BAR.

Pick a unique numeric value that does not collide with existing commands.

If the Raspberry Pi is involved, update the Pi-side listener script so it recognizes the new command.

If the command should appear in docs, update docs/protocol-reference.md and later update docs/button-command-map.md.

Rule of thumb:

if the command is purely local and never crosses UART, it can still use a CMD_* identifier, because the action routing layer classifies commands by meaning, not only by transport.

## 3. Step-By-Step: Add Or Reserve A Button ID

If you are adding a brand-new physical button, reserve its ID in both of these files:

lib/board/boardConfig.hpp

lib/app/ControlBoardButtonIds.hpp

Steps:

Add a new constant in board::buttons, for example kMute = 16.

Add the matching constant in controlSystem::controlBoardButtons, using the same numeric value.

Increase kCount if the total button count changed.

If the new button corresponds to a real MCP input or LED position, make sure the wiring and caller assumptions still hold.

Important note:

the current button LED logic uses the button ID as the SPI LED index for non-power buttons, so changing indices also changes LED bit positions.

## 4. Step-By-Step: Create The Action Object

Define the action object in the canonical input action files.

Files:

declarations: lib/input/actions/buttonActions.hpp

definitions: lib/input/actions/buttonActions.cpp

Choose the smallest action type that matches the behavior.

### 4.1 For A Simple UART Command

In lib/input/actions/buttonActions.cpp, add a SimpleCommandAction instance:

static SimpleCommandAction sMuteCmd(CMD_MUTE);
ButtonAction &MuteInstance = sMuteCmd;

In lib/input/actions/buttonActions.hpp, declare it:

extern ButtonAction &MuteInstance;

### 4.2 For A Toggle Or Timed Action

In lib/input/actions/buttonActions.hpp, add a type alias if needed:

using ToggleMute = ToggleAction<CMD_MUTE_ON, CMD_MUTE_OFF>;
extern ToggleMute ToggleMuteInstance;

In lib/input/actions/buttonActions.cpp, define it:

ToggleMute ToggleMuteInstance;

### 4.3 For A Local-Only Action

You still create an action object the same way. The difference is where it is handled later.

Example pattern:

static SimpleCommandAction sToggleFanCmd(CMD_TOGGLE_FAN);
ButtonAction &ToggleFanInstance = sToggleFanCmd;

## 5. Step-By-Step: Add The Button To The Registry

Bind the physical button ID to the action instance in lib/app/ControlBoardActionRegistry.cpp.

Steps:

Find ControlBoardActionRegistry::populate(...).

Add a map entry using the canonical button ID.

Point it at the action object you created.

Example:

rActionMap[controlBoardButtons::kMute] = &actions::MuteInstance;

At this point, the input side knows which action to execute when that button is pressed.

## 6. Step-By-Step: Wire The Behavior

This is where the three paths split.

### 6.1 Path A: Simple UART Command

If the action returns a command ID that should be sent directly as UART, update lib/app/ActionUartDispatcher.cpp.

Steps:

Add an entry to commandConfigs[].

Choose a short log tag string used for UART logging.

Make sure the command is classified as UartDispatch by the routing policy.

Example:

{"MUTE", CMD_MUTE}

If the command is already routed to ActionCommandRoute::UartDispatch, handleSimpleCommand() will send it automatically.

### 6.2 Path B: UART Message With Parameters

If the action needs parameters, add a dedicated handler in lib/app/ActionUartDispatcher.cpp.

Current examples to copy:

handleCoverViewCommand(...)

handleMeterCommand(...)

handleRotaryCommand(...)

Steps:

Add a small handler that recognizes the action response command.

Build a UartMessage.

Fill commandId and any params[] entries.

Send it through mrUartCommandSink.sendUartMessage(...).

Call that handler from ActionUartDispatcher::handle(...).

Use this path when a raw sendUartCommand() would lose necessary state.

### 6.3 Path C: Local-Only Action

Handle the new command in lib/app/actionProcessor.cpp.

Steps:

Decide which route class the command should use.

Add or update the classification logic if needed.

Extend the relevant handler, or add a new helper if the behavior deserves its own slice.

Perform the local side effect there.

Current examples:

handleRelayCommand(...)

handleDisplayCommand(...)

handleBrightnessCommand(...)

handleCommandPowerStateChange(...)

For a local-only button, the usual shape is:

action object emits CMD_*,

routing policy classifies that command as local,

ActionProcessor performs the hardware or state change,

no UART packet is sent.

## 7. Step-By-Step: Check Routing Classification

If the new command is not behaving correctly, the first thing to verify is its route classification.

The decision point is the routing layer used by lib/app/actionProcessor.cpp, via lib/app/ActionCommandRoutingPolicy.hpp.

You will typically want one of these outcomes:

UartDispatch for remote commands,

Relay for local relay changes,

Display for local display actions,

Brightness for local brightness changes,

PowerStateTransition for power-button-style behavior,

System for system-level commands.

If a new command is silently ignored or sent over the wrong path, this classification is usually the reason.

## 8. Step-By-Step: Update The Raspberry Pi Side When Needed

If the new button sends UART, update the corresponding Pi-side logic.

Typical files:

scripts/rpi/home/antho/UAart5Listener.py

any Pi-side application code that reacts to the command

Steps:

Add support for the new command ID.

If parameters are used, document and decode them consistently.

Keep the firmware-side name, numeric ID, and parameter meaning aligned.

## 9. Step-By-Step: Update The Docs

Once the button works, update the two reference docs.

Files:

docs/button-command-map.md

docs/protocol-reference.md

Update:

button index,

action type,

raw command ID,

final routed command,

final effect,

whether it is UART or local.

## 10. Worked Examples

### 10.1 Example: Add A Simple UART Button

Goal:

add a Mute button that sends CMD_MUTE to the Raspberry Pi.

Files to touch:

lib/protocol/uartProtocol.hpp

lib/board/boardConfig.hpp

lib/app/ControlBoardButtonIds.hpp

lib/input/actions/buttonActions.hpp

lib/input/actions/buttonActions.cpp

lib/app/ControlBoardActionRegistry.cpp

lib/app/ActionUartDispatcher.cpp

Pi-side listener file

docs/button-command-map.md

docs/protocol-reference.md

### 10.2 Example: Add A Local Relay Button

Goal:

add a Toggle Fan button that flips a relay on the ESP32 and never sends UART.

Files to touch:

lib/protocol/uartProtocol.hpp

lib/board/boardConfig.hpp

lib/app/ControlBoardButtonIds.hpp

lib/input/actions/buttonActions.hpp

lib/input/actions/buttonActions.cpp

lib/app/ControlBoardActionRegistry.cpp

routing policy if needed

lib/app/actionProcessor.cpp

docs/button-command-map.md

## 11. Quick Checklist

Use this checklist when adding a button:

Add or confirm the CMD_* ID.

Add or confirm the physical/control-board button ID.

Declare and define the action object.

Register the button in ControlBoardActionRegistry.

Route it to either UART dispatch or local handling.

Update Pi-side handling if UART is involved.

Update the button and protocol docs.

Run a firmware build and verify the expected side effect.

## 12. Validation

The normal validation step for this repo is the PlatformIO Build task from the workspace root.

If the button is UART-backed, also verify:

the log tag exists in ActionUartDispatcher,

the Raspberry Pi side understands the command,

any parameter encoding is documented.

If the button is local-only, verify:

the routing policy sends it to the local handler,

no accidental UART command is emitted,

the correct relay, indicator, or state change actually occurs.

Source: docs/project-structure-plan.md

# TinyControlBoard Structure Plan

## Objective

The project is already moving toward a domain-based layout. The remaining problem is mixed ownership: canonical files now exist in some domain folders, while older compatibility and generic buckets still carry real behavior.

The target state is:

one canonical location per concept,

temporary wrappers only where needed for compatibility,

domain folders for behavior, generic folders only for orchestration or shared support.

## Target Layout

src/
  main.cpp
  main.h

lib/
  app/
    ActionCommandRoutingPolicy.hpp
    ActionUartDispatcher.cpp
    ActionUartDispatcher.hpp
    ControlBoard.cpp
    ControlBoard.hpp
    ControlBoardActionRegistry.cpp
    ControlBoardActionRegistry.hpp
    ControlBoardBootstrap.cpp
    ControlBoardBootstrap.hpp
    ControlBoardButtonIds.hpp
    ControlBoardInputDispatcher.cpp
    ControlBoardInputDispatcher.hpp
    SerialHeartbeatRouter.hpp
    SerialUartCommandSink.cpp
    SerialUartCommandSink.hpp
    actionProcessor.cpp
    actionProcessor.hpp

  board/
    boardConfig.hpp
    boardIdentity.hpp

  input/
    actions/
      actionsResponse.cpp
      actionsResponse.hpp
      actionTemplates.hpp
      buttonAction.hpp
      buttonActions.cpp
      buttonActions.hpp
      SimpleCommandAction.hpp
    buttons/
      mcpInputHandler.cpp
      mcpInputHandler.hpp
      project_cfg.hpp

  indicators/
    activityStatus.hpp
    led_definitions.hpp
    ledManager.hpp
    led_manager.cpp
    led_manager.hpp
    monitorBrightnessController.cpp
    monitorBrightnessController.hpp
    powerLed.cpp
    powerLed.hpp
    spiLedDriver.cpp
    spiLedDriver.hpp
    statusLed.cpp
    statusLed.hpp
    pwm/
      pwmLed.cpp
      pwmLed.hpp

  power/
    powerState.hpp
    PowerStateTransitionHandler.cpp
    PowerStateTransitionHandler.hpp
    PowerStateTransitionPolicy.hpp
    RelayController.cpp
    RelayController.hpp
    RPIBootManager.cpp
    RPIBootManager.hpp
    relays/
      relay.cpp
      relay.hpp

  protocol/
    uartProtocol.cpp
    uartProtocol.hpp

  transport/
    uart/
      serial.cpp
      serial.hpp
      uartReceiver.cpp
      uartReceiver.hpp

  support/
    controlSystemHelpers.cpp
    controlSystemHelpers.hpp

docs/
scripts/
  rpi/
host_tests/
test/

## Canonical Ownership Rules

### app/

Owns orchestration only:
- top-level wiring,
- callback registration,
- button and UART dispatch,
- composing domain services.

It should not own relay policy, boot waiting, board pin maps, or protocol constants.

### board/

Owns physical board facts:
- pins,
- ports,
- addresses,
- board identity,
- fixed timing values tied to hardware bring-up.

### input/

Owns human input and local action mapping:
- button capture,
- MCP expander details,
- rotary decoding,
- button-to-action lookup,
- reusable button action objects.

### indicators/

Owns user-visible feedback only:
- status LEDs,
- power LED behavior,
- brightness control,
- PWM LED helpers,
- SPI LED driver.

### power/

Owns power lifecycle behavior:
- relay abstraction,
- relay sequencing,
- boot and shutdown coordination,
- power-state transition policy,
- power-state types.

### protocol/

Owns wire format and command meaning:
- framing,
- packet interpretation,
- serialization,
- command IDs.

### transport/

Owns how bytes move, not what they mean:
- UART configuration,
- receive buffering,
- handshake GPIO behavior,
- transport-facing protocol adapters left over from legacy code.

### support/

Owns small shared helpers that are not themselves a subsystem.

## Exact Move Plan

### Implemented In This Pass

These ownership moves are now complete and new code should include the canonical locations directly:

| Old location | Canonical location | Status |
| --- | --- | --- |
| lib/controlSystem/RelayController.hpp | lib/power/RelayController.hpp | wrapper removed |
| lib/controlSystem/RelayController.cpp | lib/power/RelayController.cpp | wrapper removed |
| lib/controlSystem/RPIBootManager.hpp | lib/power/RPIBootManager.hpp | wrapper removed |
| lib/controlSystem/RPIBootManager.cpp | lib/power/RPIBootManager.cpp | wrapper removed |
| lib/controlSystem/PowerStateTransitionHandler.hpp | lib/power/PowerStateTransitionHandler.hpp | wrapper removed |
| lib/controlSystem/PowerStateTransitionHandler.cpp | lib/power/PowerStateTransitionHandler.cpp | wrapper removed |
| lib/controlSystem/PowerStateTransitionPolicy.hpp | lib/power/PowerStateTransitionPolicy.hpp | wrapper removed |
| lib/controlSystem/actionProcessor.* | lib/app/actionProcessor.* | wrapper removed |
| lib/controlSystem/ActionUartDispatcher.* | lib/app/ActionUartDispatcher.* | wrapper removed |
| lib/controlSystem/ControlBoard* | lib/app/ControlBoard* | wrapper removed |
| lib/controlSystem/SerialUartCommandSink.* | lib/app/SerialUartCommandSink.* | wrapper removed |
| lib/actions/* | lib/input/actions/ | wrapper removed |
| lib/buttons/* | lib/input/buttons/ | wrapper removed |

Current note:

The firmware build now compiles the canonical lib/app, lib/input, lib/power, lib/protocol, lib/support, and lib/transport/uart sources directly.

The remaining migration work is any deliberate cleanup of still-unused legacy helpers and any naming cleanup you still want to do.

### Next File Moves

| Current file or folder | Target | Reason |
| --- | --- | --- |
| legacy helper headers if reintroduced | subsystem-owned canonical headers | avoid recreating compatibility layers |
| new transport aliases | lib/transport/uart/ only | keep one transport surface |
| new Pi deployment notes | scripts/rpi/README.md | keep deployment guidance close to the tracked assets |

## Migration Sequence

Move ownership first, names second.

Keep compatibility headers or translation-unit wrappers until all includes and build discovery are stable.

Update new code to include canonical paths immediately after each move.

After one area is fully migrated, delete its wrappers in one follow-up change.

Rename mixed-case or misspelled files only after ownership has stabilized.

## Naming Cleanup Queue

These are worthwhile, but should happen after ownership cleanup rather than during it:

consider controlSystem/ -> app/

## Test Layout Recommendation

Keep both test areas, but make the distinction explicit:

host_tests/ for host-side CMake logic tests,

test/ for embedded or PlatformIO-targeted tests.

If you want the tree to read more cleanly later, move them to:

tests/host/

tests/embedded/

## Exit Criteria

The folder cleanup is done when all of the following are true:

every subsystem has exactly one canonical folder,

common/ contains only temporary wrappers or is gone,

app/ remains the only orchestration folder,

transport/uart/ remains the only transport surface,

lib/protocol/uartProtocol.* remains the only protocol surface,

visual code is entirely under indicators/,

Pi helper scripts are no longer stored under lib/.