PROJECT DOCUMENTATION PACK

TinyControlBoard Documentation

Consolidated engineering reference and implementation guide

Generated 2026-06-05 19:50

Included source documents

docs/TinyControlBoard-Documentation.md

# Table Of Contents

Table Of Contents	2

TinyControlBoard — Project Documentation	7

Table of Contents	7

1. Introduction	7

1.1 What This Project Does	7

1.2 System at a Glance	8

2. System Architecture	8

2.1 High-Level View	8

2.2 Architecture Diagram	8

2.3 Startup Flow	9

2.4 Core Components	10

2.4.1 ControlBoard	10

2.4.2 ActionProcessor	10

2.4.3 Serial (UartTransport)	11

2.4.4 RpiBootManager	11

2.4.5 SpiBootIndicator	12

2.4.6 SpiLedDriver	12

2.4.7 MonitorBrightnessController and Button LED Coupling	13

2.4.8 RelayController	13

2.5 Input Flow	14

2.6 UART Communication Flow	14

2.7 Power and Status State Model	15

2.8 Software Design Patterns	16

2.9 Data Flow Diagram (Input → Action → Output)	17

2.10 Concurrency Model	18

FreeRTOS Tasks	18

Synchronization Primitives	19

Interrupt Handling	20

2.11 Communication Protocols Summary	20

2.12 Error Handling and Fault Recovery	21

2.13 Notable Design Characteristics	22

3. Hardware Reference	23

3.1 ESP32-S3 GPIO Pin Map	23

3.2 Raspberry Pi Side Wiring	24

UART5 and Handshake Pins	24

End-to-End Connection Table	25

Handshake Line Meaning	25

3.3 Front Panel Connector Map	26

3.4 Partition Table	28

4. UART Protocol Reference	29

4.1 Packet Format	29

4.2 Message Types	30

4.3 Application IDs	30

4.4 Command ID Catalog	31

4.5 Checksum	33

4.6 Heartbeat	33

4.7 Pi Script Command Handling	33

4.8 Protocol Limitations	34

5. Button and Command Map	35

5.1 Action Types	35

5.2 Button Index Reference	36

5.3 Full Button Mapping Table	37

5.4 Per-Button Notes	39

5.5 Local vs UART Actions	39

6. Power Sequencing	40

6.1 Power States	40

6.2 Timing Constants	41

6.3 Power Button Behavior	42

6.4 Power-On Sequence	42

6.5 Sleep Sequence	42

6.6 Deep-Sleep Sequence	42

6.7 Heartbeat Role in Power Sequencing	43

6.8 Relay GPIO Assignments	43

6.9 Current Behavior Caveats	44

6.10 Debug Flag: Simulating Pi Boot	44

7. Raspberry Pi Setup	45

7.1 What Runs on the Pi	45

7.2 UART Listener	45

7.3 Heartbeat Sender	45

7.4 Required Pi Configuration	46

7.5 Systemd Services	47

7.6 Pi File Locations	48

8. Developer Guides	49

8.1 Build and Test	49

Firmware Build	49

Host-Side Tests	49

Device Tests	49

8.2 How to Add a New Button Action	50

Step 1: Add the Command ID	51

Step 2: Add or Reserve a Button ID	51

Step 3: Register the Button	51

Step 4: Wire the Behavior	52

Step 5: Update the Pi Side (if UART)	52

Step 6: Quick Checklist	52

Worked Examples	53

8.3 Project Structure and Folder Ownership	54

Current Folder Map	54

8.4 Configuration Constants Reference	55

Timing Constants (board::timing in lib/board/boardConfig.hpp)	55

UART Constants (board::serial in lib/board/boardConfig.hpp)	56

NVS Keys	57

8.5 Dependencies and Third-Party Libraries	58

UART5 Listener Setup on Moode (Raspberry Pi)	59

Repo Layout	59

1. Install Moode Using Raspberry Pi Imager	60

2. Install Required Packages	60

3. Enable UART5	60

4. Install Python Serial Support	60

4.1 Preferred Installation (pip)	60

4.2 If pip Is Missing	60

4.3 If You Encounter externally-managed-environment	61

5. Install pigpio	61

5.1 Install Build Tools	61

5.2 Download and Build pigpio	61

5.3 Enable and Start pigpiod	61

5.4 Create pigpiod systemd Service (Manual)	62

6. Create the UART Listener systemd Service	63

7. Create the UART Listener Script	63

8. Enable and Start the UART Listener Service	63

9. Create the Heartbeat Sender Service	63

9.1 Create the Script	63

9.2 Create the systemd Service	64

9.3 Enable and Start the Service	64

10. File Locations Summary	65

11. Set auto logging	65

12. Hide the login Text	65

13  Add the splash Screen	65

add meters	65

14. Enable Auto-Login on Local Console	66

15. Troubleshooting	66

15.1 UART5 Not Working	66

15.2 Service Not Starting	66

15.3 Permission Denied Accessing UART	66

15.4 Python serial Module Not Found	67

Source: docs/TinyControlBoard-Documentation.md

# TinyControlBoard — Project Documentation

This is the consolidated reference document for the TinyControlBoard project. It covers the firmware architecture, hardware wiring, UART protocol, button mapping, power sequencing, Raspberry Pi setup, and developer guides.

The end-user operating guide is kept as a separate document: stream-dac-front-panel-user-guide.md

## 1. Introduction

### 1.1 What This Project Does

TinyControlBoard is an ESP32-S3-based control surface that:

reads physical buttons and rotary input,

controls relays and indicator outputs,

talks to a Raspberry Pi over UART,

reacts to Raspberry Pi heartbeat status,

translates user input into commands such as playback control, display control, DAC control, and power-state changes,

visually signals boot progress and failure via SPI-driven button LEDs.

In practical terms, the board is the hardware front end and the Raspberry Pi is the system it controls.

## 2. System Architecture

### 2.2 High-Level View

The firmware is built around one top-level orchestrator: ControlBoard.

At runtime, the system does four main jobs:

initialize the board and connected hardware,

read user input from buttons and rotary input,

convert those inputs into local actions or UART commands,

monitor the Raspberry Pi link and react to heartbeat state.

### 2.1 System at a Glance

The main runtime flow is:

- The ESP32 boots and waits briefly for power to settle.

app_main() creates a ControlBoard instance.

ControlBoard::init() initializes LEDs, relays, input handling, and serial communication.

Button and rotary events are converted into std::unique_ptr<actions::IAction> objects via ActionFactory::createAction().

ActionProcessor::process() executes the action via IAction::execute(ActionContext&).

Some actions change local hardware state; others send UART commands to the Raspberry Pi.

The Raspberry Pi sends heartbeat packets back so the ESP32 knows the Pi is still online. Primary entry points:

src/main.cpp

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

### 2.2 Architecture Diagram

Diagram 1

### 2.3 Startup Flow

Current startup sequence:

- app_main() waits 5 seconds for power to settle.

A ControlBoard instance is created.

NVS flash is initialized.

bootstrap::prepareStartupIndicators() sets the power LED to TURNING_ON.

The UART and relay singletons are acquired; serial RX and heartbeat-timeout callbacks are registered.

ActionProcessor and ControlBoardInputDispatcher are constructed.

Relays are initialized and set to default off states.

The MCP input handler is initialized.

The ButtonEventQueue task is started.

Button press, release, and rotary callbacks are registered.

UART hardware is initialized.

The button-to-action map is populated via ControlBoardActionRegistry.

After a board::timing::k_initDelayMs delay, triggerInitialPowerOn() is called which starts the full power-on sequence.

Note: if any critical init step fails (MCP handler or serial setup), SpiBootIndicator::notifyFailure() is called immediately, which flashes all SPI LEDs rapidly to signal the fault before the firmware aborts init.

References:

- src/main.cpp

- lib/app/ControlBoard.cpp

- lib/app/ControlBoardBootstrap.cpp

- lib/board/boardConfig.hpp

### 2.4 Core Components

#### 2.4.1 ControlBoard

ControlBoard is the top-level integration class. It owns the component instances and routes events. Initialization logic is split into helper namespaces:

bootstrap:: functions (in ControlBoardBootstrap.cpp) handle startup step sequencing.

ControlBoardActionRegistry populates the button-to-action map.

ControlBoardInputDispatcher translates button/rotary events into IAction objects and manages per-button LED state.

ButtonEventQueue serializes press, release, and rotary events onto a FreeRTOS queue processed by ControlBoardInputDispatcher.

Code References:

lib/app/ControlBoard.hpp

lib/app/ControlBoard.cpp

lib/app/ControlBoardBootstrap.hpp

lib/app/ControlBoardActionRegistry.hpp

lib/app/ControlBoardInputDispatcher.hpp

lib/app/ButtonEventQueue.hpp

#### 2.4.2 ActionProcessor

ActionProcessor receives a std::unique_ptr<actions::IAction> and executes it via the ActionContext services struct.

Side effects are implemented in concrete IAction subclasses under lib/app/commands/:

RelayAction — local relay changes,

BrightnessAction — local brightness changes,

UartDispatchAction — sends UART commands to the Raspberry Pi,

PowerTransitionAction — handles power-state transitions,

DisplayAction — display on/off control,

SystemAction — system-level commands.

ActionFactory::createAction() maps a CommandId to the appropriate concrete type via ActionCommandRoutingPolicy.

Code References:

lib/app/actionProcessor.hpp

lib/app/actionProcessor.cpp

lib/app/ActionFactory.hpp

lib/app/ActionContext.hpp

lib/app/ActionCommandRoutingPolicy.hpp

#### 2.4.3 Serial (UartTransport)

The serial layer owns the ESP32 side of the Pi link. It does three distinct jobs:

configure and use UART2,

manage the two extra data-ready handshake GPIOs,

track heartbeat timing and pass completed messages up through a callback.

Code references:

lib/transport/uart/serial.hpp

lib/transport/uart/serial.cpp

#### 2.4.4 RpiBootManager

RpiBootManager is a synchronization helper built around a FreeRTOS event group. It does not directly control hardware. Instead, it waits for lifecycle signals:

heartbeat received means the Pi is alive or has finished booting,

heartbeat timeout is treated as shutdown/offline confirmation.

The board::debug::kSimulateRpiBoot flag in lib/board/boardConfig.hpp can be set to true to skip the heartbeat wait entirely. This is useful when testing on the bench without a Raspberry Pi. The flag is evaluated at compile time (if constexpr) so there is zero overhead in release builds.

Code References:

lib/power/RPIBootManager.hpp

lib/power/RPIBootManager.cpp

#### 2.4.5 SpiBootIndicator

SpiBootIndicator is a FreeRTOS-based visual boot indicator that flashes all 16 SPI-driven button LEDs to signal boot progress or failure.

Behavior:

startWaiting() — starts a background FreeRTOS task that flashes all LEDs at 500 ms half-period (1 Hz) while the firmware waits for the Raspberry Pi heartbeat.

notifySuccess() — signals the task to stop and clears all LEDs. Called when the heartbeat is received within the timeout.

notifyFailure() — switches to a fast 150 ms half-period flash (~3.3 Hz). If the task is not yet running (firmware init failure), it starts the task directly in the failed state.

Call sites:

PowerStateTransitionHandler calls startWaiting() just before waitForRpiToBoot(), then calls notifySuccess() or notifyFailure() based on the result.

ControlBoard::init() calls notifyFailure() before each early-return failure path.

Code References:

lib/indicators/SpiBootIndicator.hpp

lib/indicators/SpiBootIndicator.cpp

#### 2.4.6 Spi Led Driver

SpiLedDriver drives 16 button LEDs via SPI shift registers (SPI2_HOST, 1 MHz, GPIO 6/7/5 for clock/MOSI/latch).

Key methods:

init() — sets up the SPI bus and device.

setLed(index, on) — sets a single LED by bit index (0–15).

setAllLeds(on) — atomically sets all 16 LEDs on or off. Used by SpiBootIndicator for whole-panel flashing.

update() — pushes the current 16-bit state to the shift register via SPI.

The overall brightness of all button LEDs is controlled by a PWM duty on GPIO 21 (k_buttonLedPwmPin) through the StatusLed instance registered as s_buttonStatusLed. The duty is updated by MonitorBrightnessController whenever screen brightness changes so the two track together.

Code References:

lib/hal/leds/spiLedDriver.hpp

lib/hal/leds/spiLedDriver.cpp

#### 2.4.7 MonitorBrightnessController and Button LED Coupling

MonitorBrightnessController owns the PWM duty on the monitor brightness pin and also drives the button LED brightness level. Every method that changes screen brightness calls getButtonStatusLed().setIdleDuty() with an inverted mapping:

level 0 (screen dimmest) → button LEDs dimmest,

level 9 (screen brightest) → button LEDs brightest.

The inversion is applied inside buttonDutyForLevel() because the button LED circuit is active-low (higher LEDC duty → dimmer output).

setIdleDuty() on StatusLed is thread-safe (std::atomic<uint32_t> m_idleDuty) and takes effect immediately when the LED task is in SolidIdle state.

The coupling is active through: init(), changeBrightnessLevel(), cycleBrightness(), setState() ON path, toggleDisplayOffOn(), and clearDisplayOffMode().

Code References:

lib/indicators/monitorBrightnessController.hpp

lib/indicators/MonitorBrightnessController.cpp

#### 2.4.8 RelayController

RelayController is a thin helper around the relay abstraction. It provides:

relay state changes with optional delay,

DAC relay toggling,

Raspberry Pi relay shutdown,

screen relay shutdown.

StandardRelay::setRelayState() returns a bool — true if gpio_set_level() succeeded, false on driver error. Physical contact state is not detectable without dedicated feedback hardware.

Code References:

lib/power/RelayController.hpp

lib/power/RelayController.cpp

lib/hal/relay/relay.hpp

### 2.5 Input Flow

The current input path is:

- MCP interrupt fires; McpInputHandler decodes it as a press, release, or rotary event.

The event is enqueued into ButtonEventQueue.

ControlBoardInputDispatcher dequeues the event and looks up the ButtonConfig for that button index.

ButtonConfig::action->produce(isPressed) is called on the IActionSource to obtain a std::unique_ptr<actions::IAction>.

ControlBoardInputDispatcher applies LedPolicy (None / Momentary / Toggle) to the SPI LED state for that button.

The IAction is passed to ActionProcessor::process().

ActionProcessor builds an ActionContext and calls iaction->execute(ctx).

The concrete command class performs the side effect (relay, UART, brightness, power transition, etc.).

Code References

lib/hal/buttons/mcpInputHandler.hpp

lib/app/ButtonEventQueue.hpp

lib/app/ControlBoardInputDispatcher.hpp

lib/input/actions/actionTemplates.hpp

lib/app/ActionFactory.hpp

lib/app/ActionContext.hpp

### 2.6 UART Communication Flow

ESP32 → Pi path:

- An action processor branch decides to send a command.

A UartMessage or command ID is passed to Serial.

Serial::sendData() writes bytes on UART2.

The ESP32 raises its data-ready pin (GPIO41) briefly so the Pi knows to read.

Pi → ESP32 path:

- The Raspberry Pi writes a UART packet.

The Pi pulses its data-ready line (GPIO42).

The ESP32 ISR wakes the UART RX task.

Incoming bytes are added to the receiver buffer.

Complete messages are deserialized and passed to the registered callback.

Current limitation: ControlBoard::handleSerialRxMessage() actively handles heartbeat messages. Other received UART messages are routed through a minimal inbound scaffold and logged, but protocol-specific behavior is still pending.

### 2.7 Power and Status State Model

Power LED states (lib/power/powerState.hpp):

OFF

TURNING_ON

ON

SLEEP

GOING_TO_SLEEP

DEEPSLEEP

SHUTTING_DOWN

GOING_INTO_DEEP_SLEEP

Activity/status LED states (lib/indicators/activityStatus.hpp):

doingWork

Idle

SolidIdle

sleeping

MaintenanceMode

Active

### 2.8 Software Design Patterns

The list of identified design patterns and where they are used

| Pattern | Where Used | Notes |
| --- | --- | --- |
| Command | IAction / IActionSource hierarchy | Each button press is converted to a unique_ptr<IAction>; execute(ctx) carries the side-effect. Adding a new command = new subclass in lib/app/commands/. |
| Factory | ActionFactory::createAction() | Maps CommandId → concrete IAction subclass via an internal switch. Centralises construction; callers never new command objects. |
| Strategy / Policy | ActionCommandRoutingPolicy | Table-driven command routing (constexpr CommandRouteEntry k_commandRouteTable[]). Default = UART dispatch; rows override for local-only actions. |
| Observer / Callback | UART RX callback, heartbeat timeout callback, MCP button/release/rotary callbacks | Producers register std::function callbacks; no direct coupling between HAL and application layers. |
| State Machine | ControlBoardPowerState + PowerStateTransitionHandler | Eight named states; handle() transitions between them based on IAction type and hold-time. |
| Event Queue | ButtonEventQueue | ISR-safe FreeRTOS queue; decouples GPIO ISR from application-thread processing. |
| Singleton | UartTransport::getInstance(), indicator accessors in ledManager.hpp | Used only for hardware peripherals that have a single physical instance. |
| RAII | ButtonEventQueue destructor, UartTransport destructor | FreeRTOS task handles and queue handles are cleaned up on destruction. |
| Service Locator (limited) | indicators::getPowerLed(), getMonitorBrightnessController(), etc. | Free-function accessors in ledManager.hpp return references to static indicator instances. |
| Table-driven registration | ControlBoardActionRegistry (constexpr ButtonRegistration k_buttons[]) | Adding a button = one table row + one CMD_* constant; no procedural wiring. |

### 2.9 Data Flow Diagram (Input → Action → Output)

Diagram 2

### 2.10 Concurrency Model

#### FreeRTOS Tasks

| Task Name | Created In | Stack | Priority | Purpose |
| --- | --- | --- | --- | --- |
| uart_rx_task | lib/hal/uart/serial.cpp | 4096 B | 10 | Blocks on GPIO notify from data-ready ISR; drains UART FIFO; pushes bytes into ring buffer; fires RX callback per complete frame. |
| heartbeat_monitor | lib/hal/uart/serial.cpp | 4096 B | 5 | Polls every 100 ms; fires timeout callback when elapsed since last RX exceeds k_heartbeatTimeoutMs. |
| action_task | lib/app/ButtonEventQueue.cpp | 4096 B | 5 | Blocks on xQueueReceive; dequeues ButtonEvent structs and calls ControlBoardInputDispatcher::dispatch(). |
| mcp_int_task | lib/hal/buttons/mcpInputHandler.cpp | 4096 B | 10 | Woken by vTaskNotifyGiveFromISR from GPIO18 ISR; reads MCP23017 registers over I2C; fires button/rotary callbacks. |
| LED_Task (×N) | lib/indicators/statusLed.cpp | 4096 B | 5 | Drives one PWM indicator LED; implements blink / breathe / solid patterns. One instance per StatusLed object. |
| BreatheTask (×N) | lib/indicators/statusLed.cpp | 2048 B | 5 | Fade-in/fade-out effect helper spawned transiently by LED_Task. |
| boot_diag_leds | lib/indicators/BootDiagnosticLeds.cpp | 2048 B | idle+1 | Created only on first boot-stage failure; flashes SPI LEDs at 3 Hz until hardware reset. |

#### Synchronization Primitives

| Primitive | Where | Purpose |
| --- | --- | --- |
| FreeRTOS Queue (depth 16) | ButtonEventQueue | Decouples ISR/callback context from application task. |
| FreeRTOS Queue (depth 1) | StatusLed (per instance) | Passes ControlBoardWorkingStatus enums to LED_Task. |
| FreeRTOS EventGroup | RpiBootManager | Bit 0 = heartbeat received; Bit 1 = shutdown confirmed. |
| std::atomic<ControlBoardPowerState> | SystemState | Thread-safe power state shared between ActionProcessor and heartbeat callback. |
| std::atomic<uint16_t> | ControlBoardInputDispatcher::m_buttonLedBitmask | Per-button LED toggle state; updated with fetch_xor. |
| std::atomic<uint32_t> | StatusLed::m_idleDuty | Allows MonitorBrightnessController to update button LED brightness from any task context. |
| std::atomic<bool> | UartTransport::m_initialized, m_stopRxTask, m_stopHeartbeatTask | Guards task lifecycle without a mutex. |

#### Interrupt Handling

| ISR | Trigger | Action |
| --- | --- | --- |
| UartTransport::gpioIsrHandler (IRAM_ATTR) | GPIO42 POSEDGE (Pi data-ready) | vTaskNotifyGiveFromISR → wakes uart_rx_task |
| McpInputHandler::gpioIsr | GPIO18 (MCP23017 /INT) | vTaskNotifyGiveFromISR → wakes mcp_int_task |

Design rules: ISRs contain no I2C or UART calls; all slow bus work happens in the unblocked task. Both ISRs use the shared ISR service (gpio_install_isr_service). IRAM_ATTR ensures gpioIsrHandler runs from IRAM and is not blocked by flash cache misses.

### 2.11 Communication Protocols Summary

| Protocol | Interface | Speed / Settings | Role |
| --- | --- | --- | --- |
| UART (custom framed) | UART2 / GPIO1 (RX) / GPIO2 (TX) | 115 200 baud, 8N1 | Bi-directional link to Raspberry Pi. 18-byte frames. |
| I2C | I2C_NUM_0 / GPIO15 (SCL) / GPIO16 (SDA) | 50 000 Hz | Reads MCP23017 I/O expander at address 0x20. |
| SPI | SPI2_HOST / GPIO6 (CLK) / GPIO7 (MOSI) / GPIO5 (latch) | 1 MHz | Drives 16-bit shift register for button panel LEDs. |
| LEDC (PWM) | Multiple GPIO channels | 4 kHz / 13-bit | Monitor brightness (GPIO43), button LED PWM (GPIO21), power LED (GPIO3/4), status LED (GPIO48). |
| GPIO handshake | GPIO41 (out) / GPIO42 (in) | Logic level | Data-ready signalling: Pi raises GPIO42 before sending; ESP32 raises GPIO41 for 10 ms after sending. |
| NVS (SPI flash) | Internal flash | — | Persists brightness level across power cycles. |

### 2.12 Error Handling and Fault Recovery

| Scenario | Detection | Recovery |
| --- | --- | --- |
| MCP23017 init failure | McpInputHandler::begin() returns esp_err_t != ESP_OK | BootDiagnosticLeds::firmwareInitFailed() (all 8 LEDs flash at 3 Hz); main.cpp retries init() every 1 s. |
| Serial/UART init failure | Serial::initUart() returns false | Same path: flash all LEDs at 3 Hz + retry loop. |
| Individual power-on relay failure | StandardRelay::setRelayState() returns false | BootDiagnosticLeds::stageFailure(stage) for the failing stage; relay sequence halts. |
| Heartbeat timeout while ON | heartbeat_monitor fires timeout callback | ActionProcessor::handleHeartbeatTimeout() forces powerState to SLEEP. |
| Pi boot timeout (60 s) | RpiBootManager::waitForRpiToBoot() returns false | BootDiagnosticLeds::stageFailure(BootStage::RpiComms); relay power-on considered failed. |
| GPIO driver error | gpio_set_level() returns non-OK | StandardRelay::setRelayState() returns false; caller decides whether to abort. |
| NVS key not found on first boot | NvsStorage::readInt8() returns false | MonitorBrightnessController::init() uses compile-time default level 2; written to NVS on first brightness change. |
| FreeRTOS task creation failure | xTaskCreate() returns != pdPASS | Logged via ESP_LOGE; init step returns false, triggering the retry path. |

### 2.13 Notable Design Characteristics

ControlBoard is the integration hub and currently owns a lot of orchestration.

Input actions are represented as objects, making it straightforward to remap buttons without rewriting processor logic.

Toggle actions maintain internal software state, so their first emitted command depends on the starting state in firmware.

Heartbeat is treated as the Raspberry Pi liveness signal for both boot completion and shutdown detection.

Non-heartbeat Pi-originated commands are not yet fully consumed on the ESP32 side.

Boot and init failures are signalled visually via SpiBootIndicator: slow flashing (~1 Hz) during normal boot wait, fast flashing (~3.3 Hz) on timeout or firmware init failure.

The board::debug::kSimulateRpiBoot compile-time flag allows full firmware testing without a connected Raspberry Pi.

Button LED brightness tracks monitor brightness automatically via MonitorBrightnessController using an inverted active-low mapping.

StandardRelay::setRelayState() returns a bool indicating GPIO driver success. Physical relay contact state cannot be detected without additional feedback hardware.

## 3. Hardware Reference

### 3.1 ESP32-S3 GPIO Pin Map

| GPIO Pin | Direction | Connected To | Protocol | Notes |
| --- | --- | --- | --- | --- |
| 1 | Input | Raspberry Pi TX | UART2 RX | Main Pi → ESP32 data |
| 2 | Output | Raspberry Pi RX | UART2 TX | Main ESP32 → Pi data |
| 3 | Output | App active LED | LEDC PWM |  |
| 4 | Output | App standby LED | LEDC PWM |  |
| 5 | Output | SPI LED latch | SPI2 | Active rising edge to latch shift register |
| 6 | Output | SPI LED clock | SPI2 |  |
| 7 | Output | SPI LED data (MOSI) | SPI2 |  |
| 9 | Output | Output stage power relay | GPIO | Active high |
| 10 | Output | Prototype DAC enable relay | GPIO | Active high |
| 11 | Output | Raspberry Pi power relay | GPIO | Active high |
| 12 | Output | DAC power relay | GPIO | Active high |
| 13 | Output | Screen power relay | GPIO | Active high |
| 15 | Output | I2C SCL | I2C_NUM_0 | MCP23017 clock |
| 16 | I/O | I2C SDA | I2C_NUM_0 | MCP23017 data |
| 17 | Output | MCP23017 enable | GPIO | Active high |
| 18 | Input | MCP23017 /INT | GPIO (ISR) | Active low interrupt, POSEDGE ISR trigger |
| 21 | Output | Button LED PWM | LEDC PWM | Overall brightness for all 16 SPI button LEDs |
| 39 | Output | General relay 2 | GPIO | Spare |
| 41 | Output | Pi data-ready (to Pi) | GPIO | Pulsed high 10 ms after sending |
| 42 | Input | Pi data-ready (from Pi) | GPIO (ISR) | POSEDGE wakes uart_rx_task |
| 43 | Output | Monitor brightness | LEDC PWM | Analogue brightness to display |
| 47 | Output | General relay 1 | GPIO | Spare |
| 48 | Output | Working status LED | LEDC PWM |  |

### 3.2 Raspberry Pi Side Wiring

#### UART5 and Handshake Pins

| Function | Raspberry Pi BCM | Physical Pin | Notes |
| --- | --- | --- | --- |
| ESP32 → Pi data-ready input | 23 | 16 | Listener waits for rising edge here |
| Pi → ESP32 data-ready output | 24 | 18 | Sender script pulses this line |
| UART5 TX | 12 | 32 | Standard dtoverlay=uart5 mapping |
| UART5 RX | 13 | 33 | Standard dtoverlay=uart5 mapping |

#### End-to-End Connection Table

| Function | Raspberry Pi Side | ESP32-S3 Side |
| --- | --- | --- |
| Pi UART5 TX → ESP32 UART RX | BCM12, pin 32 | GPIO1 |
| Pi UART5 RX ← ESP32 UART TX | BCM13, pin 33 | GPIO2 |
| ESP32 → Pi data-ready | BCM23, pin 16 | GPIO41 |
| Pi → ESP32 data-ready | BCM24, pin 18 | GPIO42 |
| Ground | Pi GND | ESP32 GND |

Important rule: UART TX always connects to the other side's RX.

#### Handshake Line Meaning

GPIO41 (ESP32 output → Pi BCM23 input): ESP32 pulses this high for 10 ms after sending a UART packet to notify the Pi that data is waiting.

GPIO42 (Pi BCM24 output → ESP32 input): Pi raises this before sending to wake the ESP32 UART RX task via ISR.

Code  references:

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

### 3.3 Front Panel Connector Map

This table maps the physical front-panel connector pins to firmware symbols. The schematic image is: docs/schema.png

Power and Ground

| Connector Pin | Diagram Label | Notes |
| --- | --- | --- |
| 1 | Power On (LED) | App active / power-state LED path |
| 2 | Power StandBy (LED) | App standby / power-state LED path |
| 35–38 | GND | Ground returns |
| 39–40 | 5V | Supply rail |

Button Input Pins

| Connector Pin | Diagram Label | Firmware Button Index | Current Firmware Mapping | Match Status |
| --- | --- | --- | --- | --- |
| 3 | Btn_Power | 0 | kPower → CMD_SYS_POWER | matches |
| 5 | CMD_Previous_Track | 1 | kPrevTrack → CMD_PREVIOUS_TRACK | matches |
| 7 | CMD_Next_Track | 2 | kNextTrack → CMD_NEXT_TRACK | matches |
| 9 | CMD_Skip_Forrard | 3 | kSkipForward → CMD_SKIP_FORWARD | matches |
| 11 | CMD_Skip_Back | 4 | kSkipBack → CMD_SKIP_BACK | matches |
| 13 | CMD_Play_Pause | 5 | kPlayPause → CMD_PLAY_PAUSE | matches |
| 15 | CMD_Stop_Track | 6 | kToggleDisplay → CMD_STOP_TRACK | matches |
| 17 | CMD_Prev_Menu_Item | 7 | current firmware uses kCover → CMD_COVER_VIEW_ON/OFF | mismatch |
| 4 | CMD_Next_Menu_Item | 8 | kNextMenu → CMD_NEXT_MENU_ITEM | matches |
| 6 | CMD_Item_Select | 9 | kMenuSelect → CMD_ITEM_SELECT | matches |
| 8 | CMD_Toggle_Dac | 10 | kToggleDac → CMD_TOGGLE_DAC_ON/OFF | matches |
| 10 | CMD_Display_Off | 11 | kToggleDisplay → CMD_DISPLAY_OFF/ON | partial match |
| 12 | CMD_Toggle_Meter_On | 12 | kToggleMeter → CMD_TOGGLE_METER_ON/OFF | partial match |
| 14 | CMD_Rotary_Action | 13 | kRotaryEventLeft → CMD_ROTARY_ACTION param 0 | matches |
| 16 | CMD_Rotary_Action | 14 | kRotaryEventRight → CMD_ROTARY_ACTION param 1 | matches |
| 18 | CMD_Cycle_Brightness | 15 | kCycleBrightness → CMD_CYCLE_BRIGHTNESS | matches |

Note: Connector pin 17 is labelled CMD_Prev_Menu_Item on the schematic but current firmware assigns that slot to Cover. CMD_PREV_MENU_ITEM exists in the codebase but is not currently bound to any front-panel button.

SPI Button LED Output Pins

| Connector Pin | Diagram Label | SPI LED Output | Current Firmware Use |
| --- | --- | --- | --- |
| 19 | LED_0_1 | bit 0 | lights for Previous Track |
| 21 | LED_0_2 | bit 1 | lights for Next Track |
| 23 | LED_0_3 | bit 2 | lights for Skip Forward |
| 25 | LED_0_4 | bit 3 | lights for Skip Back |
| 27 | LED_0_5 | bit 4 | lights for Play/Pause |
| 29 | LED_0_6 | bit 5 | lights for Stop |
| 31 | LED_0_7 | bit 6 | lights for firmware button index 7 |
| 33 | LED_0_8 | bit 7 | lights for Next Menu |
| 20 | LED_0_9 | bit 8 | lights for Menu Select |
| 22 | LED_0_10 | bit 9 | lights for Toggle DAC |
| 24 | LED_0_11 | bit 10 | lights for Toggle Display |
| 26 | LED_0_12 | bit 11 | lights for Toggle Meter |
| 28 | LED_0_13 | bit 12 | present; not driven by current rotary path |
| 30 | LED_0_14 | bit 13 | present; not driven by current rotary path |
| 32 | LED_0_15 | bit 14 | lights for Cycle Brightness |
| 34 | LED_0_16 | bit 15 | present; currently unused by firmware |

### 3.4 Partition Table

| Name | Type | SubType | Offset | Size | Purpose |
| --- | --- | --- | --- | --- | --- |
| nvs | data | nvs | 0x9000 | 24 KB | Non-volatile key-value storage |
| phy_init | data | phy | 0xF000 | 4 KB | RF calibration data (managed by ESP-IDF) |
| factory | app | factory | 0x10000 | ~15.9 MB | Firmware image |

## 4. UART Protocol Reference

Primary source: lib/protocol/uartProtocol.hpp

Code references:

lib/transport/uart/serial.cpp

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

### 4.1 Packet Format

The current packet size is 18 bytes.

| Byte Offset | Field | Size |
| --- | --- | --- |
| 0 | start byte | 1 |
| 1 | protocol version | 1 |
| 2 | source app | 1 |
| 3 | message type | 1 |
| 4 | sequence | 1 |
| 5–6 | command ID | 2 |
| 7–16 | five 16-bit parameters | 10 |
| 17 | checksum | 1 |

Packet  constants:

start byte: 0xAA

protocol version: 0x01

packet size: 18

### 4.2 Message Types

| Name | Value |
| --- | --- |
| MSG_COMMAND | 0x01 |
| MSG_STATUS | 0x02 |
| MSG_ACK | 0x03 |
| MSG_NACK | 0x04 |

### 4.3 Application IDs

| Name | Value |
| --- | --- |
| APP_ESP32 | 0x01 |
| APP_PI | 0x02 |

### 4.4 Command ID Catalog

| Command | Value |
| --- | --- |
| CMD_NO_ACTION | 0x0000 |
| CMD_SYS_POWER | 0x0001 |
| CMD_SYS_RPI_SHUTDOWN | 0x0002 |
| CMD_SYS_HEARTBEAT | 0x0003 |
| CMD_SYS_NOHEARTBEAT | 0x0004 |
| CMD_NEXT_TRACK | 0x0100 |
| CMD_PREVIOUS_TRACK | 0x0101 |
| CMD_PLAY_PAUSE | 0x0102 |
| CMD_STOP_TRACK | 0x0103 |
| CMD_SKIP_FORWARD | 0x0104 |
| CMD_SKIP_BACK | 0x0105 |
| CMD_PREV_MENU_ITEM | 0x0106 |
| CMD_NEXT_MENU_ITEM | 0x0107 |
| CMD_ITEM_SELECT | 0x0108 |
| CMD_EXIT_ITEM | 0x0109 |
| CMD_TOGGLE_DAC_ON | 0x010A |
| CMD_DISPLAY_OFF | 0x010B |
| CMD_TOGGLE_METER_ON | 0x010C |
| CMD_TOGGLE_METER_OFF | 0x010D |
| CMD_DISPLAY_ON | 0x010E |
| CMD_TOGGLE_DAC_OFF | 0x010F |
| CMD_ROTARY_LEFT | 0x0110 |
| CMD_ROTARY_RIGHT | 0x0111 |
| CMD_ROTARY_ACTION | 0x0112 |
| CMD_TOGGLE_DAC | 0x0113 |
| CMD_TOGGLE_DISPLAY | 0x0114 |
| CMD_TOGGLE_METER | 0x0115 |
| CMD_CYCLE_BRIGHTNESS | 0x0116 |
| CMD_COVER_VIEW_ON | 0x0117 |
| CMD_COVER_VIEW_OFF | 0x0118 |
| CMD_TOGGLE_COVER_VIEW | 0x0119 |
| CMD_REPEAT_ON | 0x011A |
| CMD_REPEAT_OFF | 0x011B |
| CMD_TOGGLE_REPEAT | 0x011C |
| CMD_RANDOM_ON | 0x011D |
| CMD_RANDOM_OFF | 0x011E |
| CMD_TOGGLE_RANDOM | 0x011F |

### 4.5 Checksum

The checksum is computed as the sum of bytes 1 through 16, masked to 8 bits.

Pi-side Python expression: sum(packet_bytes[1:17]) & 0xFF

### 4.6 Heartbeat

Heartbeat command ID: CMD_SYS_HEARTBEAT = 0x0003

Current firmware compatibility note: the firmware also still accepts the legacy heartbeat command 0x9999.

Heartbeat behavior:

the ESP32 starts a heartbeat timeout monitor during initialization,

when a heartbeat packet is received, the action processor is notified,

if the timeout expires (30 seconds), the action processor is notified that the Pi is offline,

during power-on, absence of heartbeat for 60 seconds causes boot timeout.

### 4.7 Pi Script Command Handling

The Pi listener currently handles:

Raspberry Pi shutdown,

playback next/previous track,

play/pause, stop,

seek forward/back,

rotary next/previous,

meter toggle,

cover view toggle,

repeat/random toggle,

panel cycling (next/prev moOde UI panel via CDP WebSocket on port 9222).

Some commands use parameters:

CMD_ROTARY_ACTION: parameter 0 carries direction (0 = left, 1 = right),

CMD_TOGGLE_METER: parameter 0 for enabled/disabled,

CMD_TOGGLE_COVER_VIEW: parameter 0 for enabled/disabled,

CMD_TOGGLE_DISPLAY: parameter 0 or 1.

### 4.8 Protocol Limitations

Some older macro aliases still exist in the header alongside the current constants.

Non-heartbeat Pi-originated messages are not yet deeply handled in ControlBoard.

The Pi scripts and firmware must be kept aligned on command IDs and parameter semantics.

## 5. Button and Command Map

Code references:

lib/board/boardConfig.hpp

lib/app/ControlBoardActionRegistry.cpp

lib/input/actions/actionTemplates.hpp

### 5.1 Action Types

There are four main action patterns:

SimpleCommandAction — sends one command when the button is pressed.

ToggleAction — flips internal state on press and emits an ON or OFF command.

TimedAction — starts timing on press and emits one command on release with the held duration.

RotaryAction — emits CMD_ROTARY_ACTION with parameter 0 for left and 1 for right.

Behavior notes:

toggle actions change state on button press only,

timed actions emit their command on button release,

simple command actions do nothing on release,

LedPolicy (None / Momentary / Toggle) controls SPI LED feedback per button.

### 5.2 Button Index Reference

| Index | Symbolic Name |
| --- | --- |
| 0 | kPower |
| 1 | kPrevTrack |
| 2 | kNextTrack |
| 3 | kSkipForward |
| 4 | kSkipBack |
| 5 | kPlayPause |
| 6 | kToggleDisplay |
| 7 | kCover |
| 8 | kRepeat |
| 9 | kToggleRandom |
| 10 | kToggleDac |
| 11 | kNextPanel |
| 12 | kToggleMeter |
| 13 | kRotaryEventLeft |
| 14 | kRotaryEventRight |
| 15 | kCycleBrightness |

### 5.3 Full Button Mapping Table

For toggle-backed buttons, the action can emit one of two raw command IDs depending on current state, and the action processor may then normalize that into a different UART command.

| Index | Button Name | Action Type | SPI Bit Pattern | SPI TX (low, high) | Raw Command ID(s) | Final Routed Command | Final Effect |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Power | TimedAction | n/a | n/a | 0x0001 CMD_SYS_POWER | CMD_SYS_POWER | Enters ON, SLEEP, or DEEPSLEEP based on state + hold time; no SPI LED |
| 1 | Previous Track | Simple | 0000 0000 0000 0010 | 0x02 0x00 | 0x0101 CMD_PREVIOUS_TRACK | CMD_PREVIOUS_TRACK | UART command to Pi |
| 2 | Next Track | Simple | 0000 0000 0000 0100 | 0x04 0x00 | 0x0100 CMD_NEXT_TRACK | CMD_NEXT_TRACK | UART command to Pi |
| 3 | Skip Forward | Simple | 0000 0000 0000 1000 | 0x08 0x00 | 0x0104 CMD_SKIP_FORWARD | CMD_SKIP_FORWARD | UART command to Pi |
| 4 | Skip Back | Simple | 0000 0000 0001 0000 | 0x10 0x00 | 0x0105 CMD_SKIP_BACK | CMD_SKIP_BACK | UART command to Pi |
| 5 | Play/Pause | Simple | 0000 0000 0010 0000 | 0x20 0x00 | 0x0102 CMD_PLAY_PAUSE | CMD_PLAY_PAUSE | UART command to Pi |
| 6 | Stop | Simple | 0000 0000 0100 0000 | 0x40 0x00 | 0x0103 CMD_STOP_TRACK | CMD_STOP_TRACK | UART command to Pi |
| 7 | Cover | Toggle | 0000 0000 1000 0000 | 0x80 0x00 | 0x0117/0x0118 | 0x0119 CMD_TOGGLE_COVER_VIEW param 1/0 | Pi toggles cover view |
| 8 | Repeat | Toggle | 0000 0001 0000 0000 | 0x00 0x01 | 0x011A/0x011B | 0x011C CMD_TOGGLE_REPEAT param 1/0 | Pi toggles repeat mode |
| 9 | Toggle Random | Toggle | 0000 0010 0000 0000 | 0x00 0x02 | 0x011D/0x011E | 0x011F CMD_TOGGLE_RANDOM param 1/0 | Pi toggles random mode |
| 10 | Toggle DAC | Toggle | 0000 0100 0000 0000 | 0x00 0x04 | 0x010A/0x010F | local relay toggle only | Toggles DAC power relay |
| 11 | Next Panel | Simple | 0000 1000 0000 0000 | 0x00 0x08 | 0x0107 CMD_NEXT_MENU_ITEM | CMD_NEXT_MENU_ITEM | Pi cycles to next moOde panel |
| 12 | Toggle Meter | Toggle | 0001 0000 0000 0000 | 0x00 0x10 | 0x010C/0x010D | 0x0115 CMD_TOGGLE_METER param 1/0 | Pi meter display change |
| 13 | Rotary Left | Rotary | n/a | n/a | 0x0112 CMD_ROTARY_ACTION | CMD_ROTARY_ACTION param 0 | Pi interprets as previous/left |
| 14 | Rotary Right | Rotary | n/a | n/a | 0x0112 CMD_ROTARY_ACTION | CMD_ROTARY_ACTION param 1 | Pi interprets as next/right |
| 15 | Cycle Brightness | Simple | 1000 0000 0000 0000 | 0x00 0x80 | 0x0116 CMD_CYCLE_BRIGHTNESS | local brightness cycle | Cycles monitor and button LED brightness |

### 5.4 Per-Button Notes

Power button (index 0):
- Press starts a timer; release emits CMD_SYS_POWER with releaseTimeMillis.
- Hold < 3000 ms → sleep path; hold ≥ 3000 ms → deep-sleep path.
- No corresponding SPI LED.

Toggle buttons (Cover, Repeat, Random, DAC, Meter):
- Keep internal software state in the action object.
- First press uses the action's default state; subsequent presses alternate ON/OFF.
- LED retention (keepLedActive) keeps the LED latched when the toggle state is ON.

Rotary (indices 13/14):
- Both directions use the same command ID; direction is carried in parameter 0.
- Rotary movement does not currently update a button LED.

SPI LED notes:
- Button 0 (power) is skipped — setLed() is not called for it; SPI LED bit 0 is unused in the normal button path.
- For all other buttons, ledIndex = buttonId.
- The SPI driver transmits the 16-bit register low byte first: byte 0 = bits 0–7, byte 1 = bits 8–15.
- If a toggle-backed LED is latched on, the transmitted SPI value will be the OR-combination of all active bits.

### 5.5 Local vs UART Actions

Actions that send UART commands to the Pi:
previous track, next track, skip forward, skip back, play/pause, stop, next menu, menu select, panel cycling, cover view toggle, meter toggle, rotary action, Raspberry Pi shutdown during power-down paths.

Actions that stay local on the ESP32:
power sequencing, DAC relay toggle, brightness cycling, relay shutdown sequencing, heartbeat wait and timeout handling.

## 6. Power Sequencing

Code references :

lib/power/PowerStateTransitionHandler.cpp

lib/power/PowerStateTransitionPolicy.hpp

lib/power/RPIBootManager.cpp

lib/power/RelayController.cpp

lib/power/powerState.hpp

lib/board/boardConfig.hpp

### 6.1 Power States

List of available power states

| State | Meaning |
| --- | --- |
| OFF | Board is off |
| TURNING_ON | Power-on sequence in progress |
| ON | Fully operational |
| SLEEP | Raspberry Pi and screen powered down; DAC and output stage remain on |
| GOING_TO_SLEEP | Sleep shutdown sequence in progress |
| DEEPSLEEP | All subsystems powered down |
| SHUTTING_DOWN | Defined in enum; not yet fully used in current code paths |
| GOING_INTO_DEEP_SLEEP | Defined in enum; not yet fully used in current code paths |

### 6.2 Timing Constants

All timing constants live in board::timing namespace in lib/board/boardConfig.hpp. The long-press threshold lives in lib/power/PowerStateTransitionPolicy.hpp.

| Constant | Value (ms) | Meaning |
| --- | --- | --- |
| k_initDelayMs | 5000 | Post-boot settle delay before triggerInitialPowerOn() |
| k_powerSettleDelayMs | 1500 | Delay after enabling DAC and output-stage relays |
| k_screenOnDelayMs | 1000 | Delay for screen and Pi relay steps during power-on |
| k_rpiBootTimeoutMs | 60000 | Max time to wait for heartbeat after power-on |
| k_rpiShutdownTimeoutMs | 60000 | Max time to wait for heartbeat timeout during shutdown |
| k_rpiShutdownSettleDelayMs | 500 | Delay between Pi relay off and screen relay off |
| k_screenPowerOffDelayMs | 5000 | Delay after screen relay off before LED state changes |
| k_heartbeatTimeoutMs | 30000 | Inactivity window after which heartbeat is considered lost |
| kLongPressThresholdMs | 3000 | Separates sleep from deep sleep on power-button release |

### 6.3 Power Button Behavior

- Button press stores the current time.

Button release computes the hold duration.

Release emits CMD_SYS_POWER plus releaseTimeMillis.

ActionProcessor chooses the power transition based on current power state and held duration.

### 6.4 Power-On Sequence

Entered when current state is OFF, SLEEP, or DEEPSLEEP and the power button is released.

- Power LED state → TURNING_ON.

Screen relay enabled with 1000 ms delay.

DAC relay enabled with 1500 ms delay.

Output stage relay enabled with 1500 ms delay.

Raspberry Pi relay enabled with 1000 ms delay.

SpiBootIndicator::startWaiting() — all SPI LEDs begin flashing slowly (~1 Hz).

Firmware waits up to 60 seconds for Pi heartbeat.

Success: SpiBootIndicator::notifySuccess(), LEDs clear. Power LED → ON, activity status → Active.

Timeout: SpiBootIndicator::notifyFailure(), LEDs switch to fast flash (~3.3 Hz). Power LED → SLEEP, activity status → sleeping.

### 6.5 Sleep Sequence

Entered when current state is ON and power button hold < 3000 ms.

- Power LED state → GOING_TO_SLEEP.

CMD_SYS_RPI_SHUTDOWN sent over UART.

Firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout.

Raspberry Pi power relay turned off.

500 ms delay.

Screen power relay turned off.

5000 ms delay.

Power LED → SLEEP, activity status → sleeping.

Practical result: Raspberry Pi and screen are shut down; DAC and output stage power remain on.

### 6.6 Deep-Sleep Sequence

Entered when current state is ON and power button hold ≥ 3000 ms.

- Power LED state → GOING_TO_SLEEP.

CMD_SYS_RPI_SHUTDOWN sent over UART.

Firmware waits up to 60 seconds for shutdown confirmation via heartbeat timeout.

Raspberry Pi power relay turned off.

500 ms delay.

Screen power relay turned off.

DAC relay turned off.

Output stage relay turned off.

Power LED → DEEPSLEEP, activity status → sleeping.

Practical result: Raspberry Pi, screen, DAC, and output stage power all removed. Board logic still remains present to respond to a future power-button press.

### 6.7 Heartbeat Role in Power Sequencing

Heartbeat is the synchronization signal between the ESP32 and the Raspberry Pi.

Heartbeat received → sets the event-group bit for boot completion. Power-on waits for this.

Heartbeat timeout → sets the event-group bit for shutdown confirmation. Sleep and deep-sleep wait for this.

This means the system uses absence of heartbeat as a shutdown/offline signal. waitForRpiShutdown() is satisfied by heartbeat timeout, which is a practical signal but not a strong explicit shutdown acknowledgment packet.

### 6.8 Relay GPIO Assignments

List of ESP32 GPIO pins that are indirectly connected to Relay actuation

| Relay Purpose | ESP32 GPIO |
| --- | --- |
| Screen power | 13 |
| DAC power | 12 |
| Raspberry Pi power | 11 |
| Output stage power | 9 |
| Prototype DAC enable | 10 |
| General relay 1 | 47 |
| General relay 2 | 39 |

### 6.9 Current Behavior Caveats

wait parameters in relay shutdown helpers are currently unused.

The code path sets GOING_TO_SLEEP for both sleep and deep-sleep paths.

The enum includes SHUTTING_DOWN and GOING_INTO_DEEP_SLEEP, but the current processor code does not clearly transition through them.

On boot timeout, the power LED still transitions to ON; the fast-flashing SPI LEDs are the only persistent failure indicator.

### 6.10 Debug Flag: Simulating Pi Boot

To test the full firmware on the bench without a connected Raspberry Pi, set the following flag in lib/board/boardConfig.hpp:

namespace board::debug {
    inline constexpr bool kSimulateRpiBoot = true; // set false for production
}

When true, RpiBootManager::waitForRpiToBoot() returns true immediately without waiting for a heartbeat, SpiBootIndicator::notifySuccess() is called normally, and a warning is logged. When false (default/production), this code path is optimized away entirely by the compiler (if constexpr).

## 7. Raspberry Pi Setup

Primary source:

scripts/rpi/README.md

code References:

scripts/rpi/home/antho/UAart5Listener.py

scripts/rpi/home/antho/heartbeat_sender.py

### 7.1 What Runs on the Pi

The Raspberry Pi side has two main Python processes:

a UART5 listener that receives commands from the ESP32 and runs local commands,

a heartbeat sender that periodically transmits a heartbeat packet back to the ESP32.

### 7.2 UART Listener

The listener script:

opens /dev/ttyAMA5,

waits on BCM GPIO23 for a rising edge,

reads one 18-byte packet,

verifies the checksum,

dispatches the command to local handlers.

Handled actions include: Raspberry Pi shutdown, playback next/previous, play/pause, stop, seek forward/back, rotary next/previous, meter toggle, cover view toggle, repeat toggle, random toggle, panel cycling (prev/next moOde UI panel via CDP WebSocket on port 9222).

### 7.3 Heartbeat Sender

The heartbeat script:

opens /dev/ttyAMA5,

waits until the ESP32 data-ready line is low,

sends a heartbeat packet every 10 seconds,

pulses BCM GPIO24 to notify the ESP32 that data is waiting.

### 7.4 Required Pi Configuration

| Configuration | Details |
| --- | --- |
| UART5 overlay | Add dtoverlay=uart5 to /boot/firmware/config.txt |
| Python serial | Python serial library must be installed |
| GPIO daemon | pigpiod must be running |
| Chromium | Add --remote-debugging-port=9222 to launch command in ~/.xinitrc (required for panel cycling via CDP) |

7.5 Systemd Services

| Service | Role | Script |
| --- | --- | --- |
| uart_listener.service | Receives ESP32 commands | /usr/bin/python3 /home/antho/UAart5Listener.py |
| heartbeat.service | Periodically sends heartbeat to ESP32 | /usr/bin/python3 /home/antho/heartbeat_sender.py |
| pigpiod | GPIO support | System service |

### 7.6 Pi File Locations

| Purpose | Path |
| --- | --- |
| UART5 overlay config | /boot/firmware/config.txt |
| Listener script | /home/antho/UAart5Listener.py |
| Listener service | /etc/systemd/system/uart_listener.service |

## 8. Developer Guides

### 8.1 Build and Test

#### Firmware Build

The verified workspace build path is the VS Code task: PlatformIO Build

Top-level configuration files:

platformio.ini

CMakeLists.txt

#### Host-Side Tests

Host tests live under host_tests/ and can be run with CMake/MSVC on this machine without a connected ESP32 or ESP-IDF.

#### Device Tests

PlatformIO device tests live under test/. They compile for the ESP32-S3 and require a connected board to run:

pio test

| Suite | What It Covers |
| --- | --- |
| test/test_power_led | PowerLed constructor defaults and brightness scaling |
| test/test_simple_command_action | SimpleCommandAction press/release response |
| test/test_uart_protocol | UART message serialization, deserialization, checksum |
| test/test_spi_boot_indicator | SpiBootIndicator state machine: start/success/failure/idempotency |
| test/test_spi_led_driver | SpiLedDriver constructor state and early-return guard paths |

All five suites build and link cleanly against the ESP32-S3 toolchain.

### 8.2 How to Add a New Button Action

The current canonical ownership for button wiring:

| Concern | File |
| --- | --- |
| Physical button ID constants | lib/board/boardButtonIds.hpp |
| Button ID namespace alias | lib/app/ControlBoardButtonIds.hpp |
| Button registration table | lib/app/ControlBoardActionRegistry.cpp |
| Action source templates | lib/input/actions/actionTemplates.hpp |
| Command routing table | lib/app/ActionCommandRoutingPolicy.hpp |
| Action factory (route → concrete IAction) | lib/app/ActionFactory.cpp |
| Concrete command implementations | lib/app/commands/ |
| UART toggle/rotary dispatch | lib/app/ActionUartDispatcher.cpp |
| Protocol command IDs | lib/protocol/uartProtocol.hpp |

There are no global action singleton files (buttonActions.hpp/buttonActions.cpp). Actions are created dynamically from the registration table.

#### Step 1: Add the Command ID

Define the new command in lib/protocol/uartProtocol.hpp:

- Pick a command name consistent with the current naming scheme (e.g. CMD_FOO_BAR).

Pick a unique numeric value that does not collide with existing commands.

If the Raspberry Pi is involved, update the Pi-side listener script.

Update Section 4 (UART Protocol Reference) and Section 5 (Button and Command Map) of this document.

#### Step 2: Add or Reserve a Button ID

If adding a brand-new physical button, reserve its ID in lib/board/boardButtonIds.hpp:

inline constexpr uint8_t k_mute = 16;

Increment k_count to match the new total.

Notes:
- ControlBoardButtonIds.hpp is a namespace alias — it picks up new constants automatically.
- The SPI LED bitmask uses the button ID as a bit index for non-power buttons, so changing existing indices also shifts LED positions. Reserve new IDs at the end of the list.

#### Step 3: Register the Button

Add one row to constexpr ButtonRegistration k_buttons[] in lib/app/ControlBoardActionRegistry.cpp:

{ controlBoardButtons::k_mute, ActionSourceType::Simple, CMD_MUTE, CMD_NO_ACTION, LedPolicy::Momentary },

Column meanings:

| Column | Purpose |
| --- | --- |
| buttonId | Physical button constant from board::buttons |
| type | Simple, Toggle, Timed, or Rotary |
| cmd1 | Main command (or CMD_ON for Toggle) |
| cmd2 | CMD_OFF for Toggle; CMD_NO_ACTION otherwise |
| ledPolicy | None, Momentary, or Toggle |

#### Step 4: Wire the Behavior

Path A — Simple UART command:

No extra work needed beyond steps 1–3. Commands not in k_commandRouteTable[] default to ActionCommandRoute::UartDispatch, and UartDispatchAction::execute() sends them with no parameters.

Path B — Toggle UART command:

Add one row to k_toggleTable[] in lib/app/ActionUartDispatcher.cpp:

{CMD_MUTE_ON, CMD_MUTE_OFF, CMD_TOGGLE_MUTE, "Mute"},

The dispatcher will match CMD_MUTE_ON or CMD_MUTE_OFF and send a UART packet with commandId = CMD_TOGGLE_MUTE and params[0] = 1 (ON) or 0 (OFF).

Path C — Local-only action:

- Classify the command in lib/app/ActionCommandRoutingPolicy.hpp:

{ CMD_TOGGLE_FAN, ActionCommandRoute::Relay },

If a new ActionCommandRoute value was added, map it in createAction() in lib/app/ActionFactory.cpp:

case ActionCommandRoute::Fan:
    return std::make_unique<actions::FanAction>(command);

Create lib/app/commands/FanAction.hpp and FanAction.cpp inheriting actions::IAction and implementing execute(ActionContext &ctx).

#### Step 5: Update the Pi Side (if UART)

Update scripts/rpi/home/antho/UAart5Listener.py to handle the new command ID, decoding parameters consistently with the firmware encoding.

#### Step 6: Quick Checklist

- Add or confirm the CMD_* constant in uartProtocol.hpp.

Add or confirm the button ID in boardButtonIds.hpp; increment k_count.

Add one row to k_buttons[] in ControlBoardActionRegistry.cpp.

If toggle UART: add a row to k_toggleTable[] in ActionUartDispatcher.cpp.

If local-only: classify in ActionCommandRoutingPolicy.hpp, map in ActionFactory.cpp, implement execute().

Update Pi-side handling if UART is involved.

Update Sections 4 and 5 of this document.

Run the PlatformIO Build task and verify the expected side effect.

#### Worked Examples

Example A — Simple UART button (Mute):

- uartProtocol.hpp — add CMD_MUTE = 0x0120

boardButtonIds.hpp — add k_mute = 16, increment k_count

ControlBoardActionRegistry.cpp — add { k_mute, ActionSourceType::Simple, CMD_MUTE, CMD_NO_ACTION, LedPolicy::Momentary }

Pi listener — handle CMD_MUTE

Update this document

Example B — Toggle UART button (Mute with state):

- uartProtocol.hpp — add CMD_MUTE_ON, CMD_MUTE_OFF, CMD_TOGGLE_MUTE

boardButtonIds.hpp — add k_mute, increment k_count

ControlBoardActionRegistry.cpp — add Toggle row

ActionUartDispatcher.cpp — add row to k_toggleTable[]

Pi listener, then update this document

Example C — Local relay button (Fan):

- uartProtocol.hpp — add CMD_TOGGLE_FAN

boardButtonIds.hpp — add k_fan, increment k_count

ControlBoardActionRegistry.cpp — add Simple row for CMD_TOGGLE_FAN

ActionCommandRoutingPolicy.hpp — add { CMD_TOGGLE_FAN, ActionCommandRoute::Relay }

Extend RelayAction.cpp to handle CMD_TOGGLE_FAN, or create a new command class

Update this document

### 8.3 Project Structure and Folder Ownership

#### Current Folder Map

| Area | Purpose |
| --- | --- |
| src/ | Firmware entry point (main.cpp, main.h) |
| lib/board/ | Board-specific constants, pin maps, identity, debug flags |
| lib/app/ | Orchestration and top-level runtime composition |
| lib/input/actions/ | Action definitions and reusable button action objects |
| lib/hal/buttons/ | MCP23017 input expander driver |
| lib/hal/leds/ | SPI and PWM LED drivers |
| lib/hal/relay/ | Low-level relay GPIO abstraction |
| lib/hal/uart/ | UART transport and handshake handling |
| lib/hal/storage/ | NVS storage helper |
| lib/protocol/ | Wire format and command IDs |
| lib/indicators/ | LED services, brightness control, and boot indication |
| lib/power/ | Power state, relay sequencing, Pi boot/shutdown coordination |
| scripts/rpi/ | Raspberry Pi listener, sender, and setup docs |
| docs/ | Project documentation |
| host_tests/ | Pure C++ host-side tests (no ESP-IDF required) |
| test/ | PlatformIO device test suites (run on ESP32-S3) |

lib/app/ — orchestration only: top-level wiring, callback registration, button and UART dispatch, composing domain services. Must not own relay policy, boot waiting, board pin maps, or protocol constants.

lib/board/ — physical board facts: pins, ports, addresses, board identity, fixed timing values tied to hardware bring-up.

lib/input/ — human input and local action mapping: button capture, MCP expander details, rotary decoding, button-to-action lookup, reusable button action objects.

lib/indicators/ — user-visible feedback only: status LEDs, power LED behavior, brightness control, PWM LED helpers.

lib/power/ — power lifecycle and shutdown coordination: power states, relay sequencing, Pi boot/shutdown waiting.

lib/protocol/ — wire format only: packet structure, command IDs, serialization.

lib/hal/ — hardware abstraction: thin wrappers around ESP-IDF peripheral APIs.

### 8.4 Configuration Constants Reference

#### Timing Constants (board::timing in lib/board/boardConfig.hpp)

| Constant | Default Value | Unit | Description |
| --- | --- | --- | --- |
| k_heartbeatTimeoutMs | 30 000 | ms | Inactivity window before heartbeat is considered lost |
| k_initDelayMs | 5 000 | ms | Post-boot settle delay before triggerInitialPowerOn() |
| k_powerSettleDelayMs | 1 500 | ms | Delay after DAC / output-stage relay enable |
| k_screenOnDelayMs | 1 000 | ms | Delay between screen and Pi relay steps during power-on |
| k_rpiBootTimeoutMs | 60 000 | ms | Max wait for Pi heartbeat after power-on |
| k_rpiShutdownTimeoutMs | 60 000 | ms | Max wait for Pi heartbeat loss during shutdown |
| k_rpiShutdownSettleDelayMs | 500 | ms | Delay between Pi relay off and screen relay off |
| k_screenPowerOffDelayMs | 5 000 | ms | Delay after screen relay off before LED state changes |
| kLongPressThresholdMs (in PowerStateTransitionPolicy.hpp) | 3 000 | ms | Power-button hold that selects deep sleep vs sleep |

#### UART Constants (board::serial in lib/board/boardConfig.hpp)

| Constant | Value | Description |
| --- | --- | --- |
| k_port | UART_NUM_2 | UART peripheral |
| k_baudRate | 115 200 | Baud rate |
| k_txPin | GPIO_NUM_2 | TX to Pi |
| k_rxPin | GPIO_NUM_1 | RX from Pi |
| k_bufferSize | 256 | RX ring buffer bytes |
| k_rpiDataReadyPin | GPIO_NUM_42 | Pi pulls high to signal data ready |
| k_esp32DataReadyPin | GPIO_NUM_41 | ESP32 pulses high after sending |

NVS Keys

| Namespace | Key | Type | Default | Written By | Description |
| --- | --- | --- | --- | --- | --- |
| brightness | level | int8_t | 2 | MonitorBrightnessController | Monitor brightness level (0–9); persisted on every user change |

### 8.5 Dependencies and Third-Party Libraries

| Library / Component | Version | Source | Purpose |
| --- | --- | --- | --- |
| Espressif ESP-IDF | ~5.1 (via PlatformIO espressif32 @ ~6.5.0) | https://github.com/espressif/esp-idf | RTOS, drivers (UART, I2C, SPI, LEDC, GPIO, NVS), FreeRTOS port |
| FreeRTOS | Bundled with ESP-IDF | Part of ESP-IDF | Task scheduling, queues, event groups, timers |
| PlatformIO | ≥6.x | https://platformio.org | Build system, environment management, flash/monitor tooling |
| Arduino / ESP32 Arduino Core | Not used | — | Framework is espidf (bare IDF); Arduino layer is not present |

# setup Moode (Raspberry Pi)

This guide explains how to enable UART5 on a Raspberry Pi running Moode, install required dependencies, build and install pigpio, and configure systemd services to automatically run:

- A UART5 listener

- A UART heartbeat sender

It also covers auto-login configuration and troubleshooting.

### Repo Layout

This repo keeps Raspberry Pi companion assets under scripts/rpi/.

Important tracked files:

- scripts/rpi/home/antho/UAart5Listener.py: Pi-side UART listener

- scripts/rpi/home/antho/heartbeat_sender.py: Pi-side heartbeat sender

- scripts/rpi/boot/firmware/config.txt: example firmware config with dtoverlay=uart5

- scripts/rpi/boot/firmware/config-user.txt: example user overlay additions

- scripts/rpi/boot/firmware/cmdline.txt: example boot command line

Deployment note:

- These files are source-controlled examples, not their final runtime locations on the Pi.

- Copy the scripts and config fragments from this folder into the deployment paths referenced below when setting up a Pi.

Current listener behaviour note:

- On startup, the UART listener forces the display to the firmware default meter-off state (webui) before processing incoming commands.

### 1. Install Moode Using Raspberry Pi Imager

Use the official Raspberry Pi Imager to flash Moode onto your SD card.

Once moOde is running, update the system:

- sudo apt update

- sudo apt upgrade -y

### 2. Install Required Packages

- sudo apt install -y ddcutil

### 3. Enable UART5

Edit the Raspberry Pi firmware configuration:

sudo nano /boot/firmware/config.txt

Add or confirm the following entry:

- dtoverlay=uart5

Reboot to apply:

- sudo reboot

### 4. Install Python Serial Support

#### 4.1 Preferred Installation (pip)

- sudo python3 -m pip install pyserial

Verify installation:

- python3 -c "import serial; print(serial.__version__)"

#### 4.2 If pip Is Missing

- sudo apt install -y python3-pip

- sudo python3 -m pip install pyserial

#### 4.3 If You Encounter externally-managed-environment

Install the distro-managed version instead:

- sudo apt install -y python3-serial

### 5. Install pigpio (no longer required)

#### 5.1 Install Build Tools

- sudo apt update

- sudo apt install -y git make gcc

#### 5.2 Download and Build pigpio

- cd /tmp

- git clone https://github.com/joan2937/pigpio.git

- cd pigpio

make

- sudo make install

This installs:

- /usr/local/bin/pigpiod

- Python module pigpio

- Command-line tools (pigs, etc.)

#### 5.3 Enable and Start pigpiod

- sudo systemctl enable pigpiod

- sudo systemctl start pigpiod

Verify installation:

- which pigpiod

Expected output:

/usr/local/bin/pigpiod

#### 5.4 Create pigpiod systemd Service (Manual)

- sudo nano /etc/systemd/system/pigpiod.service

[Unit]

Description=Pigpio daemon

After=network.target

[Service]

Type=forking

ExecStart=/usr/local/bin/pigpiod -l

ExecStop=/bin/kill -s TERM $MAINPID

Restart=always

[Install]

WantedBy=multi-user.target

Reload and start:

- sudo systemctl daemon-reexec

- sudo systemctl daemon-reload

- sudo systemctl enable pigpiod

- sudo systemctl start pigpiod

Verify it is running (number increments each run):

pigs t

### 6. Create the UART Listener systemd Service

- sudo nano /etc/systemd/system/uart_listener.service

[Unit]

Description=UART5 Listener for ESP32

After=network.target

[Service]

Type=simple

ExecStart=/usr/bin/python3 /home/antho/uart5_listener.py

Restart=always

User=antho

Group=antho

WorkingDirectory=/home/antho

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

### 7. Create the UART Listener Script

nano /home/antho/uart5_listener.py

Paste the contents of your UART listener script.

Set permissions:

- chmod +x /home/antho/uart5_listener.py

- sudo chown antho:antho /home/antho/uart5_listener.py

### 8. Enable and Start the UART Listener Service

- sudo systemctl daemon-reload

- sudo systemctl enable uart_listener.service

- sudo systemctl start uart_listener.service

- sudo systemctl status uart_listener.service

View logs:

- journalctl -u uart_listener.service -f

### 9. Create the Heartbeat Sender Service

#### 9.1 Create the Script

- sudo nano /usr/local/bin/heartbeat_sender.py

- sudo chmod +x /usr/local/bin/heartbeat_sender.py

- Paste the contents of heartbeat_sender.py and save.

#### 9.2 Create the systemd Service

- sudo nano /etc/systemd/system/heartbeat.service

[Unit]

Description=UART5 Heartbeat Sender

After=network.target multi-user.target

[Service]

Type=simple

ExecStart=/usr/bin/python3 /home/antho/heartbeat_sender.py

Restart=always

WorkingDirectory=/home/antho

[Install]

WantedBy=multi-user.target

#### 9.3 Enable and Start the Service

- sudo systemctl daemon-reload

- sudo systemctl enable heartbeat.service

- sudo systemctl restart heartbeat.service

Check logs:

- journalctl -u heartbeat.service -f

### 10. File Locations Summary

| Purpose | File Path | Notes |
| --- | --- | --- |
| UART5 overlay configuration | /boot/firmware/config.txt | Must contain dtoverlay=uart5 |
| UART listener script | /home/antho/uart5_listener.py | Must be executable |
| UART listener service | /etc/systemd/system/uart_listener.service | Auto-start on boot |
| Heartbeat service | /etc/systemd/system/heartbeat.service | Periodic UART TX |
| pigpiod binary | /usr/local/bin/pigpiod | Built from source |
| Logs | journalctl | journalctl -u <service> -f |

### 11. Set auto logging

Create and open a new file

- sudo mkdir -p /etc/systemd/system/getty@tty1.service.d

- sudo nano /etc/systemd/system/getty@tty1.service.d/autologin.conf

Add the following code

[Service]

ExecStart=

ExecStart=-/sbin/agetty --autologin pi --noclear %I $TERM

save the file this will enable auto login

### 12. Hide the login Text

sudo nano /boot/firmware/cmdline.txt

Add the following to the end of the line:

quiet loglevel=0 vt.global_cursor_default=0

### 13  Add the splash Screen

copy the splashscreen to the RPI

move into the folder where the splash screen is to be stored

make sure that the you  fcopy the file from the location where you originaly stored the file

- cd /opt

- sudo mv /home/antho/filename.png  splash.png

- sudo chmod +x splash.png

### add meters

copy your meter configuration into opt/1024x600

### 14. Enable Auto-Login on Local Console

- sudo raspi-config

Navigate to:

System Options → Boot / Auto Login

Choose one:

- B2 Console Autologin

- B4 Desktop Autologin

Reboot when prompted.

### 15. Troubleshooting

#### 15.1 UART5 Not Working

Check overlay:

- grep uart5 /boot/firmware/config.txt

Check devices:

- ls -l /dev/ttyAMA* /dev/ttyS* 2>/dev/null

Ensure no serial console is enabled:

- sudo nano /boot/firmware/cmdline.txt

Remove any console= entry referencing UART.

#### 15.2 Service Not Starting

- sudo systemctl status uart_listener.service

Reload systemd if needed:

- sudo systemctl daemon-reload

#### 15.3 Permission Denied Accessing UART

- sudo usermod -a -G dialout antho

- sudo reboot

#### 15.4 Python serial Module Not Found

- sudo apt install python3-serial

or

- sudo python3 -m pip install pyserial